| Raw Material Name: Ferric Ammonium Citrate | Batch No: | Pharmacopeia Ref: USP |
| --- | --- | --- |
| Manufacturer: | Supplier: | Logbook/Revision Number |
| MF. Date: | EX. Date: | Logbook page No.: |
| Sampled By: | Qty Received: | Logbook Volume: |
| Product Code: RMA020 | No. of containers: | Analysed by: |
| QC. AR. No: | Test Completed Date: | SOP Ref. No: BML-QDG-QA-105 |

| S/No: | Test |  | Specification | Result |
| --- | --- | --- | --- | --- |
| 1. | Appearance |  | Thin, transparent, garnet-red scales or granules or brownish-yellow powder. |  |
| 2. | Solubility |  | Very soluble in water; insoluble in alcohol. |  |
| 3. | Identification |  | - Ignition test: it chars, and leaves a residue of iron oxide. - On addition of 6 N ammonium hydroxide drop wise: the solution darkens, but no precipitate forms. A white precipitate forms. |  |
| 4. | Ferric citrate |  | No blue precipitate is formed |  |
| 5. | Sulphates |  | Turbidity formed after 10 minutes is not greater than that produced in a similarly treated control solution |  |
| 6. | Oxalate |  | No turbidity is produced within 5 minutes. |  |
| 7. | Mercury |  | NMT 10 µg. |  |
| 8. | Limit of lead |  | NMT 10 µg. |  |
| 9. | Loss on drying |  | NMT 0.1% |  |
| 10. | Assay |  | 20.5 – 22.5 % |  |
| 11. | Microbial Contamination | Total Microbial count | NMT 1000cfu/g |  |
|  |  | Total Yeast & Moulds Count | NMT 100cfu/g |  |
|  |  | Pathogens | Must be Absent |  |

CHECKER’s VERDICT (box option):  RELEASED   / FAILED / HOLD   / REPEAT

Remark: