| Raw Material Name: Glycerin | Batch No: | Pharmacopeia Ref: BP 2024 |
| --- | --- | --- |
| Manufacturer: | Supplier: | Logbook/Revision Number |
| MF. Date: | EX. Date: | Logbook page No.: |
| Sampled By: | Qty Received: | Logbook Volume: |
| Product Code: RME023 | No. of containers: | Analysed by: |
| QC. AR. No: | Test Completed Date: | SOP Ref. No: BML-QDG-QA-105 |

| S/No: | Test |  | Specification | Result |
| --- | --- | --- | --- | --- |
| 1. | Appearance |  | Syrupy liquid, unctuous to the touch, colourless or almost colourless, clear, very hygroscopic. |  |
| 2. | Solubility |  | Miscible with water and with ethanol (96%), slightly soluble in acetone, practically insoluble in fatty oils and in essential oils |  |
| 3. | Identification |  | - Refractive index - IR spectrum: similar to reference spectrum of glycerol (85%) - Blue ring interface is developed which does not diffuse into the lower layer - Vapours (acrolein) are evolved which blackens filter paper impregnated with alkaline potassium tetraiodomercurate solution |  |
| 4. | Appearance of solution |  | Solution is clear, and upon dilution, colourless. |  |
| 5. | Acidity or alkalinity |  | NMT 0.2mL of 0.1M NaOH is required to change the colour of the indicator to pink |  |
| 6. | Refractive index |  | 1.470 to 1.475 |  |
| 7. | aldehydes |  | NMT 10ppm |  |
| 8. | Esters |  | NLT 8.0mL of 0.1M HCl is required to change the colour of the indicator. |  |
| 9. | Impurity A/related substances |  | NMT 0.5% total impurity, NMT 0.1% impurity A, NMT 0.1% any other impurity. |  |
| 10. | Halogenated compounds |  | NMT 35ppm |  |
| 11. | Sugars |  | The solution remains blue and no precipitate is formed |  |
| 12. | Chlorides |  | NMT 10ppm |  |
| 13. | Water |  | NMT 2.0% |  |
| 14 | Sulphated ash |  | NMT 0.01% |  |
| 15 | Assay |  | 98.0 to 101.0% anhydrous substance |  |
| 16. | Microbial Contamination | Total Microbial count | NMT 1000cfu/g |  |
|  |  | Total Yeast & Moulds Count | NMT 100cfu/g |  |
|  |  | Pathogens | Must be Absent |  |

CHECKER’s VERDICT (box option):  RELEASED   / FAILED / HOLD   / REPEAT

Remark: