| Raw Material Name: Menthol Crystals | Batch No: | Pharmacopeia Ref: BP 2024 |
| --- | --- | --- |
| Manufacturer: | Supplier: | Logbook/Revision Number |
| MF. Date: | EX. Date: | Logbook page No.: |
| Sampled By: | Qty Received: | Logbook Volume: |
| Product Code: RME029 | No. of containers: | Analysed by: |
| QC. AR. No: | Test Completed Date: | SOP Ref. No: BML-QDG-QA-105 |

| S/No: | Test |  | Specification |  | Result |
| --- | --- | --- | --- | --- | --- |
| 1. | Appearance |  | Prismatic or acicular, colourless, shiny crystals. |  |  |
| 2. | Solubility |  | Practically insoluble in water, very soluble in ethanol (96%) and in light petroleum, freely soluble in fatty oils and in liquid paraffin, very slightly soluble in glycerol. |  |  |
| 3. | Melting point |  | About 43C |  |  |
| 4. | Identification |  | A | Specific optical rotation |  |
|  |  |  | B | Principal spot is similar in position, colour and size to reference |  |
|  |  |  | C | Same as B. for related substances |  |
|  |  |  | D | The crystals melt at 154 to 157C |  |
| 5. | Appearance of solution |  | Solution is clear and colourless |  |  |
| 6. | Acidity or Alkalinity |  | NMT 0.5mL of 0.01M NaOH is required |  |  |
| 7. | Specific Optical Rotation |  | -48 to -51 |  |  |
| 8. | Related substance |  | Total: NMT 1% |  |  |
| 9. | Residue on Evaporation |  | NMT 0.05% |  |  |
| 10. | Microbial Contamination | Total Microbial count | NMT 1000cfu/g |  |  |
|  |  | Total Yeast & Moulds Count | NMT 100cfu/g |  |  |
|  |  | Pathogens | Must be Absent |  |  |

CHECKER’s VERDICT (box option):  RELEASED   / FAILED / HOLD   / REPEAT

Remark: