| Raw Material Name: Propylene Glycol | Batch No: | Pharmacopeia Ref: USP |
| --- | --- | --- |
| Manufacturer: | Supplier: | Logbook/Revision Number |
| MF. Date: | EX. Date: | Logbook page No.: |
| Sampled By: | Qty Received: | Logbook Volume: |
| Product Code: RME002 | No. of containers: | Analysed by: |
| QC. AR. No: | Test Completed Date: | SOP Ref. No: BML-QDG-QA-105 |

| S/No: | Test |  | Specification | Result |
| --- | --- | --- | --- | --- |
| 1. | Appearance |  | A viscous, clear, colourless, hygroscopic liquid |  |
| 2. | Solubility |  | Miscible with water and with ethanol (96 per cent). |  |
| 3. | Identification |  | Crystal obtained from its reaction melts at 121oC to 128oC |  |
| 4. | Relative density |  | 1.035 to 1.040. |  |
| 5. | Refractive index |  | 1.431 to 1.433. |  |
| 6. | Acidity |  | Not more than 0.05 mL of 0.1 M sodium hydroxide is required to change the colour of the indicator to blue. |  |
| 7. | Oxidising substances |  | Not more than 0.2 mL of 0.05 M sodium thiosulfate is required |  |
| 8. | Reducing substances |  | The solution does not change its appearance after the addition of 0.15mL of 0.1M AgNO3 |  |
| 9. | Water |  | NMT 0.2% on 5.00g |  |
| 10. | Sulphated Ash |  | NMT 5mg on 50g (0.01%) |  |
| 11. | Boiling Point |  | 184 °C - 189 °C. |  |
| 12. | Microbial Contamination | Total Microbial count | NMT 1000cfu/g |  |
|  |  | Total Yeast & Moulds Count | NMT 100cfu/g |  |
|  |  | Pathogens | Must be Absent |  |

CHECKER’s VERDICT (box option):  RELEASED   / FAILED / HOLD   / REPEAT

Remark: