| Raw Material Name: Sucrose | Batch No: | Pharmacopeia Ref: BP 2024 |
| --- | --- | --- |
| Manufacturer: | Supplier: | Logbook/Revision Number |
| MF. Date: | EX. Date: | Logbook page No.: |
| Sampled By: | Qty Received: | Logbook Volume: |
| Product Code: RME009 | No. of containers: | Analysed by: |
| QC. AR. No: | Test Completed Date: | SOP Ref. No: BML-QDG-QA-105 |

| S/No: | Test |  | Specification | Result |
| --- | --- | --- | --- | --- |
| 1. | Appearance |  | White or almost white, crystalline powder, or lustrous, colourless or white or almost white crystals |  |
| 2. | Solubility |  | Very soluble in water, slightly soluble in ethanol(96%), practically insoluble in anhydrous ethanol |  |
| 3. | Identification |  | - IR absorptions spec: similar to sucrose CRS - TLC: principal spot similar in colour, position and size to that of reference. Solution is blue and clear and remains so after boiling. Further treated, orange precipitate is formed |  |
| 4. | Appearance of solution |  | Solution is clear |  |
| 5. | Conductivity |  | NMT 35µS.cm-1 |  |
| 6. | Specific optical rotation |  | +66.3 to 67.0 |  |
| 7. | Colour value |  | NMT 45 |  |
| 8. | Dextrin |  | Solution remains yellow |  |
| 9. | Reducing sugars |  | The blue colour does not disappear completely |  |
| 10. | Sulfites |  | NMT 10ppm |  |
| 11. | Loss on drying |  | NMT 0.1% |  |
| 12. | Microbial Contamination | Total Microbial count | NMT 1000cfu/g |  |
|  |  | Total Yeast & Moulds Count | NMT 100cfu/g |  |
|  |  | Pathogens | Must be Absent |  |

CHECKER’s VERDICT (box option):  RELEASED   / FAILED / HOLD   / REPEAT

Remark: