| S/N | TEST | DESCRIPTIONS OF PROCEDURE |
| --- | --- | --- |
|  | DESCRIPTION | Determine the colour and appearance of material |
|  | SOLUBILITY | Determine its solubility in water. |
|  | IDENTIFICATION | First identification: A,B . A. Subject to reaction (a) of chlorides. Dissolve in 2 mL of water R a quantity of the substance to be examined equivalent to about 2 mg of chloride (Cl–) or use 2 mL of the prescribed solution. Acidify with dilute nitric acid R and add 0.4 mL of silver nitrate solution R1. Shake and allow to stand. A curdled, white precipitate is formed. Centrifuge and wash the precipitate with three quantities, each of 1 mL, of water R. Carry out this operation rapidly in subdued light, disregarding the fact that the supernatant solution may not become perfectly clear. Suspend the precipitate in 2 mL of water R and add 1.5 mL of ammonia R. The precipitate dissolves easily with the possible exception of a few large particles which dissolve slowly. B. Subject to reaction ammonium salts. To the prescribed solution add 0.2 g of magnesium oxide R. Pass a current of air through the mixture and direct the gas that escapes just beneath the surface of a mixture of 1 mL of 0.1 M hydrochloric acid and 0.05 mL of methyl red solution R. The colour of the indicator changes to yellow. On addition of 1 mL of a freshly prepared 100 g/L solution of sodium cobaltinitrite R a yellow precipitate is formed. |
|  | APPEARANCE OF SOLUTION | Solution S ### Dissolve 10.0 g in carbon dioxide-free water R prepared from distilled water R and dilute to 100 mL with the same solvent. Observe the clarity of Solution S and a five fold dilution of solution S. Compare it colour intensity with reference solution BY6 |
|  | ACIDITY/ALKALINITY | To 10 mL of solution S add 0.15 mL of methyl red solution R and 0.25 mL of 0.01 M hydrochloric acid. The solution is pink. Titrate the solution with 0.01 M sodium hydroxide until the colour changes to yellow. |
|  | BROMIDE AND IODIDE | To 10 mL of solution S add 0.1 mL of dilute hydrochloric acid R and 0.05 mL of chloramine solution R. After 1 min, add 2 mL of chloroform R and shake vigorously. |
|  | LOSS ON DRYING | Determine the loss drying on 1.00 g by drying in an oven at 105 °C for 2 h. |
|  | SULPHATED ASH | Determine the Sulphated Ash on 2.0g of sample at 8000C. |
|  | ASSAY | Dissolve 1.000 g in 20 mL of water R and add a mixture of 5 mL of formaldehyde solution R, previously neutralised to phenolphthalein solution R, and 20 mL of water R. After 1-2 min, titrate slowly with 1 M sodium hydroxide, using a further 0.2 mL of the same indicator. 1 mL of 1 M sodium hydroxide is equivalent to 53.49 mg of NH4Cl. |
|  | MICROBIAL ENUMERATION TEST | Determination of total aerobic microbial count (TAMC) and total combined yeasts and molds count (TYMC) by the pour plate method. - Slightly open the lid of the petri plate and dispense 1 ml of diluted sample in the center of the Petri plate using a sterile calibrated pipette. - Open the lid of the prepared media. (soya bean digest agar for TAMC and Sabouraud Dextrose Agar for TYMC) - Pour about 15-20ml of the sterilized molten media at the appropriate temperature above the sample. - Close the lid of the plate then mix the sample in the media properly by gently swirling the plate. The plate is generally swirled in an “S” or “8” shape. - Allow the media to completely solidify. - Simultaneously perform the negative control by plating 1ml of diluent used instead of the sample for each medium. - Incubate the plate in an inverted position under suitable incubation conditions (For TAMC Incubate the plates of Soybean Casein Digest Agar at 30°C-35°C for 48hours and For TYMC incubate the plates of Sabouraud Dextrose Agar at 20°C - 25°C for 72hours Test for Absence of Pathogen Test for E. coli: Using same dilution - Transfer 1ml of the sample into a sterile McCartney bottle containing 9ml of sterile MacConkey broth. - Incubate the MacConkey broth bottle at 300C-350C for 24 hours. - After incubation shake the tube and transfer loop full portion of MacConkey broth on the surface of the pre incubated Eosin Methylene Blue Agar petri plate. - Incubate plates at 30°C to 35°C for 48 hours. - For negative control, proceeding same as above except without sample, streak a portion of incubated sterile MacConkey broth medium on the surface of pre incubated Eosin Methylene Blue Agar. - Observe and record the presence or absence of specified microorganisms. - Include positive and negative controls in each batch of tests. - Verify the performance of media and reagents used in the analysis. Interpretation of Results - The bacterial count should not exceed 102cfu/ml while that of the fungal count should not be more than 102cfu/ml. There should be absence of fecal coliform (E coli). - If growth occurs in the negative control plate, the test is invalid, failed negative control must be investigated - The total aerobic microbial count (TAMC) is considered to be equal to the number of CFU found using casein soya bean digest agar; if colonies of fungi are detected on this medium, they are counted as part of TAMC. - The total combined yeasts/mould count (TYMC) is considered to be equal to the number of CFU found using Sabouraud dextrose agar; if colonies of bacteria are detected on this medium, they are counted as part of TYMC - When an acceptance criterion for microbiological quality is prescribed it is interpreted as follows: 101 CFU : maximum acceptable count＝20 102 CFU: maximum acceptable count＝200, 103 CFU: maximum acceptable count＝2000. |

| - CURRENT REVISION | - SUPERSEDE | - REASON FOR REVIEW |
| --- | --- | --- |
| - 00 | - NIL | - FIRST ISSUE |
| - 01 | - 00 | - Review of the procedures - Change in header and footer format |