| S/N | TEST | DESCRIPTIONS OF PROCEDURE |
| --- | --- | --- |
|  | DESCRIPTION | Determine the colour and appearance of material |
|  | SOLUBILITY | Determine its solubility in water and in ethanol (96 per cent). |
|  | IDENTIFICATION | First identification: B, C. Second identification: A, C, D. A. Ultraviolet and visible absorption spectrophotometry. Test solution Dissolve 0.10 g in water R and dilute immediately to 100.0 mL with the same solvent. Add 1.0 mL of the solution to 10 mL of a 10.3 g/L solution of hydrochloric acid R and dilute to 100.0 mL with water R. Absorption maximum 243 nm, determined immediately after dissolution. B. Infrared absorption spectrophotometry (2.2.24). Comparisonascorbic acid CRS. C. Determine the pH of Solution S D. To 1 mL of solution S add 0.2 mL of dilute nitric acid R and 0.2 mL of silver nitrate solution R2. |
|  | APPEARANCE OF SOLUTION | ### Solution S Dissolve 1.0 g in carbon dioxide-free water R and dilute to 20 mL with the same solvent.. Using identical tubes of colourless, transparent, neutral glass with a flat base and an internal diameter of 15 mm to 25 mm, compare the liquid to be examined with water R or the solvent or the reference solution (see Tables of reference solutions) prescribed in the monograph, the depth of the layer being 40 mm. Compare the colours in diffused daylight, viewing vertically against a white background. |
|  | SPECIFIC OPTICAL ROTATION | Dissolve 2.50 g in water R and dilute to 25.0 mL with the same solvent. |
|  | IMPURITY E | Test solution Dissolve 0.25 g in 5 mL of water R. Neutralise using dilute sodium hydroxide solution R, then add 1 mL of dilute acetic acid R and 0.5 mL of calcium chloride solution R. Reference solution Dissolve 70 mg of oxalic acid R (dihydrate of impurity E) in water R and dilute to 500 mL with the same solvent; to 5 mL of the solution add 1 mL of dilute acetic acid R and 0.5 mL of calcium chloride solution R. Allow the solutions to stand for 1 h. Any opalescence in the test solution is not more intense than that in the reference solution. |
|  | RELATED SUBSTANCES | Liquid chromatography. Prepare the solutions immediately before use. Phosphate buffer solution Dissolve 6.8 g of potassium dihydrogen phosphate R in water for chromatography R and dilute to about 200 mL with the same solvent. Filter through a membrane filter (nominal pore size 0.45 µm) and dilute to 1000 mL with water for chromatography R. Test solution Dissolve 0.500 g of the substance to be examined in the mobile phase and dilute to 10.0 mL with the mobile phase. Reference solution (a) Dissolve 10.0 mg of ascorbic acid impurity C CRS in the mobile phase and dilute to 5.0 mL with the mobile phase. Reference solution (b) Dissolve 5.0 mg of ascorbic acid impurity D CRS and 5.0 mg of ascorbic acid CRS in the mobile phase and dilute to 100.0 mL with the mobile phase. Reference solution (c) Dilute 1 mL of the test solution to 200 mL with the mobile phase. Mix 1 mL of this solution and 1 mL of reference solution (a). Reference solution (d) Dilute 2.5 mL of reference solution (a) to 100.0 mL with the mobile phase. Column: — size: l = 0.25 m, Ø = 4.6 mm; — stationary phase: aminopropylsilyl silica gel for chromatography R (5 µm); — temperature: 45 °C. Mobile phase Phosphate buffer solution, acetonitrile R1 (25:75 V/V). Flow rate 1.0 mL/min. Detection Spectrophotometer at 210 nm. Injection 20 µL of the test solution and reference solutions (b), (c) and (d). Run time 2.5 times the retention time of ascorbic acid. Identification of impurities Use the chromatogram obtained with reference solution (d) to identify the peak due to impurity C; use the chromatogram obtained with reference solution (b) to identify the peak due to impurity D. Relative retention With reference to ascorbic acid (retention time = about 9 min): impurity D = about 0.5; impurity C = about 1.45. System suitability: — resolution: minimum 3.0 between the peaks due to ascorbic acid and impurity C in the chromatogram obtained with reference solution (c); — signal-to-noise ratio: minimum 20 for the peak due to impurity C in the chromatogram obtained with reference solution (d). Calculation of percentage contents: — for impurity C, use the concentration of impurity C in reference solution (d); — for impurity D, use the concentration of impurity D in reference solution (b); — for impurities other than C and D, use the concentration of ascorbic acid in reference solution (b). |
|  | COPPER | Atomic absorption spectrometry. Test solution Dissolve 2.0 g in 0.1 M nitric acid and dilute to 25.0 mL with the same acid. Reference solutions Prepare the reference solutions (0.2 ppm, 0.4 ppm and 0.6 ppm) using copper standard solution (10 ppm Cu) R, diluting with 0.1 M nitric acid. Source Copper hollow-cathode lamp. Wavelength 324.8 nm. Atomisation device Air-acetylene flame. Adjust the zero of the apparatus using 0.1 M nitric acid. |
|  | IRON | Atomic absorption spectrometry. Test solution Dissolve 5.0 g in 0.1 M nitric acid and dilute to 25.0 mL with the same acid. Reference solutions Prepare the reference solutions (0.2 ppm, 0.4 ppm and 0.6 ppm) using iron standard solution (20 ppm Fe) R, diluting with 0.1 M nitric acid. Source Iron hollow-cathode lamp. Wavelength 248.3 nm. Atomisation device Air-acetylene flame. Adjust the zero of the apparatus using 0.1 M nitric acid. |
|  | SULPHATED ASH | Determine the Sulphated Ash on 1.0g of sample at 8000C |
|  | ASSAY | Dissolve 0.150 g in a mixture of 10 mL of dilute sulfuric acid R and 80 mL of carbon dioxide-free water R. Add 1 mL of starch solution R. Titrate with 0.05 M iodine until a persistent violet-blue colour is obtained. 1 mL of 0.05 M iodine is equivalent to 8.81 mg of C6H8O6. |
|  | MICROBIAL ENUMERATION TEST | Determination of total aerobic microbial count (TAMC) and total combined yeasts and molds count (TYMC) by the pour plate method. - Slightly open the lid of the petri plate and dispense 1 ml of diluted sample in the center of the Petri plate using a sterile calibrated pipette. - Open the lid of the prepared media. (soya bean digest agar for TAMC and Sabouraud Dextrose Agar for TYMC) - Pour about 15-20ml of the sterilized molten media at the appropriate temperature above the sample. - Close the lid of the plate then mix the sample in the media properly by gently swirling the plate. The plate is generally swirled in an “S” or “8” shape. - Allow the media to completely solidify. - Simultaneously perform the negative control by plating 1ml of diluent used instead of the sample for each medium. - Incubate the plate in an inverted position under suitable incubation conditions (For TAMC Incubate the plates of Soybean Casein Digest Agar at 30°C-35°C for 48hours and For TYMC incubate the plates of Sabouraud Dextrose Agar at 20°C - 25°C for 72hours Test for Absence of Pathogen Test for E. coli: Using same dilution - Transfer 1ml of the sample into a sterile McCartney bottle containing 9ml of sterile MacConkey broth. - Incubate the MacConkey broth bottle at 300C-350C for 24 hours. - After incubation shake the tube and transfer loop full portion of MacConkey broth on the surface of the pre incubated Eosin Methylene Blue Agar petri plate. - Incubate plates at 30°C to 35°C for 48 hours. - For negative control, proceeding same as above except without sample, streak a portion of incubated sterile MacConkey broth medium on the surface of pre incubated Eosin Methylene Blue Agar. - Observe and record the presence or absence of specified microorganisms. - Include positive and negative controls in each batch of tests. - Verify the performance of media and reagents used in the analysis. Interpretation of Results - The bacterial count should not exceed 102cfu/ml while that of the fungal count should not be more than 102cfu/ml. There should be absence of fecal coliform (E coli). - If growth occurs in the negative control plate, the test is invalid, failed negative control must be investigated - The total aerobic microbial count (TAMC) is considered to be equal to the number of CFU found using casein soya bean digest agar; if colonies of fungi are detected on this medium, they are counted as part of TAMC. - The total combined yeasts/mould count (TYMC) is considered to be equal to the number of CFU found using Sabouraud dextrose agar; if colonies of bacteria are detected on this medium, they are counted as part of TYMC - When an acceptance criterion for microbiological quality is prescribed it is interpreted as follows: 101 CFU : maximum acceptable count＝20 102 CFU: maximum acceptable count＝200, 103 CFU: maximum acceptable count＝2000. |

| - CURRENT REVISION | - SUPERSEDE | - REASON FOR REVIEW |
| --- | --- | --- |
| - 00 | - NIL | - FIRST ISSUE |
| - 01 | - 00 | - Review of the procedures - Change in header and footer format |