MATERIAL NAME: ASPARTAME

SOP Ref. No.: BML-QDG-QA-105                		       		 EFFECTIVE DATE: JAN., 2024

| S/No | TESTS REQUIRED | SPECIFICATIONS | SPECIFICATION REFERENCES |
| --- | --- | --- | --- |
|  | Description | A white or almost white, slightly hygroscopic, crystalline powder. | BP 2023 |
|  | Solubility | Sparingly soluble or slightly soluble in water and in ethanol (96 percent), practically insoluble in hexane and in methylene chloride. | BP 2023 |
|  | Identification | A. The solution shows absorption maxima at 247 nm, 252 nm, 258 nm and 264 nm. B. IR spectrophotometry, comparing with the spectrum obtained with aspartame CRS. C.The spot in the chromatogram obtained with the test solution is similar in position, colour and size to the spot in the chromatogram obtained with the reference solution. D. A brownish – red colour is produced. | BP 2023 |
|  | Appearance of solution | Solution S is clear and not more intensely coloured than reference solution GY6. | BP 2023 |
|  | Conductivity | Not more than 30 microsiemen cm-1 | BP 2023 |
|  | Specific optical rotation | +14.5 to +16.5 | BP 2023 |
|  | Related Substances | Limits: ---impurity A not more than the area of the principal peak in the chromatogram obtained with reference solution ( a) (1.5 %) --- impurity C: not more than the area of the principal peak in the chromatogram obtained with reference solution ( b) (0.5 %) --- sum of impurities: other than A and C: not more than the area of the principal peak in the chromatogram obtained with reference solution ( c) (1.5 %) --- disregard limit: disregard any peak due to the solvent. | BP 2023 |
|  | Loss on drying | Maximum 4.5 % | BP 2023 |
|  | Sulphated Ash | Maximum 0.2 % | BP 2023 |
|  | Assay | 98.0 % - 102.0% | BP 2023 |
|  | Microbial Contamination; TMC Total Yeast and Mould count Pathogens | NMT 1000cfu/g NMT 100cfu/g Must be absent | BP 2023 |

STORAGE: Should be stored in an airtight container.