| S/N | TEST | DESCRIPTIONS OF PROCEDURE |
| --- | --- | --- |
|  | DESCRIPTION | Describe the colour and other characteristics of the material. |
|  | SOLUBILITY | Determine solubility of material in water, ethanol (96%), glycerol and liquid paraffin. |
|  | IDENTIFICATION | A. The infrared absorption spectrum, Appendix II A, compare sample with the reference spectrum of bronopol. B. Dissolve 0.1 g in 10 mL of water, add 10 mL of 7.5msodium hydroxide and, carefully with constant stirring and cooling, 0.5 g of nickel-aluminium alloy. Allow the reaction to subside, filter and carefully neutralise with nitric acid. C. Determine the melting point after drying over phosphorus pentoxide at a pressure not exceeding 0.7 kPa, about 130°. |
|  | ACIDITY OR ALKALINITY | Determine pH of a 1% w/v solution. |
|  | RELATED SUBSTANCES | ### Related substances Carry out the method for liquid chromatography, Appendix III D, using the following solutions in the mobile phase. (1) 0.2% w/v of the substance being examined. (2) Dilute a volume of solution (1) to produce a solution containing 0.0002% w/v of the substance being examined. (3) 0.001% w/v each of 2-methyl-2-nitropropan-1,3-diol and tris(hydroxymethyl)nitromethane. (4) 0.0002% w/v each of 2-methyl-2-nitropropane-1,3-diol, 2-nitroethanol, sodium bromide and tris(hydroxymethyl)nitromethane and 0.2% w/v of the substance being examined. ### Chromatographic conditions (a) Use a stainless steel column (15 cm × 4.6 mm) packed with octadecylsilyl silica gel for chromatography (5 μm) (Phenomenex Luna C18 (2) is suitable). (b) Use isocratic elution and the mobile phase described below. (c) Use a flow rate of 1 mL per minute. (d)Use a column temperature of 35°. (e)Use a detection wavelength of 214 nm. (f)Inject 20 µL of each solution (g)For solution (1) allow the chromatography to proceed for at least 3 times the retention time of the principal peak. ### Mobile phase 1 volume of a 10% v/v solution of orthophosphoric acid, 10 volumes of acetonitrile and 189 volumes of water, adjust the pH to 3.0 using 2msodium hydroxide. |
|  | SULPHATED ASH | Determine the sulfated ash on the sample (1g at 8000C) |
|  | WATER | Determine on 5g of sample |
|  | ASSAY | In a flask fitted with a reflux condenser dissolve 0.4 g in 15 mL of water and add 15 mL of 7.5msodium hydroxide. Slowly, with caution, add 2 g of nickel-aluminium alloy through the reflux condenser, agitating the flask whilst cooling under running water. Allow the mixture to stand for 10 minutes and boil for 1 hour. Cool and filter under reduced pressure, washing the condenser, flask and residue with 150 mL of water. Combine the filtrate and washings, add 25 mL of nitric acid and 40 mL of 0.1msilver nitrate VS, shake vigorously and titrate with 0.1m ammonium thiocyanate VS using ammonium iron(III) sulfate solution R2 as indicator. Repeat the operation without the substance being examined. The difference between the titrations represents the amount of silver nitrate required. Each mL of 0.1msilver nitrate VS is equivalent to 20.00 mg of C3H6BrNO4. |
|  | MICROBIAL ENUMERATION TEST | Determination of total aerobic microbial count (TAMC) and total combined yeasts and molds count (TYMC) by the pour plate method. - Slightly open the lid of the petri plate and dispense 1 ml of diluted sample in the center of the Petri plate using a sterile calibrated pipette. - Open the lid of the prepared media. (soya bean digest agar for TAMC and Sabouraud Dextrose Agar for TYMC) - Pour about 15-20ml of the sterilized molten media at the appropriate temperature above the sample. - Close the lid of the plate then mix the sample in the media properly by gently swirling the plate. The plate is generally swirled in an “S” or “8” shape. - Allow the media to completely solidify. - Simultaneously perform the negative control by plating 1ml of diluent used instead of the sample for each medium. - Incubate the plate in an inverted position under suitable incubation conditions (For TAMC Incubate the plates of Soybean Casein Digest Agar at 30°C-35°C for 48hours and For TYMC incubate the plates of Sabouraud Dextrose Agar at 20°C - 25°C for 72hours Test for Absence of Pathogen Test for E. coli: Using same dilution - Transfer 1ml of the sample into a sterile McCartney bottle containing 9ml of sterile MacConkey broth. - Incubate the MacConkey broth bottle at 300C-350C for 24 hours. - After incubation shake the tube and transfer loop full portion of MacConkey broth on the surface of the pre incubated Eosin Methylene Blue Agar petri plate. - Incubate plates at 30°C to 35°C for 48 hours. - For negative control, proceeding same as above except without sample, streak a portion of incubated sterile MacConkey broth medium on the surface of pre incubated Eosin Methylene Blue Agar. - Observe and record the presence or absence of specified microorganisms. - Include positive and negative controls in each batch of tests. - Verify the performance of media and reagents used in the analysis. Interpretation of Results - The bacterial count should not exceed 102cfu/ml while that of the fungal count should not be more than 102cfu/ml. There should be absence of fecal coliform (E coli). - If growth occurs in the negative control plate, the test is invalid, failed negative control must be investigated - The total aerobic microbial count (TAMC) is considered to be equal to the number of CFU found using casein soya bean digest agar; if colonies of fungi are detected on this medium, they are counted as part of TAMC. - The total combined yeasts/mould count (TYMC) is considered to be equal to the number of CFU found using Sabouraud dextrose agar; if colonies of bacteria are detected on this medium, they are counted as part of TYMC - When an acceptance criterion for microbiological quality is prescribed it is interpreted as follows: 101 CFU : maximum acceptable count＝20 102 CFU: maximum acceptable count＝200, 103 CFU: maximum acceptable count＝2000. |

| - CURRENT REVISION | - SUPERSEDE | - REASON FOR REVIEW |
| --- | --- | --- |
| - 00 | - NIL | - FIRST ISSUE |
| - 01 | - 00 | - Review of the procedures - Change in header and footer format |