MATERIAL NAME: CALCIUM PANTOTHENATE

SOP Ref. No.: BML-QDG-QA-105                		       		 EFFECTIVE DATE: JAN., 2024

| S/No | TESTS REQUIRED | SPECIFICATIONS | SPECIFICATION REFERENCES |
| --- | --- | --- | --- |
|  | Description | White or almost white, slightly hygroscopic powder. | BP 2023 |
|  | Solubility | Freely soluble in water, slightly soluble in ethanol (96 per cent) and practically insoluble in heptane. | BP 2023 |
|  | Identification | A.+ 25.5 to + 27.5 (dried substance) B. Infrared absorption spectrophotometry. Comparison calcium pantothenate CRS. C.The principal spot in the chromatogram obtained with the test solution is similar in position, colour and size to the principal spot in the chromatogram obtained with the reference solution. D. It gives reaction (a) of calcium | BP 2023 |
|  | Apperance of Solution | Solution S is clear and colourless | BP 2023 |
|  | pH | 6.8 to 8.0 | BP 2023 |
|  | Specific Optical Rotation | + 25.5 to + 27.5 (dried substance). | BP 2023 |
|  | Impurities A and other aminocarboxylic acid impurities | Maximum 0.50 per cent. | BP 2023 |
|  | Related Substances | Limits: — impurity B: maximum 0.8 per cent; — impurity C: maximum 0.3 per cent; — impurity E: maximum 0.25 per cent; — impurity H: maximum 0.15 per cent; — unspecified impurities: for each impurity, maximum 0.10 per cent; — total: maximum 1.2 per cent; — reporting threshold: 0.05 per cent; disregard any peak due to impurity A (eluting before 1 min). | BP 2023 |
|  | Chloride | Maximum 200 ppm. | BP 2023 |
|  | Loss on drying | Maximum 3.0 per cent. | BP 2023 |
|  | Assay | 98.0 % to 101.0 % (dried substance). | BP 2023 |
|  | Microbial Contamination; TMC Total Yeast and Mould count Pathogens | NMT 1000cfu/g NMT 100cfu/g Must be absent | BP 2023 |