| S/N | TEST | DESCRIPTIONS OF PROCEDURE |
| --- | --- | --- |
|  | DESCRIPTION | Determine the colour and appearance of material |
|  | SOLUBILITY | Determine its solubility in water and in ethanol (96 per cent). |
|  | IDENTIFICATION | First identification: C, D. Second identification: A, B, D. A. Dissolve 0.100 g in water R and dilute to 100.0 mL with the same solvent. Dilute 1.0 mL of this solution to 100.0 mL with water R. Examined between 210 nm and 370 nm B. Infrared absorption spectrophotometry Comparison chloroquine sulfate CRS. C. Dissolve 25 mg in 20 mL of water R and add 8 mL of picric acid solution R1. The precipitate, washed with water R, with alcohol R and finally with methylene chloride R. D. Subject to reaction (b) of phosphates. Mix 1 mL of the prescribed solution with 2 mL of molybdovanadic reagent R. A yellow colour develops. |
|  | APPEARANCE OF SOLUTION | ### Solution S Dissolve 2.5 g in carbon dioxide-free water R and dilute to 25 mL with the same solvent. Observe the clarity of Solution S and a five fold dilution of solution S. Compare it colour intensity with reference solution BY5 or GY5. |
|  | pH | Determine the pH of solution S |
|  | RELATED SUBSTANCES | Thin-layer chromatography Test solution Dissolve 0.50 g of the substance to be examined in water R and dilute to 10 mL with the same solvent. Reference solution (a) Dilute 1 mL of the test solution to 100 mL with water R. Reference solution (b) Dilute 5 mL of reference solution (a) to 10 mL with water R. Apply to the plate 2 µL of each solution. Develop over a path of 12 cm using a mixture of 10 volumes of diethylamine R, 40 volumes of cyclohexane R and 50 volumes of chloroform R. Allow the plate to dry in air. Examine in ultraviolet light at 254 nm. Any spot in the chromatogram obtained with the test solution, apart from the principal spot, is not more intense than the spot in the chromatogram obtained with reference solution (a) (1.0 per cent) and not more than one such spot is more intense than the spot in the chromatogram obtained with reference solution (b) (0.5 per cent). |
|  | MELTING POINT | Determine the melting point. |
|  | LOSS ON DRYING | Determine the Loss on drying on 1.000 g by drying in an oven at 105 °C. |
|  | ASSAY | Dissolve 0.200 g in 50 mL of anhydrous acetic acid R. Titrate with 0.1 M perchloric acid determining the end-point potentiometrically (2.2.20). 1 mL of 0.1 M perchloric acid is equivalent to 25.79 mg of C18H32ClN3O8P2. |
|  | MICROBIAL ENUMERATION TEST | Determination of total aerobic microbial count (TAMC) and total combined yeasts and molds count (TYMC) by the pour plate method. - Slightly open the lid of the petri plate and dispense 1 ml of diluted sample in the center of the Petri plate using a sterile calibrated pipette. - Open the lid of the prepared media. (soya bean digest agar for TAMC and Sabouraud Dextrose Agar for TYMC) - Pour about 15-20ml of the sterilized molten media at the appropriate temperature above the sample. - Close the lid of the plate then mix the sample in the media properly by gently swirling the plate. The plate is generally swirled in an “S” or “8” shape. - Allow the media to completely solidify. - Simultaneously perform the negative control by plating 1ml of diluent used instead of the sample for each medium. - Incubate the plate in an inverted position under suitable incubation conditions (For TAMC Incubate the plates of Soybean Casein Digest Agar at 30°C-35°C for 48hours and For TYMC incubate the plates of Sabouraud Dextrose Agar at 20°C - 25°C for 72hours Test for Absence of Pathogen Test for E. coli: Using same dilution - Transfer 1ml of the sample into a sterile McCartney bottle containing 9ml of sterile MacConkey broth. - Incubate the MacConkey broth bottle at 300C-350C for 24 hours. - After incubation shake the tube and transfer loop full portion of MacConkey broth on the surface of the pre incubated Eosin Methylene Blue Agar petri plate. - Incubate plates at 30°C to 35°C for 48 hours. - For negative control, proceeding same as above except without sample, streak a portion of incubated sterile MacConkey broth medium on the surface of pre incubated Eosin Methylene Blue Agar. - Observe and record the presence or absence of specified microorganisms. - Include positive and negative controls in each batch of tests. - Verify the performance of media and reagents used in the analysis. Interpretation of Results - The bacterial count should not exceed 102cfu/ml while that of the fungal count should not be more than 102cfu/ml. There should be absence of fecal coliform (E coli). - If growth occurs in the negative control plate, the test is invalid, failed negative control must be investigated - The total aerobic microbial count (TAMC) is considered to be equal to the number of CFU found using casein soya bean digest agar; if colonies of fungi are detected on this medium, they are counted as part of TAMC. - The total combined yeasts/mould count (TYMC) is considered to be equal to the number of CFU found using Sabouraud dextrose agar; if colonies of bacteria are detected on this medium, they are counted as part of TYMC - When an acceptance criterion for microbiological quality is prescribed it is interpreted as follows: 101 CFU : maximum acceptable count＝20 102 CFU: maximum acceptable count＝200, 103 CFU: maximum acceptable count＝2000. |

| - CURRENT REVISION | - SUPERSEDE | - REASON FOR REVIEW |
| --- | --- | --- |
| - 00 | - NIL | - FIRST ISSUE |
| - 01 | - 00 | - Review of the procedures - Change in header and footer format |