| S/N | TEST | DESCRIPTIONS OF PROCEDURE |
| --- | --- | --- |
|  | DESCRIPTION | Determine the colour and appearance of material |
|  | SOLUBILITY | Determine its solubility in water, methanol and ethanol (96 percent). |
|  | IDENTIFICATION | A. Add several drops of dilute ammonia R2(about 2M) to 1 mL of solution S . A blue precipitate is formed. Further add dilute ammonia R2 B. Loss on drying . Determined on 0.500 g by drying in an oven at 250 ± 10 °C C. Dilute 1 mL of solution S to 5 mL with water R. Add 1 mL of dilute hydrochloric acid R and 1 mL of barium chloride solution R1. |
|  | APPEARANCE OF SOLUTION | ### Solution S Dissolve 1.6 g in water R and dilute to 50 mL with the same solvent. Dilute 10 mL of solution S to 15 mL with water R. To 15 mL of the prescribed solution add 1 mL of dilute nitric acid R and pour the mixture as a single addition into a test-tube containing 1 mL of silver nitrate solution R2. Prepare a standard in the same manner using 10 mL of chloride standard solution (5 ppm Cl) R and 5 mL of water R. Examine the tubes laterally against a black background. |
|  | CHLORIDES | After standing for 5 min protected from light, any opalescence in the test solution is not more intense than that in the standard. |
|  | IRON | Atomic absorption spectrometry. Test solution Dissolve 0.32 g in 10 mL of water R, add 2.5 mL of lead-free nitric acid R and dilute to 25.0 mL with water R. Reference solutions Prepare the reference solutions using iron standard solution (20 ppm Fe) R, adding 2.5 mL of lead-free nitric acid R and diluting to 25.0 mL with water R. Source Iron hollow-cathode lamp. Wavelength 248.3 nm. Atomisation device Air-acetylene flame. Copper may form explosive acetylides with acetylene. Therefore, clean the burner thoroughly before any residues become dry. |
|  | LEAD | Atomic absorption spectrometry . Test solution Dissolve 1.6 g in 10 mL of water R, add 2.5 mL of lead-free nitric acid R and dilute to 25.0 mL with water R. Reference solutions Prepare the reference solutions using lead standard solution (100 ppm Pb) R, adding 2.5 mL of lead-free nitric acid R and diluting to 25.0 mL with water R. Source Lead hollow-cathode lamp. Wavelength 217.0 nm. Atomisation device Air-acetylene flame. Copper may form explosive acetylides with acetylene. Therefore, clean the burner thoroughly before any residues become dry. |
|  | LOSS ON DRYING | Determined on 0.500 g by drying in an oven at 250 ± 10 °C |
|  | ASSAY | Dissolve 0.125 g in 50 mL of water R. Add 2 mL of sulfuric acid R and 3 g of potassium iodide R. Titrate with 0.1 M sodium thiosulfate, using 1 mL of starch solution R, added towards the end of the titration. 1 mL of 0.1 M sodium thiosulfate is equivalent to 15.96 mg of CuSO4. |
|  | MICROBIAL ENUMERATION TEST | Determination of total aerobic microbial count (TAMC) and total combined yeasts and molds count (TYMC) by the pour plate method. - Slightly open the lid of the petri plate and dispense 1 ml of diluted sample in the center of the Petri plate using a sterile calibrated pipette. - Open the lid of the prepared media. (soya bean digest agar for TAMC and Sabouraud Dextrose Agar for TYMC) - Pour about 15-20ml of the sterilized molten media at the appropriate temperature above the sample. - Close the lid of the plate then mix the sample in the media properly by gently swirling the plate. The plate is generally swirled in an “S” or “8” shape. - Allow the media to completely solidify. - Simultaneously perform the negative control by plating 1ml of diluent used instead of the sample for each medium. - Incubate the plate in an inverted position under suitable incubation conditions (For TAMC Incubate the plates of Soybean Casein Digest Agar at 30°C-35°C for 48hours and For TYMC incubate the plates of Sabouraud Dextrose Agar at 20°C - 25°C for 72hours Test for Absence of Pathogen Test for E. coli: Using same dilution - Transfer 1ml of the sample into a sterile McCartney bottle containing 9ml of sterile MacConkey broth. - Incubate the MacConkey broth bottle at 300C-350C for 24 hours. - After incubation shake the tube and transfer loop full portion of MacConkey broth on the surface of the pre incubated Eosin Methylene Blue Agar petri plate. - Incubate plates at 30°C to 35°C for 48 hours. - For negative control, proceeding same as above except without sample, streak a portion of incubated sterile MacConkey broth medium on the surface of pre incubated Eosin Methylene Blue Agar. - Observe and record the presence or absence of specified microorganisms. - Include positive and negative controls in each batch of tests. - Verify the performance of media and reagents used in the analysis. Interpretation of Results - The bacterial count should not exceed 102cfu/ml while that of the fungal count should not be more than 102cfu/ml. There should be absence of fecal coliform (E coli). - If growth occurs in the negative control plate, the test is invalid, failed negative control must be investigated - The total aerobic microbial count (TAMC) is considered to be equal to the number of CFU found using casein soya bean digest agar; if colonies of fungi are detected on this medium, they are counted as part of TAMC. - The total combined yeasts/mould count (TYMC) is considered to be equal to the number of CFU found using Sabouraud dextrose agar; if colonies of bacteria are detected on this medium, they are counted as part of TYMC - When an acceptance criterion for microbiological quality is prescribed it is interpreted as follows: 101 CFU : maximum acceptable count＝20 102 CFU: maximum acceptable count＝200, 103 CFU: maximum acceptable count＝2000. |

| - CURRENT REVISION | - SUPERSEDE | - REASON FOR REVIEW |
| --- | --- | --- |
| - 00 | - NIL | - FIRST ISSUE |
| - 01 | - 00 | - Review of the procedures - Change in header and footer format |