MATERIAL NAME: CHLORPHENIRAMINE MALEATE

SOP Ref. No.: BML-QDG-QA-105                		       		 EFFECTIVE DATE: JAN., 2024

| S/No | TESTS REQUIRED | SPECIFICATIONS | SPECIFICATION REFERENCES |
| --- | --- | --- | --- |
|  | Description | White or almost white, crystalline powder. | BP 2023 |
|  | Solubility | Freely soluble in water, soluble in ethanol (96%). | BP 2023 |
|  | Identification | A. Melting point: 130 0C to 135 0C. B. Infrared absorption spectrophotometry comparing with the spectrum obtained for chlorpheniramine maleate CRS. C. Optical rotation on solution S: -- 0.100 to +0.100 | BP 2023 |
|  | Appearance of solution | Solution S is clear and not more intensely coloured than reference solution BY6. | BP 2023 |
|  | Optical rotation |  | BP 2023 |
|  | Related substances | -- 0.100 to +0.100 determined on solution S -- impurity A: not more than 0.4 times the area of the principal peak in the chromatogram obtained with reference solution (a) (0.2 %). -- impurities B, C, D: for each impurity, not more than 0.2 times the area of the principal peak in the chromatogram obtained with reference solution (a) (0.1%); -- unspecified impurities: for each impurity, not more than 0.2 times the area of the principal peak in the chromatogram obtained with reference solution (a) (0.10 %); -- total: not more than the area of the principal peak in the chromatogram obtained with reference solution (a) (0.5 %); -- disregard limit: the area of the principal peak in the chromatogram obtained with reference solution (b) (0.05 %); disregard the peak due to maleic acid. | BP 2023 |
|  | Loss on drying | Maximum 0.5 % | BP 2023 |
|  | Sulphated ash | Maximum 0.1 % | BP 2023 |
|  | Assay | 98.0 % - 101.0 % (dried substance) | BP 2023 |
|  | Microbial Contamination; TMC Total Yeast and Mould count Pathogens | NMT 1000cfu/g NMT 100cfu/g Must be absent | BP 2023 |

STORAGE: Should be properly stored protected from light.