| S/N | TEST | DESCRIPTIONS OF PROCEDURE |
| --- | --- | --- |
|  | DESCRIPTION | Determine the colour and appearance of material |
|  | SOLUBILITY | Determine its solubility in water, in methanol and in ethanol (96 per cent). |
|  | IDENTIFICATION | A.Infrared absorption spectrophotometry. Comparison Cyproheptadine Hcl CRS. B.Dissolve 20 mg in 2 mL of methanol R. |
|  | ACIDITY | Dissolve 0.10 g in water R and dilute to 25 mL with the same solvent. Add 0.1 mL of methyl red solution R. Titrate with 0.40 g/L solution of sodium hydroxide R |
|  | RELATED SUBSTANCES | Liquid chromatography. Buffer solution Dissolve 6.12 g of potassium dihydrogen phosphate R in 900 mL of water for chromatography R, adjust to pH 4.5 with phosphoric acid R and dilute to 1000 mL with water for chromatography R. Test solution Dissolve 40.0 mg of the substance to be examined in mobile phase A and dilute to 20.0 mL with mobile phase A. Reference solution (a) Dilute 1.0 mL of the test solution to 100.0 mL with mobile phase A. Dilute 1.0 mL of this solution to 10.0 mL with mobile phase A. Reference solution (b) Dissolve 2.0 mg of dibenzocycloheptene CRS (impurity A), 2.0 mg of dibenzosuberone CRS (impurity B) and 2.0 mg of cyproheptadine impurity C CRS in mobile phase A, add 1.0 mL of the test solution and dilute to 100.0 mL with mobile phase A. Reference solution (c) Dilute 1.0 mL of reference solution (b) to 10.0 mL with mobile phase A. Column: — size: l = 0.25 m, Ø = 4.6 mm; — stationary phase: end-capped octylsilyl silica gel for chromatography R (5 µm). Mobile phase: — mobile phase A: acetonitrile for chromatography R, buffer solution (40:60 V/V); — mobile phase B: buffer solution, acetonitrile for chromatography R (40:60 V/V); Time (min); Mobile phase A (per cent V/V); Mobile phase B (per cent V/V). 0 - 10.0; 100; 0. 10.0 - 10.1; 100 → 0; 0 → 100. 10.1 - 35; 0; 100. Flow rate 1.0 mL/min. Detection Spectrophotometer at 230 nm. Injection 10 µL. Identification of impurities Use the chromatogram obtained with reference solution (b) to identify the peaks due to impurities A, B and C. Relative retention With reference to cyproheptadine (retention time = about 8 min): impurity C = about 0.7; impurity B = about 2.6; impurity A = about 3.9. System suitability Reference solution (b): — resolution: minimum 7.0 between the peaks due to impurity C and cyproheptadine. |
|  | WATER | Determined on 0.200 g. |
|  | ASSAY | Dissolve 0.250 g in a mixture of 5.0 mL of 0.01 M hydrochloric acid and 50 mL of ethanol (96 per cent) R. Carry out a potentiometric titration (2.2.20), using 0.1 M sodium hydroxide. Read the volume added between the 2 points of inflexion. 1 mL of 0.1 M sodium hydroxide is equivalent to 32.39 mg of C21H22ClN. |
|  | MICROBIAL ENUMERATION TEST | Determination of total aerobic microbial count (TAMC) and total combined yeasts and molds count (TYMC) by the pour plate method. - Slightly open the lid of the petri plate and dispense 1 ml of diluted sample in the center of the Petri plate using a sterile calibrated pipette. - Open the lid of the prepared media. (soya bean digest agar for TAMC and Sabouraud Dextrose Agar for TYMC) - Pour about 15-20ml of the sterilized molten media at the appropriate temperature above the sample. - Close the lid of the plate then mix the sample in the media properly by gently swirling the plate. The plate is generally swirled in an “S” or “8” shape. - Allow the media to completely solidify. - Simultaneously perform the negative control by plating 1ml of diluent used instead of the sample for each medium. - Incubate the plate in an inverted position under suitable incubation conditions (For TAMC Incubate the plates of Soybean Casein Digest Agar at 30°C-35°C for 48hours and For TYMC incubate the plates of Sabouraud Dextrose Agar at 20°C - 25°C for 72hours Test for Absence of Pathogen Test for E. coli: Using same dilution - Transfer 1ml of the sample into a sterile McCartney bottle containing 9ml of sterile MacConkey broth. - Incubate the MacConkey broth bottle at 300C-350C for 24 hours. - After incubation shake the tube and transfer loop full portion of MacConkey broth on the surface of the pre incubated Eosin Methylene Blue Agar petri plate. - Incubate plates at 30°C to 35°C for 48 hours. - For negative control, proceeding same as above except without sample, streak a portion of incubated sterile MacConkey broth medium on the surface of pre incubated Eosin Methylene Blue Agar. - Observe and record the presence or absence of specified microorganisms. - Include positive and negative controls in each batch of tests. - Verify the performance of media and reagents used in the analysis. Interpretation of Results - The bacterial count should not exceed 102cfu/ml while that of the fungal count should not be more than 102cfu/ml. There should be absence of fecal coliform (E coli). - If growth occurs in the negative control plate, the test is invalid, failed negative control must be investigated - The total aerobic microbial count (TAMC) is considered to be equal to the number of CFU found using casein soya bean digest agar; if colonies of fungi are detected on this medium, they are counted as part of TAMC. - The total combined yeasts/mould count (TYMC) is considered to be equal to the number of CFU found using Sabouraud dextrose agar; if colonies of bacteria are detected on this medium, they are counted as part of TYMC - When an acceptance criterion for microbiological quality is prescribed it is interpreted as follows: 101 CFU : maximum acceptable count＝20 102 CFU: maximum acceptable count＝200, 103 CFU: maximum acceptable count＝2000. |

| - CURRENT REVISION | - SUPERSEDE | - REASON FOR REVIEW |
| --- | --- | --- |
| - 00 | - NIL | - FIRST ISSUE |
| - 01 | - 00 | - Review of the procedures - Change in header and footer format |