MATERIAL NAME: DIPHENHYDRAMINE HCL

SOP Ref. No.: BML-QDG-QA-105                		       		 EFFECTIVE DATE: JAN., 2024

| S/No | TESTS REQUIRED | SPECIFICATIONS | SPECIFICATION REFERENCES |
| --- | --- | --- | --- |
|  | Description | White or almost white, crystalline powder. | BP 2023 |
|  | Solubility | Very soluble in water, freely soluble in ethanol (96%). | BP 2023 |
|  | Identification | A. Melting point: 1680C to 1720C B. The solution shows three absorption maxima at 253 nm, 258 nm and 264 nm. The ratio of the absorbance measured at the maximum at 258nm to that measured at the maximum at 253 nm is 1:1 to 1:3. The ratio of the absorbance measured at the maximum at 258 nm to that measured at the maximum at 264 nm is 1:2 to 1:4 C. IR absorption spec, comparing with the spectrum obtained with diphenhydramine hydrochloride CRS. D. It gives reaction (a) of chlorides | BP 2023 |
|  | Appearance of solution | Solution S and a fivefold of solution S are clear. Solution S is not more intensely coloured than reference solution BY6. | BP 2023 |
|  | Acidity or alkalinity | Not more than 0.5 ml of 0.01M sodium hydroxide is required to change the colour of the indicator to yellow. | BP 2023 |
|  | Related substances | -- impurity A: not more than the area of the principal peak in the chromatogram obtained with reference solution (a) (0.5 %). -- any other impurity: not more than 0.6 times the area of the principal peak in the chromatogram obtained with reference solution (a) (0.3 %); -- total: not more than twice the area of the principal peak in the chromatogram obtained with reference solution (a) (1.0 %); -- disregard limit: 0.1 times the area of the principal peak in the chromatogram obtained with reference solution (a) (0.05%). | BP 2023 |
|  | Loss on drying | Maximum 0.5 % | BP 2023 |
|  | Sulphated ash | Maximum 0.1 % | BP 2023 |
|  | Assay | 99.0 % to 101.0 % | BP 2023 |
|  | Microbial Contamination; TMC Total Yeast and Mould count Pathogens | NMT 1000cfu/g NMT 100cfu/g Must be absent | BP 2023 |

STORAGE: Should be protected from light.