| S/N | TEST | DESCRIPTIONS OF PROCEDURE |
| --- | --- | --- |
|  | DESCRIPTION | Determine the colour and appearance of material. |
|  | SOLUBILITY | Determine its solubility in water, in dilute mineral acids and in solutions of alkali hydroxides. |
|  | IDENTIFICATION | Solution S gives the reaction of aluminium. Dissolve about 15 mg of the substance to be examined in 2 mL of water R or use 2 mL of the prescribed solution. Add about 0.5 mL of dilute hydrochloric acid R and about 0.5 mL of thioacetamide reagent R. No precipitate is formed. Add dropwise dilute sodium hydroxide solution R. A gelatinous white precipitate is formed which dissolves on further addition of dilute sodium hydroxide solution R. Gradually add ammonium chloride solution R. The gelatinous white precipitate is re-formed. |
|  | APPEARANCE OF SOULTION | ### Solution S Dissolve 2.5 g in 15 mL of hydrochloric acid R, heating on a water-bath. Dilute to 100 mL with distilled water R |
|  | ALKALINE IMPURITIES | Shake 1.0 g with 20 mL of carbon dioxide-free water R for 1 min and filter. To 10 mL of the filtrate add 0.1 mL of phenolphthalein solution R. Add 0.3ml of 0.1M hydrochloric acid. |
|  | CHLORIDES | Dissolve 0.1 g with heating in 10 mL of dilute nitric acid R and dilute to 100 mL with water R. Dilute 5 mL of the solution to 15 mL with water R. |
|  | NEUTRALIZING CAPACITY | Carry out the test at 37 °C. Disperse 0.5 g in 100 mL of water R, heat, add 100.0 mL of 0.1 M hydrochloric acid, previously heated, and stir continuously; the pH of the solution after 10 min, 15 min and 20 min is not less than 1.8, 2.3 and 3.0 respectively and is at no time greater than 4.5. Add 10.0 mL of 0.5 M hydrochloric acid, previously heated, stir continuously for 1 h and titrate with 0.1 M sodium hydroxide to pH 3.5; |
|  | SULPHATES | Dilute 4 mL of solution S to 100 mL with distilled water R. Add 3 mL of a 250 g/L solution of barium chloride R to 4.5 mL of sulfate standard solution (10 ppm SO4) R1. Shake and allow to stand for 1 min. To 2.5 mL of this suspension add 15 mL of the prescribed solution and 0.5 mL of acetic acid R. Prepare a standard in the same manner using 15 mL of sulfate standard solution (10 ppm SO4) R instead of the prescribed solution. After 5 min, any opalescence in the test solution is not more intense than that in the standard. |
|  | ARSENIC | Determined on 10 mL of solution S. Introduce the prescribed quantity of the substance to be examined into a test-tube containing 4 mL of hydrochloric acid R and about 5 mg of potassium iodide R and add 3 mL of hypophosphorous reagent R. Heat the mixture on a water-bath for 15 min, shaking occasionally. Prepare a standard in the same manner, using 0.5 mL of arsenic standard solution (10 ppm As) R. After heating on the water-bath, any colour in the test solution is not more intense than that in the standard. |
|  | ASSAY | Dissolve 0.800 g in 10 mL of hydrochloric acid R1, heating on a water-bath. Cool and dilute to 50.0 mL with water R. To 10.0 mL of the solution add dilute ammonia R1 until a precipitate begins to appear. Add the smallest quantity of dilute hydrochloric acid R needed to dissolve the precipitate and dilute to 20 mL with water R. Carry out the complexometric titration of aluminium; Introduce 20.0 mL of the prescribed solution into a 500 mL conical flask, add 25.0 mL of 0.1 M sodium edetate and 10 mL of a mixture of equal volumes of a 155 g/L solution of ammonium acetate R and dilute acetic acid R. Boil for 2 min, then cool. Add 50 mL of ethanol R and 3 mL of a freshly prepared 0.25 g/L solution of dithizone R in ethanol R. Titrate the excess of sodium edetate with 0.1 M zinc sulfate until the colourchanges from greenish-blue to reddish-violet. 1 mL of 0.1 M sodium edetate is equivalent to 2.698 mg of Al. 1 mL of 0.1 M sodium edetate is equivalent to 5.098 mg of Al2O3. |
|  | MICROBIAL ENUMERATION TEST | Determination of total aerobic microbial count (TAMC) and total combined yeasts and molds count (TYMC) by the pour plate method. - Slightly open the lid of the petri plate and dispense 1 ml of diluted sample in the center of the Petri plate using a sterile calibrated pipette. - Open the lid of the prepared media. (soya bean digest agar for TAMC and Sabouraud Dextrose Agar for TYMC) - Pour about 15-20ml of the sterilized molten media at the appropriate temperature above the sample. - Close the lid of the plate then mix the sample in the media properly by gently swirling the plate. The plate is generally swirled in an “S” or “8” shape. - Allow the media to completely solidify. - Simultaneously perform the negative control by plating 1ml of diluent used instead of the sample for each medium. - Incubate the plate in an inverted position under suitable incubation conditions (For TAMC Incubate the plates of Soybean Casein Digest Agar at 30°C-35°C for 48hours and For TYMC incubate the plates of Sabouraud Dextrose Agar at 20°C - 25°C for 72hours Test for Absence of Pathogen Test for E. coli: Using same dilution - Transfer 1ml of the sample into a sterile McCartney bottle containing 9ml of sterile MacConkey broth. - Incubate the MacConkey broth bottle at 300C-350C for 24 hours. - After incubation shake the tube and transfer loop full portion of MacConkey broth on the surface of the pre incubated Eosin Methylene Blue Agar petri plate. - Incubate plates at 30°C to 35°C for 48 hours. - For negative control, proceeding same as above except without sample, streak a portion of incubated sterile MacConkey broth medium on the surface of pre incubated Eosin Methylene Blue Agar. - Observe and record the presence or absence of specified microorganisms. - Include positive and negative controls in each batch of tests. - Verify the performance of media and reagents used in the analysis. Interpretation of Results - The bacterial count should not exceed 102cfu/ml while that of the fungal count should not be more than 102cfu/ml. There should be absence of fecal coliform (E coli). - If growth occurs in the negative control plate, the test is invalid, failed negative control must be investigated - The total aerobic microbial count (TAMC) is considered to be equal to the number of CFU found using casein soya bean digest agar; if colonies of fungi are detected on this medium, they are counted as part of TAMC. - The total combined yeasts/mould count (TYMC) is considered to be equal to the number of CFU found using Sabouraud dextrose agar; if colonies of bacteria are detected on this medium, they are counted as part of TYMC - When an acceptance criterion for microbiological quality is prescribed it is interpreted as follows: 101 CFU : maximum acceptable count＝20 102 CFU: maximum acceptable count＝200, 103 CFU: maximum acceptable count＝2000. |

| - CURRENT REVISION | - SUPERSEDE | - REASON FOR REVIEW |
| --- | --- | --- |
| - 00 | - NIL | - FIRST ISSUE |
| - 01 | - 00 | - Review of the procedures - Change in header and footer format |