MATERIAL NAME: FERROUS FUMARATE

SOP Ref. No.: BML-QDG-QA-105                		       		 EFFECTIVE DATE: JAN., 2024

| S/No | TESTS REQUIRED | SPECIFICATIONS | SPECIFICATION REFERENCES |
| --- | --- | --- | --- |
|  | Description | A fine, reddish-orange or reddish-brown powder. | BP 2023 |
|  | Solubility | Slightly soluble in water, very slightly soluble in ethanol (96 %). | BP 2023 |
|  | Identification | A. The principal spot in the chromatogram obtained with the test solution is similar in position and size to the principal spot in the chromatogram obtained with the reference solution B. A dark red semi- solid mass is formed. On adding the mass, with care to 100 mL of water R, an orange-yellow colour develops and the solution shows no fluorescence. C. The filtrate obtained for the test solution in identification test A gives reaction (a) of iron. | BP 2023 |
|  | Sulphates | Maximum 0.2 % | BP 2023 |
|  | Arsenic | Maximum 5 ppm | BP 2023 |
|  | Ferric ion | Maximum 2.0 % | BP 2023 |
|  | Cadmium | Maximum 10 ppm | BP 2023 |
|  | Chromium | Maximum 200 ppm | BP 2023 |
|  | Lead | Maximum 20 ppm | BP 2023 |
|  | Mercury | Maximum 1 ppm | BP 2023 |
|  | Nickel | Maximum 200 ppm | BP 2023 |
|  | Zinc | Maximum 500 ppm | BP 2023 |
|  | Loss on drying | Maximum 1.0 % | BP 2023 |
|  | Assay | 93.0 % - 101.0 % (dried substance) | BP 2023 |
|  | Microbial Contamination; TMC Total Yeast and Mould count Pathogens | NMT 1000cfu/g NMT 100cfu/g Must be absent | BP 2023 |

STORAGE: Should be stored in an airtight container, protected from light..