| S/N | TEST | DESCRIPTIONS OF PROCEDURE |
| --- | --- | --- |
|  | DESCRIPTION | Determine the colour and appearance of material |
|  | SOLUBILITY | Determine its solubility in water, most organic solvents, dilute acids and alkaline solutions. |
|  | IDENTIFICATION | First identification: A, B Second identification: A, C A. Specific optical rotation: Dissolve 0.25 g in 0.1 M sodium hydroxide and dilute to 25.0 mL with the same solvent. B. Examine the chromatograms obtained in the assay. C. Thin-layer chromatography Test solution Dissolve 50 mg of the substance to be examined in a mixture of 2 volumes of concentrated ammonia R and 9 volumes of methanol R and dilute to 100 mL with the same mixture of solvents. Reference solution Dissolve 50 mg of folic acid CRS in a mixture of 2 volumes of concentrated ammonia R and 9 volumes of methanol R and dilute to 100 mL with the same mixture of solvents. Plate TLC silica gel G plate R. Mobile phase concentrated ammonia R, propanol R, ethanol (96 per cent) R (20:20:60 V/V/V). Application 2 µL. Development Over 3/4 of the plate. Drying In air. Detection Examine in ultraviolet light at 365 nm. |
|  | RELATED SUBSTANCES | Liquid chromatography. Test solution Dissolve 0.100 g of the substance to be examined in 5 mL of a 28.6 g/L solution of sodium carbonate R and dilute to 100.0 mL with the mobile phase. Dilute 2.0 mL of this solution to 10.0 mL with the mobile phase. Reference solution (a) Dissolve 0.100 g of folic acid CRS in 5 mL of a 28.6 g/L solution of sodium carbonate R and dilute to 100.0 mL with the mobile phase. Dilute 2.0 mL of this solution to 10.0 mL with the mobile phase. Reference solution (b) To 20 mg of folic acid impurity D CRS add 5 mL of a 28.6 g/L solution of sodium carbonate R, dilute to 100.0 mL with the same solution and mix until completely dissolved. Mix 1.0 mL of this solution with 1.0 mL of reference solution (a) and dilute to 100.0 mL with the mobile phase. Reference solution (c) Dilute 2.0 mL of the test solution to 20.0 mL with the mobile phase. Dilute 1.0 mL of this solution to 20.0 mL with the mobile phase. Reference solution (d) Dissolve 10.0 mg of folic acid impurity A CRS in 1 mL of a 28.6 g/L solution of sodium carbonate R and dilute to 100.0 mL with the mobile phase. Dilute 1.0 mL of this solution to 100.0 mL with the mobile phase. Reference solution (e) To 12.0 mg of folic acid impurity D CRS add 1 mL of a 28.6 g/L solution of sodium carbonate R, dilute to 100.0 mL with the same solution and mix until completely dissolved. Dilute 1.0 mL of this solution to 100.0 mL with the mobile phase. Column: — size: l = 0.25 m, Ø = 4.0 mm; — stationary phase: spherical octylsilyl silica gel for chromatography R (5 µm) with a specific surface area of 350 m2/g, a pore size of 10 nm and a carbon loading of 12.5 per cent. Mobile phase Mix 12 volumes of methanol R and 88 volumes of a solution containing 11.16 g/L of potassium dihydrogen phosphate R and 5.50 g/L of dipotassium hydrogen phosphate R. Flow rate 0.6 mL/min. Detection Spectrophotometer at 280 nm. Injection 5 µL of the test solution and reference solutions (b), (c), (d) and (e). Run time 3 times the retention time of folic acid. Relative retention With reference to folic acid (retention time = about 8.5 min): impurity A = about 0.5; impurity B = about 0.6; impurity C = about 0.9; impurity E = about 1.27; impurity D = about 1.33; impurity F = about 2.2. System suitability: reference solution (b): — resolution: minimum 4.0 between the peaks due to folic acid and impurity D. |
|  | WATER | Determined on 0.150 g |
|  | SULPHATED ASH | Determine the Sulphated Ash on 1.0g of sample. |
|  | ASSAY | Liquid chromatography as described in the test for related substances with the following modification. Injection Test solution and reference solution (a). |
|  | MICROBIAL ENUMERATION TEST | Determination of total aerobic microbial count (TAMC) and total combined yeasts and molds count (TYMC) by the pour plate method. - Slightly open the lid of the petri plate and dispense 1 ml of diluted sample in the center of the Petri plate using a sterile calibrated pipette. - Open the lid of the prepared media. (soya bean digest agar for TAMC and Sabouraud Dextrose Agar for TYMC) - Pour about 15-20ml of the sterilized molten media at the appropriate temperature above the sample. - Close the lid of the plate then mix the sample in the media properly by gently swirling the plate. The plate is generally swirled in an “S” or “8” shape. - Allow the media to completely solidify. - Simultaneously perform the negative control by plating 1ml of diluent used instead of the sample for each medium. - Incubate the plate in an inverted position under suitable incubation conditions (For TAMC Incubate the plates of Soybean Casein Digest Agar at 30°C-35°C for 48hours and For TYMC incubate the plates of Sabouraud Dextrose Agar at 20°C - 25°C for 72hours Test for Absence of Pathogen Test for E. coli: Using same dilution - Transfer 1ml of the sample into a sterile McCartney bottle containing 9ml of sterile MacConkey broth. - Incubate the MacConkey broth bottle at 300C-350C for 24 hours. - After incubation shake the tube and transfer loop full portion of MacConkey broth on the surface of the pre incubated Eosin Methylene Blue Agar petri plate. - Incubate plates at 30°C to 35°C for 48 hours. - For negative control, proceeding same as above except without sample, streak a portion of incubated sterile MacConkey broth medium on the surface of pre incubated Eosin Methylene Blue Agar. - Observe and record the presence or absence of specified microorganisms. - Include positive and negative controls in each batch of tests. - Verify the performance of media and reagents used in the analysis. Interpretation of Results - The bacterial count should not exceed 102cfu/ml while that of the fungal count should not be more than 102cfu/ml. There should be absence of fecal coliform (E coli). - If growth occurs in the negative control plate, the test is invalid, failed negative control must be investigated - The total aerobic microbial count (TAMC) is considered to be equal to the number of CFU found using casein soya bean digest agar; if colonies of fungi are detected on this medium, they are counted as part of TAMC. - The total combined yeasts/mould count (TYMC) is considered to be equal to the number of CFU found using Sabouraud dextrose agar; if colonies of bacteria are detected on this medium, they are counted as part of TYMC - When an acceptance criterion for microbiological quality is prescribed it is interpreted as follows: 101 CFU : maximum acceptable count＝20 102 CFU: maximum acceptable count＝200, 103 CFU: maximum acceptable count＝2000. |

| - CURRENT REVISION | - SUPERSEDE | - REASON FOR REVIEW |
| --- | --- | --- |
| - 00 | - NIL | - FIRST ISSUE |
| - 01 | - 00 | - Review of the procedures - Change in header and footer format |