MATERIAL NAME: GLYCEROL

SOP Ref. No.: BML-QDG-QA-105                		       		 EFFECTIVE DATE: JAN., 2024

| S/No | TESTS REQUIRED | SPECIFICATIONS | SPECIFICATION REFERENCES |
| --- | --- | --- | --- |
|  | Description | Syrupy liquid, unctuous to the touch, colourless or almost colourless, clear, very hygroscopic. | BP 2023 |
|  | Solubility | Miscible with water and with ethanol (96%), slightly soluble in acetone, practically insoluble in fatty oils and in essential oils. | BP 2023 |
|  | Identification | A. Refractive index: 1.470 to 1.475 B. Infrared absorption spectrophotometry, comparing with the spectrum obtained with glycerol CRS (85%). C. A blue ring develops at the interface of the liquids. Within 10 minutes, the blue colour does not diffuse into the lower layer. D. Vapours (acrolein) are evolved which blacken filter paper impregnated with alkaline potassium tetraiodomercurate solution R. | BP 2023 |
|  | Appearance of solution | Solution S is clear. Diluted solution S is colourless. | BP 2023 |
|  | Acidity or alkalinity | Not more than 0.2 mL of 0.1M sodium hydroxide is required to change the colour of the indicator to pink. | BP 2023 |
|  | Refractive Index | 1.470 to 1.475 | BP 2023 |
|  | Aldehydes | Maximum 10 ppm | BP 2023 |
|  | Esters | Not less than 8.0 mL of 0.1 M hydrochloric acid is required to change the colour of the indicator. | BP 2023 |
|  | Impurity A and related substances | -- impurity A: not more than the area of the corresponding peak in the chromatogram obtained with reference solution (c) (0.1 %), -- any other impurity with a retention time less than the retention time of glycerol: not more than the area of the peak due to impurity A in the chromatogram obtained with reference solution (c ) (0.1 %), -- total of all impurities with retention times greater than the retention time of glycerol: not more than 5 times the area of the peak due to impurity A in the chromatogram obtained with reference solution ( c) (0.5 %), -- disregard limit: 0.05 times the area of the peak due to impurity A in the chromatogram obtained with reference solution (e) (0.05 %). | BP 2023 |
|  | Halogenated Compounds | Any opalescence in the solution is not more intense than that in a standard prepared at the same time by mixing 7.0 mL of chloride standard solution (5 ppm Cl) R,4 mL of ethanol (96 percent) R, 0.5 mL of water R, 0.5 mL of nitric acid R and 0.05 mL of silver nitrate solution R2. | BP 2023 |
|  | Sugars | The solution remains clear and blue with no precipitate formed. | BP 2023 |
|  | Chloride | Maximum 10 ppm. | BP 2023 |
|  | Water | Maximum 2 % | BP 2023 |
|  | Sulphated ash | Maximum 0.01 % | BP 2023 |
|  | Assay | 98.0 % m/m to 101.0 % m/m (anhydrous substance). | BP 2023 |
|  | Microbial Contamination; TMC Total Yeast and Mould count Pathogens | NMT 1000cfu/g NMT 100cfu/g Must be absent | BP 2023 |

STORAGE: Should be stored in an airtight container.