| S/N | TEST | DESCRIPTIONS OF PROCEDURE |
| --- | --- | --- |
|  | DESCRIPTION | Determine the colour and appearance of material |
|  | SOLUBILITY | Determine its solubility in water, in acetone, in ethanol (96 per cent) and in methylene chloride. |
|  | IDENTIFICATION | First identification: C. Second identification: A, B, D. A. Determine the Melting point B. Ultraviolet and visible absorption spectrophotometry. Test solution Dissolve 0.100 g in a 103 g/L solution of hydrochloric acid R and dilute to 100.0 mL with the same acid. Dilute 1.0 mL of the solution to 100.0 mL with a 103 g/L solution of hydrochloric acid R. Spectral range 220-350 nm. Absorption maxima At 232 nm and 275 nm. Specific absorbance at the absorption maximum at 232 nm 525 to 575. C. Infrared absorption spectrophotometry. Preparation Discs. Comparisonmetronidazole CRS. D. To about 10 mg add about 10 mg of zinc powder R, 1 mL of water R and 0.25 mL of dilute hydrochloric acid R. Heat on a water-bath for 5 min. Cool. |
|  | APPEARANCE OF SOLUTION | Dissolve 1.0 g in dimethylformamide R and dilute to 10 mL with the same solvent. |
|  | ACIDITY | Dissolve 2.0 g in a mixture of 20 mL of dimethylformamide R and 20 mL of water R, previously neutralised with 0.02 M hydrochloric acid or 0.02 M sodium hydroxide using 0.2 mL of methyl red solution R. Not more than 0.25 mL of 0.02 M sodium hydroxide is required to change the colour of the indicator. |
|  | RELATED SUBSTANCES | Liquid chromatography. Solvent mixture Mobile phase B, mobile phase A (45:55 V/V). Test solution Dissolve 0.100 g of the substance to be examined in the solvent mixture and dilute to 10.0 mL with the solvent mixture. Reference solution (a) Dilute 1.0 mL of the test solution to 100.0 mL with the solvent mixture. Dilute 1.0 mL of this solution to 10.0 mL with the solvent mixture. Reference solution (b) Dissolve 5.0 mg of metronidazole CRS (impurity A), 5.0 mg of metronidazole impurity A CRS (impurity B) and 5.0 mg of benzoic acid CRS (impurity C) in the solvent mixture and dilute to 50.0 mL with the solvent mixture. Dilute 1.0 mL of the solution to 10.0 mL with the solvent mixture. Column: — size: l = 0.25 m, Ø = 4.6 mm; — stationary phase: spherical di-isobutyloctadecylsilyl silica gel for chromatography R (5 µm) with a specific surface area of 180 m2/g, a pore size of 8 nm and a carbon loading of 10 per cent. Mobile phase: — mobile phase A: 1.5 g/L solution of potassium dihydrogen phosphate R adjusted to pH 3.2 with phosphoric acid R; — mobile phase B: acetonitrile R; Time (min); Mobile phase A (per cent V/V); Mobile phase B (per cent V/V). 0 - 5; 80; 20. 5 - 15; 80 → 55; 20 → 45. 15 - 40; 55; 45. Flow rate 1.0 mL/min. Detection Spectrophotometer at 235 nm. Injection 10 µL. Relative retention With reference to metronidazole benzoate (retention time = about 20 min): impurity B = about 0.17; impurity A = about 0.20; impurity C = about 0.7. System suitability Reference solution (b): — resolution: minimum 2.0 between the peaks due to impurities A and B. Limits: — impurities A, B, C: for each impurity, not more than the area of the corresponding peak in the chromatogram obtained with reference solution (b) (0.1 per cent); — any other impurity: for each impurity, not more than the area of the principal peak in the chromatogram obtained with reference solution (a) (0.1 per cent); — total: not more than twice the area of the principal peak in the chromatogram obtained with reference solution (a) (0.2 per cent); — disregard limit: 0.1 times the area of the principal peak in the chromatogram obtained with reference solution (a) (0.01 per cent). |
|  | LOSS ON DRYING | Determined on 1.000 g by drying in an oven at 80 °C for 3 h. |
|  | SULPHATED ASH | Determine the Sulphated Ash on 1.0g at 8000C of sample. |
|  | ASSAY | Dissolve 0.250 g in 50 mL of anhydrous acetic acid R. Titrate with 0.1 M perchloric acid, determining the end-point potentiometrically (2.2.20). 1 mL of 0.1 M perchloric acid is equivalent to 27.53 mg of C13H13N3O4. |
|  | MICROBIAL ENUMERATION TEST | Determination of total aerobic microbial count (TAMC) and total combined yeasts and molds count (TYMC) by the pour plate method. - Slightly open the lid of the petri plate and dispense 1 ml of diluted sample in the center of the Petri plate using a sterile calibrated pipette. - Open the lid of the prepared media. (soya bean digest agar for TAMC and Sabouraud Dextrose Agar for TYMC) - Pour about 15-20ml of the sterilized molten media at the appropriate temperature above the sample. - Close the lid of the plate then mix the sample in the media properly by gently swirling the plate. The plate is generally swirled in an “S” or “8” shape. - Allow the media to completely solidify. - Simultaneously perform the negative control by plating 1ml of diluent used instead of the sample for each medium. - Incubate the plate in an inverted position under suitable incubation conditions (For TAMC Incubate the plates of Soybean Casein Digest Agar at 30°C-35°C for 48hours and For TYMC incubate the plates of Sabouraud Dextrose Agar at 20°C - 25°C for 72hours Test for Absence of Pathogen Test for E. coli: Using same dilution - Transfer 1ml of the sample into a sterile McCartney bottle containing 9ml of sterile MacConkey broth. - Incubate the MacConkey broth bottle at 300C-350C for 24 hours. - After incubation shake the tube and transfer loop full portion of MacConkey broth on the surface of the pre incubated Eosin Methylene Blue Agar petri plate. - Incubate plates at 30°C to 35°C for 48 hours. - For negative control, proceeding same as above except without sample, streak a portion of incubated sterile MacConkey broth medium on the surface of pre incubated Eosin Methylene Blue Agar. - Observe and record the presence or absence of specified microorganisms. - Include positive and negative controls in each batch of tests. - Verify the performance of media and reagents used in the analysis. Interpretation of Results - The bacterial count should not exceed 102cfu/ml while that of the fungal count should not be more than 102cfu/ml. There should be absence of fecal coliform (E coli). - If growth occurs in the negative control plate, the test is invalid, failed negative control must be investigated - The total aerobic microbial count (TAMC) is considered to be equal to the number of CFU found using casein soya bean digest agar; if colonies of fungi are detected on this medium, they are counted as part of TAMC. - The total combined yeasts/mould count (TYMC) is considered to be equal to the number of CFU found using Sabouraud dextrose agar; if colonies of bacteria are detected on this medium, they are counted as part of TYMC - When an acceptance criterion for microbiological quality is prescribed it is interpreted as follows: 101 CFU : maximum acceptable count＝20 102 CFU: maximum acceptable count＝200, 103 CFU: maximum acceptable count＝2000. |

| - CURRENT REVISION | - SUPERSEDE | - REASON FOR REVIEW |
| --- | --- | --- |
| - 00 | - NIL | - FIRST ISSUE |
| - 01 | - 00 | - Review of the procedures - Change in header and footer format |