| S/N | TEST | DESCRIPTIONS OF PROCEDURE |
| --- | --- | --- |
|  | DESCRIPTION | Determine the colour and appearance of material |
|  | SOLUBILITY | Determine its solubility in water, in ethanol (96 per cent) and in methylene chloride. |
|  | IDENTIFICATION | First identification: B. Second identification: A. - Determine the melting point - Infrared absorption spectrophotometry Comparison paracetamol CRS. |
|  | RELATED SUBSTANCES | Liquid chromatography. Solvent mixture methanol R, water R (15:85 V/V). Test solution Dissolve 50.0 mg of the substance to be examined in 0.75 mL of methanol R and dilute to 5.0 mL with water R. Reference solution (a) Dilute 1.0 mL of the test solution to 100.0 mL with the solvent mixture. Dilute 1.0 mL of this solution to 20.0 mL with the solvent mixture. Reference solution (b) Dissolve 5.0 mg of paracetamol impurity J CRS in 25 mL of methanol R and dilute to 250.0 mL with the solvent mixture. Dilute 1.0 mL of the solution to 200.0 mL with the solvent mixture. Reference solution (c) Dissolve 5.0 mg of paracetamol impurity K CRS in the solvent mixture and dilute to 100.0 mL with the solvent mixture. Dilute 1.0 mL of the solution to 10.0 mL with the solvent mixture. Reference solution (d) Dilute 1.0 mL of reference solution (c) to 10.0 mL with the solvent mixture. Reference solution (e) Mix 1 mL of reference solution (a) and 1 mL of reference solution (c) and dilute to 10 mL with the solvent mixture. Column: — size: l = 0.15 m, Ø = 4.6 mm; — stationary phase: end-capped solid core octadecylsilyl silica gel for chromatography R (5 µm); — temperature: 30 °C. Mobile phase: — mobile phase A: dissolve 1.7 g of potassium dihydrogen phosphate R and 1.8 g of dipotassium hydrogen phosphate R in water for chromatography R and dilute to 1000 mL with the same solvent; — mobile phase B: methanol R; Time (min); Mobile phase A (per cent V/V); Mobile phase B (per cent V/V). 0 - 1.5; 95; 5. 1.5 - 14.4; 95 → 90; 5 → 10. 14.4 - 28.8; 90; 10. 28.8 - 57.6; 90 → 66; 10 → 34. 57.6 - 60; 66; 34. Flow rate 1.5 mL/min. Detection Spectrophotometer at 254 nm. Autosampler Set at 5 °C. Injection 50 µL of the test solution and reference solutions (a), (b), (d) and (e). Identification of impurities Use the chromatogram obtained with reference solution (b) to identify the peak due to impurity J; use the chromatogram obtained with reference solution (d) to identify the peak due to impurity K. Relative retention With reference to paracetamol (retention time = about 4 min): impurity K = about 0.4; impurity J = about 10.1. System suitability Reference solution (e): — resolution: minimum 5.0 between the peaks due to impurity K and paracetamol. Calculation of contents: — for impurity J, use the concentration of impurity J in reference solution (b); — for impurity K, use the concentration of impurity K in reference solution (d); — for impurities other than J and K, use the concentration of paracetamol in reference solution (a). Limits: — impurity K: maximum 50 ppm; — impurity J: maximum 10 ppm; — unspecified impurities: for each impurity, maximum 0.05 per cent; — total: maximum 0.2 per cent; — reporting threshold: 0.03 per cent, except for impurities J and K |
|  | LOSS ON DRYING | Determined the Loss on drying on 1.000 g by drying in an oven at 105 °C. |
|  | SULPHATED ASH | Determine the Sulphated Ash on 1.0g of sample @ 8000C. |
|  | ASSAY | Dissolve 0.300 g in a mixture of 10 mL of water R and 30 mL of dilute sulfuric acid R. Boil under a reflux condenser for 1 h, cool and dilute to 100.0 mL with water R. To 20.0 mL of the solution add 40 mL of water R, 40 g of ice, 15 mL of dilute hydrochloric acid R and 0.1 mL of ferroin R. Titrate with 0.1 M cerium sulfate until a greenish-yellow colour is obtained. Carry out a blank titration. 1 mL of 0.1 M cerium sulfate is equivalent to 7.56 mg of C8H9NO2. |
|  | MICROBIAL ENUMERATION TEST | Determination of total aerobic microbial count (TAMC) and total combined yeasts and molds count (TYMC) by the pour plate method. - Slightly open the lid of the petri plate and dispense 1 ml of diluted sample in the center of the Petri plate using a sterile calibrated pipette. - Open the lid of the prepared media. (soya bean digest agar for TAMC and Sabouraud Dextrose Agar for TYMC) - Pour about 15-20ml of the sterilized molten media at the appropriate temperature above the sample. - Close the lid of the plate then mix the sample in the media properly by gently swirling the plate. The plate is generally swirled in an “S” or “8” shape. - Allow the media to completely solidify. - Simultaneously perform the negative control by plating 1ml of diluent used instead of the sample for each medium. - Incubate the plate in an inverted position under suitable incubation conditions (For TAMC Incubate the plates of Soybean Casein Digest Agar at 30°C-35°C for 48hours and For TYMC incubate the plates of Sabouraud Dextrose Agar at 20°C - 25°C for 72hours Test for Absence of Pathogen Test for E. coli: Using same dilution - Transfer 1ml of the sample into a sterile McCartney bottle containing 9ml of sterile MacConkey broth. - Incubate the MacConkey broth bottle at 300C-350C for 24 hours. - After incubation shake the tube and transfer loop full portion of MacConkey broth on the surface of the pre incubated Eosin Methylene Blue Agar petri plate. - Incubate plates at 30°C to 35°C for 48 hours. - For negative control, proceeding same as above except without sample, streak a portion of incubated sterile MacConkey broth medium on the surface of pre incubated Eosin Methylene Blue Agar. - Observe and record the presence or absence of specified microorganisms. - Include positive and negative controls in each batch of tests. - Verify the performance of media and reagents used in the analysis. Interpretation of Results - The bacterial count should not exceed 102cfu/ml while that of the fungal count should not be more than 102cfu/ml. There should be absence of fecal coliform (E coli). - If growth occurs in the negative control plate, the test is invalid, failed negative control must be investigated - The total aerobic microbial count (TAMC) is considered to be equal to the number of CFU found using casein soya bean digest agar; if colonies of fungi are detected on this medium, they are counted as part of TAMC. - The total combined yeasts/mould count (TYMC) is considered to be equal to the number of CFU found using Sabouraud dextrose agar; if colonies of bacteria are detected on this medium, they are counted as part of TYMC - When an acceptance criterion for microbiological quality is prescribed it is interpreted as follows: 101 CFU : maximum acceptable count＝20 102 CFU: maximum acceptable count＝200, 103 CFU: maximum acceptable count＝2000. |

| - CURRENT REVISION | - SUPERSEDE | - REASON FOR REVIEW |
| --- | --- | --- |
| - 00 | - NIL | - FIRST ISSUE |
| - 01 | - 00 | - Review of the procedures - Change in header and footer format |