| S/N | TEST | DESCRIPTIONS OF PROCEDURE |
| --- | --- | --- |
|  | DESCRIPTION | Describe the colour, form and any characteristic odour of the material. |
|  | SOLUBILITY | Determine its miscibility with ethanol (96 per cent) and with methylene chloride. |
|  | IDENTIFICATION | First identification: B. Second identification: A. Thin-layer chromatography. Test solution Mix 0.1 g of the substance to be examined with toluene R and dilute to 10 mL with the same solvent. Reference solution Dissolve 50 mg of menthol R, 20 µL of cineole R, 10 mg of thymol R and 10 µL of menthyl acetate R in toluene R and dilute to 10 mL with the same solvent. Plate TLC silica gel F254 plate R (5-40 µm) [or TLC silica gel F254 plate R (2-10 µm)]. Mobile phase ethyl acetate R, toluene R (5:95 V/V). Application 10 µL [or 1 µL] of the reference solution and 20 µL [or 2 µL] of the test solution, as bands of 10 mm [or 8 mm]. Development Over a path of 15 cm [or 6 cm]. Drying In air. Detection A Examine in ultraviolet light at 254 nm. Results A See below the sequence of zones present in the chromatograms obtained with the reference solution and the test solution. Top of the plate. _______; _______. Thymol: a quenching zone. Quenching zones may be present (carvone, pulegone). _______; _______. Reference solution; Test solution. Detection B Treat with anisaldehyde solution R and heat at 100-105 °C for 5-10 min; examine immediately in daylight. Results B See below the sequence of zones present in the chromatograms obtained with the reference solution and the test solution. Furthermore, other less intensely coloured zones may be present in the chromatogram obtained with the test solution. Top of the plate. An intense violet-red zone (near the solvent front) (hydrocarbons) A brownish-yellow zone (menthofuran). _______; _______. Menthyl acetate: a violet-blue zone; A violet-blue zone (menthyl acetate) A greenish-blue zone (menthone). Thymol: a pink zone. Light pink or greyish-blue or greyish-green zones may be present (carvone, pulegone, isomenthone). 1,8-Cineole: a violet-blue or brown zone; A faint violet-blue or brown zone (1,8-cineole). _______; _______. Menthol: an intense blue or violet zone; An intense blue or violet zone (menthol). Reference solution; Test solution. B.Examine the chromatograms obtained in the test for chromatographic profile. Results The characteristic peaks due to limonene, 1,8-cineole, menthone, menthofuran, isomenthone, menthyl acetate and menthol in the chromatogram obtained with the test solution are similar in retention time to those in the chromatogram obtained with reference solution (a). Isopulegol, pulegone and carvone may be present in the chromatogram obtained with the test solution. |
|  | RELATIVE DENSITY | Determine the relative density of the material. |
|  | REFRACTIVE INDEX | Determine the refractive index of the material. |
|  | OPTICAL ROTATION | Determine the optical rotation of the material. |
|  | ACID VALUE | Determine the acid value on 5.0 g of the material diluted in 50 mL of the prescribed mixture of solvents. |
|  | MINT OIL | Examine the chromatograms obtained in the test for chromatographic profile. Results The chromatogram obtained with the test solution does not show a peak with the retention time of isopulegol that has an area of more than 0.2 per cent of the total area. |
|  | CHROMATOGRAPHIC PROFILE | Gas chromatography : Use the normalisation procedure. Test solution Mix 0.20 mL of the substance to be examined with heptane R and dilute to 10.0 mL with the same solvent. Reference solution (a) Dissolve 10 µL of limonene R, 20 µL of cineole R, 40 µL of menthone R, 10 µL of menthofuran R, 10 µL of (+)-isomenthone R, 40 µL of menthyl acetate R, 20 µL of isopulegol R, 60 mg of menthol R, 20 µL of pulegone R, 10 µL of piperitone R and 10 µL of carvone R in heptane R and dilute to 10.0 mL with the same solvent. Reference solution (b) Dissolve 5 µL of isopulegol R in heptane R and dilute to 10.0 mL with the same solvent. Dilute 0.1 mL of the solution to 5.0 mL with heptane R. Column: — material: fused silica; — size: l = 60 m, Ø = 0.25 mm; — stationary phase: macrogol 20 000 R (film thickness 0.25 µm). Carrier gas helium for chromatography R. Flow rate 1.5 mL/min. Split ratio 1:50. Temperature: Time (min); Temperature (°C). Column; 0 - 10; 60. 10 - 70; 60 → 180. 70 - 75; 180. Injection port; 200. Detector; 220. Detection Flame ionisation. Injection 1 µL. Elution order Order indicated in the composition of reference solution (a); record the retention times of these substances. Identification of peaks Using the retention times determined from the chromatogram obtained with reference solution (a), locate the components of reference solution (a) in the chromatogram obtained with the test solution. System suitability Reference solution (a): — resolution: minimum 1.5 between the peaks due to limonene and 1,8-cineole; minimum 1.5 between the peaks due to piperitone and carvone. Determine the percentage content of each of the following components. The limits are within the following ranges: — limonene: 1.0 per cent to 3.5 per cent; — 1,8-cineole: 3.5 per cent to 8.0 per cent; — menthone: 14.0 per cent to 32.0 per cent; — menthofuran: 1.0 per cent to 8.0 per cent; — isomenthone: 1.5 per cent to 10.0 per cent; — menthyl acetate: 2.8 per cent to 10.0 per cent; — isopulegol: maximum 0.2 per cent; — menthol: 30.0 per cent to 55.0 per cent; — pulegone: maximum 3.0 per cent; — carvone: maximum 1.0 per cent; — disregard limit: the area of the principal peak in the chromatogram obtained with reference solution (b) (0.05 per cent). The ratio of 1,8-cineole content to limonene content is a minimum of 2. |
|  | MICROBIAL ENUMERATION TEST | Determination of total aerobic microbial count (TAMC) and total combined yeasts and molds count (TYMC) by the pour plate method. - Slightly open the lid of the petri plate and dispense 1 ml of diluted sample in the center of the Petri plate using a sterile calibrated pipette. - Open the lid of the prepared media. (soya bean digest agar for TAMC and Sabouraud Dextrose Agar for TYMC) - Pour about 15-20ml of the sterilized molten media at the appropriate temperature above the sample. - Close the lid of the plate then mix the sample in the media properly by gently swirling the plate. The plate is generally swirled in an “S” or “8” shape. - Allow the media to completely solidify. - Simultaneously perform the negative control by plating 1ml of diluent used instead of the sample for each medium. - Incubate the plate in an inverted position under suitable incubation conditions (For TAMC Incubate the plates of Soybean Casein Digest Agar at 30°C-35°C for 48hours and For TYMC incubate the plates of Sabouraud Dextrose Agar at 20°C - 25°C for 72hours Test for Absence of Pathogen Test for E. coli: Using same dilution - Transfer 1ml of the sample into a sterile McCartney bottle containing 9ml of sterile MacConkey broth. - Incubate the MacConkey broth bottle at 300C-350C for 24 hours. - After incubation shake the tube and transfer loop full portion of MacConkey broth on the surface of the pre incubated Eosin Methylene Blue Agar petri plate. - Incubate plates at 30°C to 35°C for 48 hours. - For negative control, proceeding same as above except without sample, streak a portion of incubated sterile MacConkey broth medium on the surface of pre incubated Eosin Methylene Blue Agar. - Observe and record the presence or absence of specified microorganisms. - Include positive and negative controls in each batch of tests. - Verify the performance of media and reagents used in the analysis. Interpretation of Results - The bacterial count should not exceed 102cfu/ml while that of the fungal count should not be more than 102cfu/ml. There should be absence of fecal coliform (E coli). - If growth occurs in the negative control plate, the test is invalid, failed negative control must be investigated - The total aerobic microbial count (TAMC) is considered to be equal to the number of CFU found using casein soya bean digest agar; if colonies of fungi are detected on this medium, they are counted as part of TAMC. - The total combined yeasts/mould count (TYMC) is considered to be equal to the number of CFU found using Sabouraud dextrose agar; if colonies of bacteria are detected on this medium, they are counted as part of TYMC - When an acceptance criterion for microbiological quality is prescribed it is interpreted as follows: 101 CFU : maximum acceptable count＝20 102 CFU: maximum acceptable count＝200, 103 CFU: maximum acceptable count＝2000. |

| - CURRENT REVISION | - SUPERSEDE | - REASON FOR REVIEW |
| --- | --- | --- |
| - 00 | - NIL | - FIRST ISSUE |
| - 01 | - 00 | - Review of the procedures - Change in header and footer format |