MATERIAL NAME: PEPPER MINT OIL

SOP Ref. No.: BML-QDG-QA-105                		       		 EFFECTIVE DATE: JAN., 2024

| S/No | TESTS REQUIRED | SPECIFICATIONS | SPECIFICATION REFERENCES |
| --- | --- | --- | --- |
|  | Description | A colourless, pale yellow or pale greenish- yellow liquid with a characteristic odour and taste followed by a sensation of cold. | BP 2023 |
|  | Solubility | Miscible with ethanol (96 %) and with methylene chloride. | BP 2023 |
|  | Identification | Examine the chromatogram obtained in the test for chromatographic profile Results The characteristic peaks due limonene, 1,8-cineole, menthone, menthofuran, isomenthone, menthyl acetate and menthol in the chromatogram obtained with the test solution are similar in retention time to those in the chromatogram obtained with the reference solution (a). Isopulegol, and carvone may be present in the chromatogram obtained with the test solution. | BP 2023 |
|  | Relative density | 0.900 to 0.916 | BP 2023 |
|  | Refractive index | 1.457 to 1.467 | BP 2023 |
|  | Optical rotation | -- 300 to -- 100 | BP 2023 |
|  | Acid value | Maximum 1.4 | BP 2023 |
|  | Mint oil | The chromatogram obtained with the test solution shows no blue zone between the zones due to 1,8- cineole and menthol. b. The chromatogram obtained with the test solution does not show a peak with the retention time of isopulegol that has an area of more than 0.2 % of the total area. | BP 2023 |
|  | Chromatographic profile | The limits for the percentage content of the components are listed in the chromatographic profile method. The ratio of 1, 8- cineole content to limonene content is minimum 2. | BP 2023 |
|  | Microbial Contamination; TMC Total Yeast and Mould count Pathogens | NMT 1000cfu/g NMT 100cfu/g Must be absent | BP 2023 |

STORAGE: Store in a well – filled, airtight container, protected from light, at a temperature not exceeding 25 0C.

TMC= Total Microbial Count, NMT= Not More Than, cfu/g= Colony Forming Unit per Gram