| S/N | TEST | DESCRIPTIONS OF PROCEDURE |
| --- | --- | --- |
|  | DESCRIPTION | Determine the colour and appearance of material |
|  | SOLUBILITY | Determine its solubility in water and in ethanol (96 per cent),in ethyl acetate, methanol, fatty oils and liquid paraffin. |
|  | IDENTIFICATION | First identification: A, D. ♢Second identification: B, C, D, E. A.Infrared absorption spectrophotometry . ComparisonPh. Eur. reference spectrum of polysorbate 80. B. Hydroxyl value . The hydroxyl value IOH is the number that expresses in milligrams the quantity of potassium hydroxide required to neutralise the acid combined by acylation in 1 g of the substance. ## METHOD A Introduce the quantity of the substance to be examined shown in Table 2.5.3.-1 (m g) into a 150 mL acetylation flask fitted with an air condenser, unless another quantity is prescribed in the monograph. Add the quantity of acetic anhydride solution R1 stated in Table 2.5.3.-1 and attach the air condenser. Heat the flask in a water-bath for 1 h keeping the level of the water about 2.5 cm above the level of the liquid in the flask. Withdraw the flask and allow to cool. Add 5 mL of water R through the upper end of the condenser. If a cloudiness appears add sufficient pyridine R to clear it, noting the volume added. Shake the flask and replace in the water-bath for 10 min. Withdraw the flask and allow to cool. Rinse the condenser and the walls of the flask with 5 mL of alcohol R, previously neutralised to phenolphthalein solution R1. Titrate with 0.5 M alcoholic potassium hydroxide using 0.2 mL of phenolphthalein solution R1 as indicator (n1 mL of 0.5 M alcoholic potassium hydroxide). Carry out a blank test under the same conditions (n2 mL of 0.5 M alcoholic potassium hydroxide). ## METHOD B Introduce the prescribed quantity of the substance to be examined (m g) into a perfectly dry 5 mL conical flask fitted with a ground-glass or suitable plastic stopper and add 2.0 mL of propionic anhydride reagent R. Close the flask and shake gently to dissolve the substance. Allow to stand for 2 h unless otherwise prescribed. Remove the stopper and transfer the flask and its contents into a wide-mouthed 500 mL conical flask containing 25.0 mL of a 9 g/L solution of aniline R in cyclohexane R and 30 mL of glacial acetic acid R. Swirl the contents of the flask, allow to stand for 5 min, add 0.05 mL of crystal violet solution R and titrate with 0.1 M perchloric acid until an emerald-green colour is obtained (n1 mL of 0.1 M perchloric acid). Carry out a blank test under the same conditions (n2 mL of 0.1 M perchloric acid). To take account of any water present, determine this (y per cent) by the semi-micro determination of water (2.5.12). The hydroxyl value is then given by the equation: C.Saponification value . The saponification value is the number of mg of potassium hydroxide required to neutralise the free acids and to saponify the esters in 1 g of the substance. Use Method I unless otherwise specified in the monograph. ### Method I (No Ph. Eur. method) Dissolve 35 to 40 g of potassium hydroxide in 20 mL of water and add sufficient ethanol (96%) to produce 1000 mL. Allow to stand overnight and pour off the clear liquid. Weigh 2 g of the substance into a 200-mL flask, add 25.0 mL of the ethanolic solution of potassium hydroxide and boil under a reflux condenser for 1 hour, rotating the contents frequently. While the solution is still hot, titrate the excess of alkali with 0.5m hydrochloric acid VS using 1 mL of phenolphthalein solution R1 as indicator. Repeat the operation without the substance being examined. Calculate the saponification value from the expression 28.05 v/w where v is the difference, in mL, between the titrations and w is the weight, in g, of substance taken. ### Method II (Ph. Eur. method 2.5.6) The saponification value IS is the number that expresses in milligrams the quantity of potassium hydroxide required to neutralise the free acids and to saponify the esters present in 1 g of the substance. Unless otherwise prescribed, use the quantities indicated in Table 2.5.6.-1 for the determination. Introduce the prescribed quantity of the substance to be examined (m g) into a 250 mL borosilicate glass flask fitted with a reflux condenser. Add 25.0 mL of 0.5 M alcoholic potassium hydroxide and a few glass beads. Attach the condenser and heat under reflux for 30 min, unless otherwise prescribed. Add 1 mL of phenolphthalein solution R1 and titrate immediately (while still hot) with 0.5 M hydrochloric acid (n1 mL of 0.5 M hydrochloric acid). Carry out a blank test under the same conditions (n2 mL of 0.5 M hydrochloric acid). D.Composition of fatty acids. This method is not applicable to oils that contain glycerides of fatty acids with epoxy-, hydroepoxy-, hydroperoxy-, aldehyde, ketone, cyclopropyl and cyclopropenyl groups, and conjugated polyunsaturated and acetylenic compounds because of partial or complete destruction of these groups. Test solution Dissolve 0.10 g of the substance to be examined in 2 mL of a 20 g/L solution of sodium hydroxide R in methanol R in a 25 mL conical flask and boil under a reflux condenser for 30 min. Add 2.0 mL of boron trifluoride-methanol solution R through the condenser and boil for 30 min. Add 4 mL of heptane R through the condenser and boil for 5 min. Cool and add 10.0 mL of saturated sodium chloride solution R, shake for about 15 s and add a quantity of saturated sodium chloride solution R such that the upper phase is brought into the neck of the flask. Collect 2 mL of the upper phase, wash with 3 quantities, each of 2 mL, of water R and dry over anhydrous sodium sulfate R. Reference solutions, chromatographic procedure and assessment of chromatograms Where there is no specific prescription in the individual monograph, proceed as described under Method A. E.Dissolve 0.1 g in 5 mL of methylene chloride R. Add 0.1 g of cobalt nitrate R and 0.1 g of potassium thiocyanate R. Stir with a glass rod. |
|  | ACID VALUE | Dissolve 5.0 g in 50 mL of the prescribed mixture of solvents. |
|  | HYDROXYL VALUE | Introduce the quantity of the substance to be examined shown in Table 2.5.3.-1 (m g) into a 150 mL acetylation flask fitted with an air condenser, unless another quantity is prescribed in the monograph. Add the quantity of acetic anhydride solution R1 stated in Table 2.5.3.-1 and attach the air condenser. Heat the flask in a water-bath for 1 h keeping the level of the water about 2.5 cm above the level of the liquid in the flask. Withdraw the flask and allow to cool. Add 5 mL of water R through the upper end of the condenser. If a cloudiness appears add sufficient pyridine R to clear it, noting the volume added. Shake the flask and replace in the water-bath for 10 min. Withdraw the flask and allow to cool. Rinse the condenser and the walls of the flask with 5 mL of alcohol R, previously neutralised to phenolphthalein solution R1. Titrate with 0.5 M alcoholic potassium hydroxide using 0.2 mL of phenolphthalein solution R1 as indicator (n1 mL of 0.5 M alcoholic potassium hydroxide). Carry out a blank test under the same conditions (n2 mL of 0.5 M alcoholic potassium hydroxide). |
|  | PEROXIDE VALUE | Introduce 10.0 g into a 100 mL beaker and dissolve with 20 mL of glacial acetic acid R. Add 1 mL of saturated potassium iodide solution R, mix and allow to stand for 1 min. Add 50 mL of carbon dioxide-free water R and a magnetic stirring bar. Titrate with 0.01 M sodium thiosulfate, determining the end-point potentiometrically (2.2.20). Carry out a blank titration. Determine the peroxide value using the following expression: (n1-n2)×M×1000m n1; =; volume of 0.01 M sodium thiosulfate required for the substance to be examined, in millilitres;. n2; =; volume of 0.01 M sodium thiosulfate required for the blank titration, in millilitres;. M; =; molarity of the sodium thiosulfate solution, in moles per litre;. m; =; mass of the substance to be examined, in grams.. |
|  | SAPONIFICATION VALUE | Dissolve 35 to 40 g of potassium hydroxide in 20 mL of water and add sufficient ethanol (96%) to produce 1000 mL. Allow to stand overnight and pour off the clear liquid. Weigh 2 g of the substance into a 200-mL flask, add 25.0 mL of the ethanolic solution of potassium hydroxide and boil under a reflux condenser for 1 hour, rotating the contents frequently. While the solution is still hot, titrate the excess of alkali with 0.5m hydrochloric acid VS using 1 mL of phenolphthalein solution R1 as indicator. Repeat the operation without the substance being examined. Calculate the saponification value from the expression 28.05 v/w where v is the difference, in mL, between the titrations and w is the weight, in g, of substance taken. |
|  | COMPOSITION OF FATTY ACID | Use the mixture of calibrating substances in Table 2.4.22.-3. Column: — material: fused silica; — size: l = 30 m, Ø = 0.32 mm; — stationary phase: macrogol 20 000 R (film thickness 0.5 µm). Carrier gas helium for chromatography R. Linear velocity 50 cm/s. Temperature: Time (min); Temperature (°C). Column; 0 - 14; 80 → 220. 14 - 54; 220. Injection port; 250. Detector; 250. Detection Flame ionisation. Injection 1 µL. |
|  | ETHYLENE OXIDE AND DIOXAN | Head-space gas chromatography . Ethylene oxide stock solution Dilute 0.5 mL of a commercially available solution of ethylene oxide in methylene chloride (50 mg/mL) to 50.0 mL with water R. [NOTE: the solution is stable for 3 months, if stored in vials with polytetrafluoroethylene-coated silicone membrane crimp caps at -20 °C]. Allow to reach room temperature. Dilute 1.0 mL of this solution to 250.0 mL with water R. Dioxan stock solution Dilute 1.0 mL of dioxan R to 200.0 mL with water R. Dilute 1.0 mL of this solution to 100.0 mL with water R. Acetaldehyde stock solution Weigh about 0.100 g of acetaldehyde R into a 100 mL volumetric flask and dilute to 100.0 mL with water R. Dilute 1.0 mL of this solution to 100.0 mL with water R. Standard solution To 6.0 mL of ethylene oxide stock solution add 2.5 mL of dioxan stock solution and dilute to 25.0 mL with water R. Test solution (a) Weigh 1.00 g of the substance to be examined into a 10 mL head-space vial. Add 2.0 mL of water R, seal the vial immediately with a polytetrafluoroethylene-coated silicon membrane and an aluminum cap. Mix carefully. Test solution (b) Weigh 1.00 g of the substance to be examined into a 10 mL head-space vial. Add 2.0 mL of standard solution, seal the vial immediately with a polytetrafluoroethylene-coated silicon membrane and an aluminum cap. Mix carefully. Reference solution Introduce 2.0 mL of acetaldehyde stock solution and 2.0 mL of ethylene oxide stock solution into a 10 mL head-space vial and seal the vial immediately with a polytetrafluoroethylene-coated silicon membrane and an aluminum cap. Mix carefully. Column: — material: fused silica; — size: l = 50 m, Ø = 0.53 mm; — stationary phase: phenyl(5)methyl(95)polysiloxane R (5 µm). Carrier gas helium for chromatography R. Flow rate 4.0 mL/min. Split ratio 1:3.5. Static head-space conditions that may be used: — equilibration temperature: 80 °C; — equilibration time: 30 min. Temperature: Time (min); Temperature (°C). Column; 0 - 18; 70 → 250. 18 - 23; 250. Injection port; 85. Detector; 250. Detection Flame ionisation. Injection 1.0 mL of test solutions (a) and (b) and of the reference solution. Relative retention With reference to ethylene oxide (retention time = about 6.5 min): acetaldehyde = about 0.9; dioxan = about 1.9. System suitability Reference solution: — resolution: minimum 2.0 between the peaks due to acetaldehyde and ethylene oxide. Calculate the content of ethylene oxide in ppm using the following expression: CEO; =; concentration of added ethylene oxide in test solution (b), in micrograms per millilitre;. Aa; =; peak area of ethylene oxide in the chromatogram obtained with test solution (a);. Ab; =; peak area of ethylene oxide in the chromatogram obtained with test solution (b).. Calculate the content of dioxan in ppm using the following expression: CD; =; concentration of added dioxan in test solution (b), in microlitres per millilitre;. 1.03; =; density of dioxan, in grams per millilitre;. Aa′; =; peak area of dioxan in the chromatogram obtained with test solution (a);. Ab′; =; peak area of dioxan in the chromatogram obtained with test solution (b).. |
|  | WATER | Maximum 3.0 per cent, determined on 1.00 g. |
|  | TOTAL ASH | Determined on 2.0 g . Incinerate 2 to 3 g of the ground drug in a tared platinum or silica dish at a temperature not exceeding 450° until free from carbon, cool and weigh. If a carbon-free ash cannot be obtained in this way, exhaust the charred mass with hot water, collect the residue on an ashless filter paper, incinerate the residue and filter paper, add the filtrate, evaporate to dryness and ignite at a temperature not exceeding 450°. Calculate the percentage of ash with reference to the air-dried drug. |
|  | RELATIUVE DENSITY | Determine the relative density |
|  | VISCOSITY | Determine the Viscosity of the material. About 400 mPa·s at 25 °C. |
|  | MICROBIAL ENUMERATION TEST | Determination of total aerobic microbial count (TAMC) and total combined yeasts and molds count (TYMC) by the pour plate method. - Slightly open the lid of the petri plate and dispense 1 ml of diluted sample in the center of the Petri plate using a sterile calibrated pipette. - Open the lid of the prepared media. (soya bean digest agar for TAMC and Sabouraud Dextrose Agar for TYMC) - Pour about 15-20ml of the sterilized molten media at the appropriate temperature above the sample. - Close the lid of the plate then mix the sample in the media properly by gently swirling the plate. The plate is generally swirled in an “S” or “8” shape. - Allow the media to completely solidify. - Simultaneously perform the negative control by plating 1ml of diluent used instead of the sample for each medium. - Incubate the plate in an inverted position under suitable incubation conditions (For TAMC Incubate the plates of Soybean Casein Digest Agar at 30°C-35°C for 48hours and For TYMC incubate the plates of Sabouraud Dextrose Agar at 20°C - 25°C for 72hours Test for Absence of Pathogen Test for E. coli: Using same dilution - Transfer 1ml of the sample into a sterile McCartney bottle containing 9ml of sterile MacConkey broth. - Incubate the MacConkey broth bottle at 300C-350C for 24 hours. - After incubation shake the tube and transfer loop full portion of MacConkey broth on the surface of the pre incubated Eosin Methylene Blue Agar petri plate. - Incubate plates at 30°C to 35°C for 48 hours. - For negative control, proceeding same as above except without sample, streak a portion of incubated sterile MacConkey broth medium on the surface of pre incubated Eosin Methylene Blue Agar. - Observe and record the presence or absence of specified microorganisms. - Include positive and negative controls in each batch of tests. - Verify the performance of media and reagents used in the analysis. Interpretation of Results - The bacterial count should not exceed 102cfu/ml while that of the fungal count should not be more than 102cfu/ml. There should be absence of fecal coliform (E coli). - If growth occurs in the negative control plate, the test is invalid, failed negative control must be investigated - The total aerobic microbial count (TAMC) is considered to be equal to the number of CFU found using casein soya bean digest agar; if colonies of fungi are detected on this medium, they are counted as part of TAMC. - The total combined yeasts/mould count (TYMC) is considered to be equal to the number of CFU found using Sabouraud dextrose agar; if colonies of bacteria are detected on this medium, they are counted as part of TYMC - When an acceptance criterion for microbiological quality is prescribed it is interpreted as follows: 101 CFU : maximum acceptable count＝20 102 CFU: maximum acceptable count＝200, 103 CFU: maximum acceptable count＝2000. |

| - CURRENT REVISION | - SUPERSEDE | - REASON FOR REVIEW |
| --- | --- | --- |
| - 00 | - NIL | - FIRST ISSUE |
| - 01 | - 00 | - Review of the procedures - Change in header and footer format |