MATERIAL NAME: PYRIDOXINE HYDROCHLORIDE

SOP Ref. No.: BML-QDG-QA-105                		       		 EFFECTIVE DATE: JAN., 2024

| S/No | TESTS REQUIRED | SPECIFICATIONS | SPECIFICATION REFERENCES |
| --- | --- | --- | --- |
|  | Description | White or almost white, crystalline powder. | BP 2023 |
|  | Solubility | Freely soluble in water and slightly soluble in ethanol (96 per cent). | BP 2023 |
|  | Identification | First identification: C, D. Second identification: A, B, D. A.Ultraviolet and visible absorption spectrophotometry Specific absorbances at the absorption maxima: — 425-445 for solution B at 288-296 nm; — 175-195 for solution C at 248-256 nm; — 345-365 for solution C at 320-327 nm. B.Infrared absorption spectrophotometry Comparison pyridoxine hydrochloride CRS. C. Thin-Layer Chromatography The principal spot in the chromatogram obtained with the test solution is similar in position, colour and size to the principal spot in the chromatogram obtained with the reference solution. | BP 2023 |
|  | Appearance of Solution | Solution S is clear and not more intensely coloured than reference solution |  |
|  | pH | 2.4 to 3.0 | BP 2023 |
|  | Related Substances | Limits: — correction factor: for the calculation of content, multiply the peak area of impurity B by 1.5; — impurity B: not more than 1.5 times the area of the principal peak in the chromatogram obtained with reference solution (a) (0.15 per cent); — unspecified impurities: for each impurity, not more than the area of the principal peak in the chromatogram obtained with reference solution (a) (0.10 per cent); — total: not more than twice the area of the principal peak in the chromatogram obtained with reference solution (a) (0.2 per cent); — disregard limit: 0.5 times the area of the principal peak in the chromatogram obtained with reference solution (a) (0.05 per cent). c). | BP 2023 |
|  | Loss on Drying | Maximum 0.5 per cent | BP 2023 |
|  | Sulphated Ash | Maximum 0.1 per cent | BP 2023 |
|  | Assay | 99.0 % to 101.0 % (dried substance). | BP 2023 |
|  | Microbial Contamination; TMC Total Yeast and Mould count Pathogens | NMT 1000cfu/g NMT 100cfu/g Must be absent | BP 2023 |

TMC= Total Microbial Count, NMT= Not More Than, cfu/g= Colony Forming Unit per Gram