| S/N | TEST | DESCRIPTIONS OF PROCEDURE |
| --- | --- | --- |
|  | DESCRIPTION | Determine the colour and appearance of material |
|  | SOLUBILITY | Determine its solubility in water, methylene chloride and in ethanol (96 per cent). |
|  | IDENTIFICATION | First identification: B, E. Second identification: A, C, D, E A. Ultraviolet and visible absorption spectrophotometry Test solution: Dissolve 80.0 mg in a 10 g/L solution of hydrochloric acid R and dilute to 100.0 mL with the same acid. Dilute 10.0 mL of the solution to 100.0 mL with a 10 g/L solution of hydrochloric acid R. B.Infrared absorption spectrophotometry. If the spectra obtained show differences, dissolve the substance to be examined and the reference substance separately in anhydrous ethanol R. Dry the residues and record new spectra using the residues. C.Thin-layer chromatography . Test solution: Dissolve 12 mg of the substance to be examined in water R and dilute to 10 mL with the same solvent. Reference solution: Dissolve 12 mg of salbutamol sulfate CRS in water R and dilute to 10 mL with the same solvent. Plate TLC silica gel plate R. Mobile phase concentrated ammonia R, water R, ethyl acetate R, 2-propanol R, methyl isobutyl ketone R (3:18:35:45:50 V/V/V/V/V). Application 1 µL. Development Over 2/3 of the plate. Drying In air. Detection: Spray with a 1 g/L solution of methylbenzothiazolone hydrazone hydrochloride R in a 90 per cent V/V solution of methanol R, followed by a 20 g/L solution of potassium ferricyanide R in a mixture of 1 volume of concentrated ammonia R1 and 3 volumes of water R, followed by a further spraying with a 1 g/L solution of methylbenzothiazolone hydrazone hydrochloride R in a 90 per cent V/V solution of methanol R. D. Dissolve about 10 mg in 50 mL of a 20 g/L solution of disodium tetraborate R. Add 1 mL of a 30 g/L solution of aminopyrazolone R, 10 mL of methylene chloride R and 10 mL of a 20 g/L solution of potassium ferricyanide R. Shake and allow to separate. E. Subject to reaction (a) of sulfates. Dissolve about 45 mg of the substance to be examined in 5 mL of water R or use 5 mL of the prescribed solution. Add 1 mL of dilute hydrochloric acid R and 1 mL of barium chloride solution R1. A white precipitate is formed. |
|  | APPERANCE OF SOLUTION | Solution S Dissolve 0.250 g in carbon dioxide-free water R and dilute to 25.0 mL with the same solvent. |
|  | OPTICAL ROTATION | Solution S Dissolve 0.250 g in carbon dioxide-free water R and dilute to 25.0 mL with the same solvent. |
|  | ACIDITY/ALKALINITY | To 10 mL of solution S add 0.15 mL of methyl red solution R and 0.2 mL of 0.01 M sodium hydroxide. The solution is yellow. Titrate with 0.01M hydrochloric acid. |
|  | RELATED SUBSTANCES | Liquid chromatography. Test solution Dissolve 20.0 mg of the substance to be examined in mobile phase A and dilute to 50.0 mL with mobile phase A. Reference solution (a) Dissolve 3.0 mg of salbutamol impurity D CRS and 3.0 mg of salbutamol impurity F CRS in mobile phase A and dilute to 50.0 mL with mobile phase A. Dilute 2.0 mL of the solution to 100.0 mL with mobile phase A. Reference solution (b) Dilute 1.0 mL of the test solution to 100.0 mL with mobile phase A. Dilute 1.0 mL of this solution to 10.0 mL with mobile phase A. Reference solution (c) With the aid of ultrasound, dissolve the contents of a vial of salbutamol impurity J CRS in 1.0 mL of the test solution. Reference solution (d) Dissolve 1 mg of salbutamol impurity D CRS in mobile phase A and dilute to 100.0 mL with mobile phase A. Reference solution (e) Dissolve 4 mg of salbutamol sulfate for system suitability CRS (containing impurities C, F, N and O) in mobile phase A, add 0.4 mL of reference solution (d) and dilute to 10.0 mL with mobile phase A. Column: — size: l = 0.15 m, Ø = 4.6 mm; — stationary phase: spherical end-capped octylsilyl silica gel for chromatography R (3 μm); — temperature: 30 °C. Mobile phase: — mobile phase A: dissolve 3.45 g of sodium dihydrogen phosphate monohydrate R in 900 mL of a 0.05 per cent V/V solution of triethylamine R, adjust to pH 3.0 with dilute phosphoric acid R and dilute to 1000 mL with a 0.05 per cent V/V solution of triethylamine R; — mobile phase B: methanol R, acetonitrile R (35:65 V/V); Flow rate 1.0 mL/min. Detection Spectrophotometer at 273 nm. Injection 20 μl of the test solution and reference solutions (a), (b), (c) and (e). Relative retention With reference to salbutamol (retention time = about 7 min): impurity J = about 0.9; impurity C = about 1.6; impurity N = about 1.67; impurity D = about 1.68; impurity F = about 1.77; impurity O = about 1.82. Identification of impurities Use the chromatogram supplied with salbutamol sulfate for system suitability CRS and the chromatogram obtained with reference solution (e) to identify the peaks due to impurities C, D, F, N and O; use the chromatogram obtained with reference solution (c) to identify the peak due to impurity J. System suitability: — peak-to-valley ratio: minimum 1.2, where Hp = height above the baseline of the peak due to impurity N and Hv = height above the baseline of the lowest point of the curve separating this peak from the peak due to impurity D in the chromatogram obtained with reference solution (e); minimum 2.0, where Hp = height above the baseline of the peak due to impurity J and Hv = height above the baseline of the lowest point of the curve separating this peak from the peak due to salbutamol in the chromatogram obtained with reference solution (c). |
|  | BORON | Test solution: To 50 mg of the substance to be examined add 5 mL of a solution containing 13 g/L of anhydrous sodium carbonate R and 17 g/L of potassium carbonate R. Evaporate to dryness on a water-bath and dry at 120 °C. Ignite the residue rapidly until the organic matter has been destroyed, allow to cool and add 0.5 mL of water R and 3.0 mL of a freshly prepared 1.25 g/L solution of curcumin R in glacial acetic acid R. Warm gently to effect solution, allow to cool and add 3.0 mL of a mixture prepared by adding 5 mL of sulfuric acid R, slowly and with stirring, to 5 mL of glacial acetic acid R. Mix and allow to stand for 30 min. Dilute to 100.0 mL with ethanol (96 per cent) R, filter and use the filtrate. Reference solution Dissolve 0.572 g of boric acid R in 1000.0 mL of water R. Dilute 1.0 mL of the solution to 100.0 mL with water R. To 2.5 mL of this solution add 5 mL of a solution containing 13 g/L of anhydrous sodium carbonate R and 17 g/L of potassium carbonate R, and treat this mixture in the same manner as the test solution. Measure the absorbance of the test solution and of the reference solution at the absorption maximum at about 555 nm. The absorbance of the test solution is not greater than that of the reference solution. |
|  | LOSS ON DRYING | Determined the Loss on drying on 1.000 g by drying in an oven at 105 °C. |
|  | SULPHATED ASH | Determine the Sulphated Ash on 1.0g of sample @ 8000C. |
|  | ASSAY | Dissolve 0.400 g in 5 mL of anhydrous formic acid R and add 35 mL of anhydrous acetic acid R. Titrate with 0.1 M perchloric acid, determining the end-point potentiometrically. 1 mL of 0.1 M perchloric acid is equivalent to 57.67 mg of C26H44N2O10S. |
|  | MICROBIAL ENUMERATION TEST | Determination of total aerobic microbial count (TAMC) and total combined yeasts and molds count (TYMC) by the pour plate method. - Slightly open the lid of the petri plate and dispense 1 ml of diluted sample in the center of the Petri plate using a sterile calibrated pipette. - Open the lid of the prepared media. (soya bean digest agar for TAMC and Sabouraud Dextrose Agar for TYMC) - Pour about 15-20ml of the sterilized molten media at the appropriate temperature above the sample. - Close the lid of the plate then mix the sample in the media properly by gently swirling the plate. The plate is generally swirled in an “S” or “8” shape. - Allow the media to completely solidify. - Simultaneously perform the negative control by plating 1ml of diluent used instead of the sample for each medium. - Incubate the plate in an inverted position under suitable incubation conditions (For TAMC Incubate the plates of Soybean Casein Digest Agar at 30°C-35°C for 48hours and For TYMC incubate the plates of Sabouraud Dextrose Agar at 20°C - 25°C for 72hours Test for Absence of Pathogen Test for E. coli: Using same dilution - Transfer 1ml of the sample into a sterile McCartney bottle containing 9ml of sterile MacConkey broth. - Incubate the MacConkey broth bottle at 300C-350C for 24 hours. - After incubation shake the tube and transfer loop full portion of MacConkey broth on the surface of the pre incubated Eosin Methylene Blue Agar petri plate. - Incubate plates at 30°C to 35°C for 48 hours. - For negative control, proceeding same as above except without sample, streak a portion of incubated sterile MacConkey broth medium on the surface of pre incubated Eosin Methylene Blue Agar. - Observe and record the presence or absence of specified microorganisms. - Include positive and negative controls in each batch of tests. - Verify the performance of media and reagents used in the analysis. Interpretation of Results - The bacterial count should not exceed 102cfu/ml while that of the fungal count should not be more than 102cfu/ml. There should be absence of fecal coliform (E coli). - If growth occurs in the negative control plate, the test is invalid, failed negative control must be investigated - The total aerobic microbial count (TAMC) is considered to be equal to the number of CFU found using casein soya bean digest agar; if colonies of fungi are detected on this medium, they are counted as part of TAMC. - The total combined yeasts/mould count (TYMC) is considered to be equal to the number of CFU found using Sabouraud dextrose agar; if colonies of bacteria are detected on this medium, they are counted as part of TYMC - When an acceptance criterion for microbiological quality is prescribed it is interpreted as follows: 101 CFU : maximum acceptable count＝20 102 CFU: maximum acceptable count＝200, 103 CFU: maximum acceptable count＝2000. |

| - CURRENT REVISION | - SUPERSEDE | - REASON FOR REVIEW |
| --- | --- | --- |
| - 00 | - NIL | - FIRST ISSUE |
| - 01 | - 00 | - Review of the procedures - Change in header and footer format |