MATERIAL NAME: SALBUTAMOL SULPHATE

SOP Ref. No.: BML-QDG-QA-105                		       		 EFFECTIVE DATE: JAN., 2024

| S/No | TESTS REQUIRED | SPECIFICATIONS | SPECIFICATION REFERENCES |
| --- | --- | --- | --- |
|  | Description | White or almost white, crystalline powder. | BP 2023 |
|  | Solubility | Freely soluble in water, practically insoluble or very slightly soluble in ethanol (96 per cent) and in methylene chloride. | BP 2023 |
|  | Identification | First identification: C, D. Second identification: A, B, D. A.Ultraviolet and visible absorption spectrophotometry Spectral range 230-350 nm. Absorption maximum At 276 nm. Specific absorbance at the absorption maximum 55 to 64. B.Infrared absorption spectrophotometry Comparison salbutamol sulfate CRS. C. Thin-Layer Chromatography The principal spot in the chromatogram obtained with the test solution is similar in position, colour and size to the principal spot in the chromatogram obtained with the reference solution. D. An orange-red colour develops in the methylene chloride layer. E. It gives reaction (a) of sulfates. | BP 2023 |
|  | Appearance of Solution | Solution S is clear and not more intensely coloured than reference solution |  |
|  | Acidity/Alkalinity | Not more than 0.4 mL of 0.01 M hydrochloric acid is required to change the colour of the indicator from yellow to red. | BP 2023 |
|  | Optical Rotation | -0.10° to + 0.10° |  |
|  | Related Substances | impurities D, F: for each impurity, not more than the area of the corresponding peak in the chromatogram obtained with reference solution (a) (0.3 per cent); — impurities C, N, O: for each impurity, not more than twice the area of the principal peak in the chromatogram obtained with reference solution (b) (0.2 per cent); — unspecified impurities: for each impurity, not more than the area of the principal peak in the chromatogram obtained with reference solution (b) (0.10 per cent); — total: maximum 0.9 per cent; — disregard limit: 0.5 times the area of the principal peak in the chromatogram obtained with reference solution (b) (0.05 per cent). | BP 2023 |
|  | Boron | Maximum 50 ppm. |  |
|  | Loss on Drying | Maximum 0.5 per cent | BP 2023 |
|  | Sulphated Ash | Maximum 0.1 per cent | BP 2023 |
|  | Assay | 98.0 % to 101.0 % (dried substance). | BP 2023 |
|  | Microbial Contamination; TMC Total Yeast and Mould count Pathogens | NMT 1000cfu/g NMT 100cfu/g Must be absent | BP 2023 |

TMC= Total Microbial Count, NMT= Not More Than, cfu/g= Colony Forming Unit per Gram