| S/N | TEST | DESCRIPTIONS OF PROCEDURE |
| --- | --- | --- |
|  | DESCRIPTION | Describe the colour and appearance of the material. |
|  | SOLUBILITY | Determine its solubility in water and in ethanol (90 % v/v). |
|  | IDENTIFICATION | - Subject to the reactions (b) and (c) of benzoates. b) Place 0.2 g of the substance to be examined, treated if necessary as prescribed, in a test-tube. Moisten with 0.2 mL to 0.3 mL of sulfuric acid R. Gently warm the bottom of the tube. A white sublimate is deposited on the inner wall of the tube. c) Dissolve 0.5 g of the substance to be examined in 10 mL of water R or use 10 mL of the prescribed solution. Add 0.5 mL of hydrochloric acid R. The precipitate obtained, after crystallisation from warm water R and drying in vacuo, has a melting point of 120 °C to 124 °C. - Subject to the reaction (a) of sodium. a) Dissolve 0.1 g of the substance to be examined in 2 mL of water R or use 2 mL of the prescribed solution. Add 2 mL of a 150 g/L solution of potassium carbonate R and heat to boiling. No precipitate is formed. Add 4 mL of potassium pyroantimonate solution R and heat to boiling. Allow to cool in iced water and if necessary rub the inside of the test-tube with a glass rod. A dense white precipitate is formed. |
|  | APPEARANCE OF SOLUTION | Observe the clarity of solution S and compare its colour intensity with reference solution Y6. |
|  | ACIDITY OR ALKALINITY | To 10 ml of solution S add 10 ml of carbon dioxide - free water R and 0.2 ml of phenolphthalein solution R. Titrate with 0.1M sodium hydroxide or 0.1M hydrochloric acid to change the colour of the indicator. |
|  | LOSS ON DRYING | Determine the loss on drying on 1.00 g by drying in an oven at 1000C to 1050C. |
|  | HALOGENATED COMPOUND | Maximum 200 ppm for ionised chlorine and maximum 300 ppm for total chlorine. All glassware used must be chloride-free and may be prepared by soaking overnight in a 500 g/L solution of nitric acid R, rinsed with water R and stored full of water R. It is recommended that glassware be reserved exclusively for this test. Test solution To 20.0 mL of solution S add 5 mL of water R and dilute to 50.0 mL with ethanol (96 per cent) R. Determination of ionised chlorine In three 25 mL volumetric flasks, prepare the following solutions. Solution (a) To 4.0 mL of the test solution add 3 mL of dilute sodium hydroxide solution R and 3 mL of ethanol (96 per cent) R. This solution is used to prepare solution A. Solution (b) To 3 mL of dilute sodium hydroxide solution R add 2 mL of water R and 5 mL of ethanol (96 per cent) R. This solution is used to prepare solution B. Solution (c) To 4.0 mL of chloride standard solution (8 ppm Cl) R add 6.0 mL of water R. This solution is used to prepare solution C. In a fourth 25 mL volumetric flask, place 10 mL of water R. To each flask add 5 mL of ferric ammonium sulfate solution R5, mix and add dropwise and with swirling 2 mL of nitric acid R and 5 mL of mercuric thiocyanate solution R. Shake. Dilute the contents of each flask to 25.0 mL with water R and allow the solutions to stand in a water-bath at 20 °C for 15 min. Measure at 460 nm in a 2 cm cell the absorbance (2.2.25) of solution A using solution B as the compensation liquid, and the absorbance of solution C using the solution obtained with 10 mL of water R as the compensation liquid. The absorbance of solution A is not greater than that of solution C. Determination of total chlorine Solution (a) To 10.0 mL of the test solution add 7.5 mL of dilute sodium hydroxide solution R and 0.125 g of nickel-aluminium alloy R and heat on a water-bath for 10 min. Allow to cool to room temperature, filter into a 25 mL volumetric flask and wash the filter with 3 quantities, each of 2 mL, of ethanol (96 per cent) R (a slight precipitate may form that disappears on acidification). Dilute the filtrate and washings to 25.0 mL with water R. This solution is used to prepare solution A. Solution (b) In the same manner, prepare a similar solution replacing the test solution by a mixture of 5 mL of ethanol (96 per cent) R and 5 mL of water R. This solution is used to prepare solution B. Solution (c) To 6.0 mL of chloride standard solution (8 ppm Cl) R add 4.0 mL of water R. This solution is used to prepare solution C. In four 25 mL volumetric flasks, place separately 10 mL of solution (a), 10 mL of solution (b), 10 mL of solution (c) and 10 mL of water R. To each flask add 5 mL of ferric ammonium sulfate solution R5, mix and add dropwise and with swirling 2 mL of nitric acid R and 5 mL of mercuric thiocyanate solution R. Shake. Dilute the contents of each flask to 25.0 mL with water R and allow the solutions to stand in a water-bath at 20 °C for 15 min. Measure at 460 nm in a 2 cm cell the absorbance of solution A using solution B as the compensation liquid, and the absorbance of solution C using the solution obtained with 10 mL of water R as the compensation liquid. The absorbance of solution A is not greater than that of solution C. |
|  | ASSAY | Dissolve 0.250 g in 20 ml of anhydrous acetic acid R, heating to 500C if necessary. Cool. Using 0.05 ml of naphtholbenzein solution R as indicator, titrate with 0.1M perchloric acid until a green colour is obtained. 1 mL of 0.1 M perchloric acid is equivalent to 14.41 mg of C7H5NaO2. |
|  | MICROBIAL ENUMERATION TEST | Determination of total aerobic microbial count (TAMC) and total combined yeasts and molds count (TYMC) by the pour plate method. - Slightly open the lid of the petri plate and dispense 1 ml of diluted sample in the center of the Petri plate using a sterile calibrated pipette. - Open the lid of the prepared media. (soya bean digest agar for TAMC and Sabouraud Dextrose Agar for TYMC) - Pour about 15-20ml of the sterilized molten media at the appropriate temperature above the sample. - Close the lid of the plate then mix the sample in the media properly by gently swirling the plate. The plate is generally swirled in an “S” or “8” shape. - Allow the media to completely solidify. - Simultaneously perform the negative control by plating 1ml of diluent used instead of the sample for each medium. - Incubate the plate in an inverted position under suitable incubation conditions (For TAMC Incubate the plates of Soybean Casein Digest Agar at 30°C-35°C for 48hours and For TYMC incubate the plates of Sabouraud Dextrose Agar at 20°C - 25°C for 72hours Test for Absence of Pathogen Test for E. coli: Using same dilution - Transfer 1ml of the sample into a sterile McCartney bottle containing 9ml of sterile MacConkey broth. - Incubate the MacConkey broth bottle at 300C-350C for 24 hours. - After incubation shake the tube and transfer loop full portion of MacConkey broth on the surface of the pre incubated Eosin Methylene Blue Agar petri plate. - Incubate plates at 30°C to 35°C for 48 hours. - For negative control, proceeding same as above except without sample, streak a portion of incubated sterile MacConkey broth medium on the surface of pre incubated Eosin Methylene Blue Agar. - Observe and record the presence or absence of specified microorganisms. - Include positive and negative controls in each batch of tests. - Verify the performance of media and reagents used in the analysis. Interpretation of Results - The bacterial count should not exceed 102cfu/ml while that of the fungal count should not be more than 102cfu/ml. There should be absence of fecal coliform (E coli). - If growth occurs in the negative control plate, the test is invalid, failed negative control must be investigated - The total aerobic microbial count (TAMC) is considered to be equal to the number of CFU found using casein soya bean digest agar; if colonies of fungi are detected on this medium, they are counted as part of TAMC. - The total combined yeasts/mould count (TYMC) is considered to be equal to the number of CFU found using Sabouraud dextrose agar; if colonies of bacteria are detected on this medium, they are counted as part of TYMC - When an acceptance criterion for microbiological quality is prescribed it is interpreted as follows: 101 CFU : maximum acceptable count＝20 102 CFU: maximum acceptable count＝200, 103 CFU: maximum acceptable count＝2000. |

| - CURRENT REVISION | - SUPERSEDE | - REASON FOR REVIEW |
| --- | --- | --- |
| - 00 | - NIL | - FIRST ISSUE |
| - 01 | - 00 | - Review of the procedures - Change in header and footer format |