| S/N | TEST | DESCRIPTIONS OF PROCEDURE |
| --- | --- | --- |
|  | DESCRIPTION | Describe the colour and appearance of the material. |
|  | SOLUBILITY | Determine its solubility in water and in ethanol (96 %). |
|  | IDENTIFICATION | A. To 5 mL of solution S, add 0.1 mL of phenolphthalein solution R. A pale pink colour is produced. Heat and record your observation. B. Subject to the reaction of carbonates and bicarbonates. Introduce into a test-tube 0.1 g of the substance to be examined and suspend in 2 mL of water R or use 2 mL of the prescribed solution. Add 3 mL of dilute acetic acid R. Close the tube immediately using a stopper fitted with a glass tube bent twice at right angles. The solution or the suspension becomes effervescent and gives off a colourless and odourless gas. Heat gently and collect the gas in 5 mL of barium hydroxide solution R. A white precipitate is formed that dissolves on addition of an excess of hydrochloric acid R1. C. Subject solution S to reaction (a) of sodium. a) Dissolve 0.1 g of the substance to be examined in 2 mL of water R or use 2 mL of the prescribed solution. Add 2 mL of a 150 g/L solution of potassium carbonate R and heat to boiling. No precipitate is formed. Add 4 mL of potassium pyroantimonate solution R and heat to boiling. Allow to cool in iced water and if necessary rub the inside of the test-tube with a glass rod. A dense white precipitate is formed. |
|  | APPEARANCE OF SOLUTION | SOLUTION S: Dissolve 5.0 g in 90 mL of carbon dioxide – free water R and dilute to 100.0 mL with the same solvent. Observe the clarity and colour of solution S. |
|  | CARBONATES | Determine the pH of freshly prepared solution S. |
|  | CHLORIDES | To 7 mL of solution S add 2 mL of nitric acid R and dilute to 15 mL with water R. Subject the solution to the limit test for chlorides.To 15 mL of the prescribed solution add 1 mL of dilute nitric acid R and pour the mixture as a single addition into a test-tube containing 1 mL of silver nitrate solution R2. Prepare a standard in the same manner using 10 mL of chloride standard solution (5 ppm Cl) R and 5 mL of water R. Examine the tubes laterally against a black background. After standing for 5 min protected from light, any opalescence in the test solution is not more intense than that in the standard. |
|  | SULHATES | To a suspension of 1.0 g in 10 mL of distilled water R add hydrochloric acid R until neutral and about 1 mL in excess. Dilute to 15 mL with distilled water R. Subject the solution to the limit test for sulphates (150 ppm). All solutions used for this test must be prepared with distilled water R. Add 3 mL of a 250 g/L solution of barium chloride R to 4.5 mL of sulfate standard solution (10 ppm SO4) R1. Shake and allow to stand for 1 min. To 2.5 mL of this suspension add 15 mL of the prescribed solution and 0.5 mL of acetic acid R. Prepare a standard in the same manner using 15 mL of sulfate standard solution (10 ppm SO4) R instead of the prescribed solution. After 5 min, any opalescence in the test solution is not more intense than that in the standard. |
|  | AMMONIUM | 10 mL of solution S is diluted to 15 mL with water R complies with the limit test for ammonium (20 ppm). Prepare the standard using a mixture of 5 mL of water R and 10 mL of ammonium standard solution (1 ppm NH4) R. Introduce the prescribed solution into a test-tube or dissolve the prescribed quantity of the substance to be examined in 14 mL of water R in a test-tube. Make the solution alkaline if necessary by the addition of dilute sodium hydroxide solution R, dilute to 15 mL with water R and add 0.3 mL of alkaline potassium tetraiodomercurate solution R. Prepare a standard by mixing 10 mL of ammonium standard solution (1 ppm NH4) R, 5 mL of water R and 0.3 mL of alkaline potassium tetraiodomercurate solution R. Stopper the test-tubes. After 5 min, any yellow colour in the test solution is not more intense than that in the standard. |
|  | CALCIUM | To a suspension of 1.0 g in 10 mL of distilled water R, add hydrochloric acid R until neutral and dilute to 15 mL with distilled water R. The solution complies with the limit test for calcium (100 ppm). All solutions used for this test are prepared with distilled water R. To 0.2 mL of alcoholic calcium standard solution (100 ppm Ca) R add 1 mL of ammonium oxalate solution R. After 1 min add a mixture of 1 mL of dilute acetic acid R and 15 mL of the prescribed solution or of a solution containing the prescribed quantity of the substance to be examined, and shake. Prepare a standard in the same manner using a mixture of 10 mL of aqueous calcium standard solution (10 ppm Ca) R, 1 mL of dilute acetic acid R and 5 mL of distilled water R. After 15 min, any opalescence in the test solution is not more intense than that in the standard. |
|  | IRON | Dissolve 0.5 g in 5 mL of dilute hydrochloric acid R and dilute to 10 mL with water R. Subject the solution to the limit test for Iron (20 ppm). Dissolve the prescribed quantity of the substance to be examined in water R and dilute to 10 mL with the same solvent or use 10 mL of the prescribed solution. Add 2 mL of a 200 g/L solution of citric acid monohydrate R and 0.1 mL of thioglycollic acid R. Mix, make alkaline with ammonia R and dilute to 20 mL with water R. Prepare a standard in the same manner, using 10 mL of iron standard solution (1 ppm Fe) R. After 5 min, any pink colour in the test solution is not more intense than that in the standard. |
|  | ASSAY | Dissolve 0.750 g in 50 mL of carbon dioxide – free water R. Titrate with 1 M hydrochloric acid, determining the end-point potentiometrically. Read the volume added at the 2nd point of inflection, or at the point of inflection if only 1 point is detected. 1 mL of 1 M hydrochloric acid is equivalent to 84.0 mg of NaHCO3. |
|  | MICROBIAL ENUMERATION TEST | Determination of total aerobic microbial count (TAMC) and total combined yeasts and molds count (TYMC) by the pour plate method. - Slightly open the lid of the petri plate and dispense 1 ml of diluted sample in the center of the Petri plate using a sterile calibrated pipette. - Open the lid of the prepared media. (soya bean digest agar for TAMC and Sabouraud Dextrose Agar for TYMC) - Pour about 15-20ml of the sterilized molten media at the appropriate temperature above the sample. - Close the lid of the plate then mix the sample in the media properly by gently swirling the plate. The plate is generally swirled in an “S” or “8” shape. - Allow the media to completely solidify. - Simultaneously perform the negative control by plating 1ml of diluent used instead of the sample for each medium. - Incubate the plate in an inverted position under suitable incubation conditions (For TAMC Incubate the plates of Soybean Casein Digest Agar at 30°C-35°C for 48hours and For TYMC incubate the plates of Sabouraud Dextrose Agar at 20°C - 25°C for 72hours Test for Absence of Pathogen Test for E. coli: Using same dilution - Transfer 1ml of the sample into a sterile McCartney bottle containing 9ml of sterile MacConkey broth. - Incubate the MacConkey broth bottle at 300C-350C for 24 hours. - After incubation shake the tube and transfer loop full portion of MacConkey broth on the surface of the pre incubated Eosin Methylene Blue Agar petri plate. - Incubate plates at 30°C to 35°C for 48 hours. - For negative control, proceeding same as above except without sample, streak a portion of incubated sterile MacConkey broth medium on the surface of pre incubated Eosin Methylene Blue Agar. - Observe and record the presence or absence of specified microorganisms. - Include positive and negative controls in each batch of tests. - Verify the performance of media and reagents used in the analysis. Interpretation of Results - The bacterial count should not exceed 102cfu/ml while that of the fungal count should not be more than 102cfu/ml. There should be absence of fecal coliform (E coli). - If growth occurs in the negative control plate, the test is invalid, failed negative control must be investigated - The total aerobic microbial count (TAMC) is considered to be equal to the number of CFU found using casein soya bean digest agar; if colonies of fungi are detected on this medium, they are counted as part of TAMC. - The total combined yeasts/mould count (TYMC) is considered to be equal to the number of CFU found using Sabouraud dextrose agar; if colonies of bacteria are detected on this medium, they are counted as part of TYMC - When an acceptance criterion for microbiological quality is prescribed it is interpreted as follows: 101 CFU : maximum acceptable count＝20 102 CFU: maximum acceptable count＝200, 103 CFU: maximum acceptable count＝2000. |

| - CURRENT REVISION | - SUPERSEDE | - REASON FOR REVIEW |
| --- | --- | --- |
| - 00 | - NIL | - FIRST ISSUE |
| - 01 | - 00 | - Review of the procedures - Change in header and footer format |