| S/N | TEST | DESCRIPTIONS OF PROCEDURE |
| --- | --- | --- |
|  | DESCRIPTION | Determine the colour and appearance of material |
|  | SOLUBILITY | Determine its solubility in water and in ethanol (96 per cent). |
|  | IDENTIFICATION | A. To 1 mL of solution S add 4 mL of water R. The solution gives the reaction of citrates. To a neutral solution of the substance being examined add a solution of calcium chloride; no precipitate is produced. Boil the solution; a white precipitate is produced which is soluble in 6M acetic acid. B. 1 mL of solution S gives reaction (a) of sodium. Dissolve 0.1 g of the substance to be examined in 2 mL of water R or use 2 mL of the prescribed solution. Add 2 mL of a 150 g/L solution of potassium carbonate R and heat to boiling. No precipitate is formed. Add 4 mL of potassium pyroantimonate solution R and heat to boiling. Allow to cool in iced water and if necessary rub the inside of the test-tube with a glass rod. A dense white precipitate is formed. |
|  | APPEARANCE OF SOLUTION | ### Solution S Dissolve 10.0 g in carbon dioxide-free water R prepared from distilled water R and dilute to 100 mL with the same solvent. |
|  | ACIDITY/ALKALINITY | To 10 mL of solution S add 0.1 mL of phenolphthalein solution R. Titrate the solution with 0.1 M sodium hydroxide or 0.1M hydrochloric acid until the colour changes. |
|  | READILY CARBONISABLE SUBSTANCES | To 0.20 g of the powdered substance to be examined add 10 mL of sulfuric acid R and heat in a water-bath at 90 ± 1 °C for 60 min. Cool rapidly. |
|  | CHLORIDE | Dilute 10 mL of solution S to 15 mL with water R. To 15 mL of the prescribed solution add 1 mL of dilute nitric acid R and pour the mixture as a single addition into a test-tube containing 1 mL of silver nitrate solution R2. Prepare a standard in the same manner using 10 mL of chloride standard solution (5 ppm Cl) R and 5 mL of water R. Examine the tubes laterally against a black background. After standing for 5 min protected from light, any opalescence in the test solution is not more intense than that in the standard. |
|  | OXALATES | Dissolve 0.50 g in 4 mL of water R, add 3 mL of hydrochloric acid R and 1 g of zinc R in granules and heat on a water-bath for 1 min. Allow to stand for 2 min, decant the liquid into a test-tube containing 0.25 mL of a 10 g/L solution of phenylhydrazine hydrochloride R and heat to boiling. Cool rapidly, transfer to a graduated cylinder and add an equal volume of hydrochloric acid R and 0.25 mL of potassium ferricyanide solution R. Shake and allow to stand for 30 min. Any pink colour in the solution is not more intense than that in a standard prepared at the same time in the same manner using 4 mL of a 50 mg/L solution of oxalic acid R. |
|  | SULFATES | To 10 mL of solution S add 2 mL of hydrochloric acid R1 and dilute to 15 mL with distilled water R. All solutions used for this test must be prepared with distilled water R. Add 3 mL of a 250 g/L solution of barium chloride R to 4.5 mL of sulfate standard solution (10 ppm SO4) R1. Shake and allow to stand for 1 min. To 2.5 mL of this suspension add 15 mL of the prescribed solution and 0.5 mL of acetic acid R. Prepare a standard in the same manner using 15 mL of sulfate standard solution (10 ppm SO4) R instead of the prescribed solution. After 5 min, any opalescence in the test solution is not more intense than that in the standard. |
|  | WATER | Determined on 0.300 g. Use as the solvent a mixture of 20 mL of methanol R, 30 mL of formamide R and 5 g of salicylic acid R |
|  | ASSAY | Dissolve 0.150 g in 20 mL of anhydrous acetic acid R, heating to about 50 °C. Allow to cool. Titrate with 0.1 M perchloric acid, using 0.25 mL of naphtholbenzein solution R as indicator until a green colour is obtained. 1 mL of 0.1 M perchloric acid is equivalent to 8.602 mg of C6H5Na3O7. |
|  | MICROBIAL ENUMERATION TEST | Determination of total aerobic microbial count (TAMC) and total combined yeasts and molds count (TYMC) by the pour plate method. - Slightly open the lid of the petri plate and dispense 1 ml of diluted sample in the center of the Petri plate using a sterile calibrated pipette. - Open the lid of the prepared media. (soya bean digest agar for TAMC and Sabouraud Dextrose Agar for TYMC) - Pour about 15-20ml of the sterilized molten media at the appropriate temperature above the sample. - Close the lid of the plate then mix the sample in the media properly by gently swirling the plate. The plate is generally swirled in an “S” or “8” shape. - Allow the media to completely solidify. - Simultaneously perform the negative control by plating 1ml of diluent used instead of the sample for each medium. - Incubate the plate in an inverted position under suitable incubation conditions (For TAMC Incubate the plates of Soybean Casein Digest Agar at 30°C-35°C for 48hours and For TYMC incubate the plates of Sabouraud Dextrose Agar at 20°C - 25°C for 72hours Test for Absence of Pathogen Test for E. coli: Using same dilution - Transfer 1ml of the sample into a sterile McCartney bottle containing 9ml of sterile MacConkey broth. - Incubate the MacConkey broth bottle at 300C-350C for 24 hours. - After incubation shake the tube and transfer loop full portion of MacConkey broth on the surface of the pre incubated Eosin Methylene Blue Agar petri plate. - Incubate plates at 30°C to 35°C for 48 hours. - For negative control, proceeding same as above except without sample, streak a portion of incubated sterile MacConkey broth medium on the surface of pre incubated Eosin Methylene Blue Agar. - Observe and record the presence or absence of specified microorganisms. - Include positive and negative controls in each batch of tests. - Verify the performance of media and reagents used in the analysis. Interpretation of Results - The bacterial count should not exceed 102cfu/ml while that of the fungal count should not be more than 102cfu/ml. There should be absence of fecal coliform (E coli). - If growth occurs in the negative control plate, the test is invalid, failed negative control must be investigated - The total aerobic microbial count (TAMC) is considered to be equal to the number of CFU found using casein soya bean digest agar; if colonies of fungi are detected on this medium, they are counted as part of TAMC. - The total combined yeasts/mould count (TYMC) is considered to be equal to the number of CFU found using Sabouraud dextrose agar; if colonies of bacteria are detected on this medium, they are counted as part of TYMC - When an acceptance criterion for microbiological quality is prescribed it is interpreted as follows: 101 CFU : maximum acceptable count＝20 102 CFU: maximum acceptable count＝200, 103 CFU: maximum acceptable count＝2000. |

| - CURRENT REVISION | - SUPERSEDE | - REASON FOR REVIEW |
| --- | --- | --- |
| - 00 | - NIL | - FIRST ISSUE |
| - 01 | - 00 | - Review of the procedures - Change in header and footer format |