| S/N | TEST | DESCRIPTIONS OF PROCEDURE |
| --- | --- | --- |
|  | DESCRIPTION | Describe the colour and other characteristics of the material |
|  | SOLUBILITY | Determine solubility of material in water, ethanol (96%) and methylene chloride. |
|  | IDENTIFICATION | First identification: B, D. Second identification: A, C, D. A.Dissolve 0.5 g in 50 mL of water R. Immediately add 5 mL of hydrochloric acid R1. Filter and wash the precipitate with water R. Dry in vacuo at 80 °C for 2 h. B.Infrared absorption spectrophotometry . Preparation Precipitate obtained in identification test A. Comparison methyl parahydroxybenzoate CRS. C.Thin-layer chromatography Test solution (a) Dissolve 0.10 g of the substance to be examined in 10 mL of water R. Immediately add 2 mL of hydrochloric acid R and shake with 50 mL of 1,1-dimethylethyl methyl ether R. Evaporate the upper layer to dryness and take up the residue with 10 mL of acetone R. Test solution (b) Dilute 1 mL of test solution (a) to 10 mL with acetone R. Reference solution (a) Dissolve 10 mg of methyl parahydroxybenzoate CRS in acetone R and dilute to 10 mL with the same solvent. Reference solution (b) Dissolve 10 mg of ethyl parahydroxybenzoate CRS in 1 mL of test solution (a) and dilute to 10 mL with acetone R. PlateTLC octadecylsilyl silica gel F254 plate R. Mobile phaseglacial acetic acid R, water R, methanol R (1:30:70 V/V/V). Application 5 µL of test solution (b) and reference solutions (a) and (b). Development Over 2/3 of the plate. Drying In air. Detection Examine in ultraviolet light at 254 nm. System suitability Reference solution (b): — the chromatogram shows 2 clearly separated principal spots. D.To 1 mL of solution S add 1 mL of water R. The solution gives reaction (a) of sodium. |
|  | APPEARANCE OF SOLUTION | Solution S Dissolve 5.0 g in carbon dioxide-free water R prepared from distilled water R and dilute to 50 mL with the same solvent. Solution S examined immediately after preparation |
|  | pH | Dilute 1 mL of solution S to 100 mL with carbon dioxide-free water R. |
|  | RELATED SUBSTANCES | Liquid chromatography. Test solution Dissolve 50.0 mg of the substance to be examined in 2.5 mL of methanol R and dilute to 50.0 mL with the mobile phase. Dilute 10.0 mL of the solution to 100.0 mL with the mobile phase. Reference solution (a) Dissolve 5 mg of 4-hydroxybenzoic acid R (impurity A) and 5 mg of the substance to be examined in the mobile phase and dilute to 100.0 mL with the mobile phase. Dilute 1.0 mL of the solution to 10.0 mL with the mobile phase. Reference solution (b) Dissolve 50.0 mg of methyl parahydroxybenzoate CRS in 2.5 mL of methanol R and dilute to 50.0 mL with the mobile phase. Dilute 10.0 mL of the solution to 100.0 mL with the mobile phase. Reference solution (c) Dilute 1.0 mL of the test solution to 20.0 mL with the mobile phase. Dilute 1.0 mL of this solution to 10.0 mL with the mobile phase. Column: — size: l = 0.15 m, Ø = 4.6 mm; — stationary phase: end-capped octadecylsilyl silica gel for chromatography R (5 µm). Mobile phase 6.8 g/L solution of potassium dihydrogen phosphate R, methanol R (35:65 V/V). Flow rate 1.3 mL/min. Detection Spectrophotometer at 272 nm. Injection 10 µL of the test solution and reference solutions (a) and (c). Run time 5 times the retention time of methyl parahydroxybenzoate. Relative retention With reference to methyl parahydroxybenzoate (retention time = about 2.3 min): impurity A = about 0.6. System suitability Reference solution (a): — resolution: minimum 2.0 between the peaks due to impurity A and methyl parahydroxybenzoate. |
|  | CHLORIDE | Maximum 350 ppm. To 10 mL of solution S, add 30 mL of water R and 1 mL of nitric acid R and dilute to 50 mL with water R. Shake and filter. Dilute 10 mL of the filtrate to 15 mL with water R. Prepare the standard using 14 mL of chloride standard solution (5 ppm Cl) R to which 1 mL of water R has been added. To 15 mL of the prescribed solution add 1 mL of dilute nitric acid R and pour the mixture as a single addition into a test-tube containing 1 mL of silver nitrate solution R2. Prepare a standard in the same manner using 10 mL of chloride standard solution (5 ppm Cl) R and 5 mL of water R. Examine the tubes laterally against a black background. After standing for 5 min protected from light, any opalescence in the test solution is not more intense than that in the standard. |
|  | SULPHATES | To 25 mL of solution S, add 5 mL of distilled water R and 10 mL of hydrochloric acid R and dilute to 50 mL with distilled water R. Shake and filter. Dilute 10 mL of the filtrate to 15 mL with distilled water R. All solutions used for this test must be prepared with distilled water R. Add 3 mL of a 250 g/L solution of barium chloride R to 4.5 mL of sulfate standard solution (10 ppm SO4) R1. Shake and allow to stand for 1 min. To 2.5 mL of this suspension add 15 mL of the prescribed solution and 0.5 mL of acetic acid R. Prepare a standard in the same manner using 15 mL of sulfate standard solution (10 ppm SO4) R instead of the prescribed solution. After 5 min, any opalescence in the test solution is not more intense than that in the standard. |
|  | WATER | Determined on 0.500 g. |
|  | ASSAY | Liquid chromatography as described in the test for related substances with the following modification. Injection Test solution and reference solution (b). Calculate the percentage content of C8H7NaO3 using the declared content of methyl parahydroxybenzoate CRS and multiplying by a correction factor of 1.145. |
|  | MICROBIAL ENUMERATION TEST | Determination of total aerobic microbial count (TAMC) and total combined yeasts and molds count (TYMC) by the pour plate method. - Slightly open the lid of the petri plate and dispense 1 ml of diluted sample in the center of the Petri plate using a sterile calibrated pipette. - Open the lid of the prepared media. (soya bean digest agar for TAMC and Sabouraud Dextrose Agar for TYMC) - Pour about 15-20ml of the sterilized molten media at the appropriate temperature above the sample. - Close the lid of the plate then mix the sample in the media properly by gently swirling the plate. The plate is generally swirled in an “S” or “8” shape. - Allow the media to completely solidify. - Simultaneously perform the negative control by plating 1ml of diluent used instead of the sample for each medium. - Incubate the plate in an inverted position under suitable incubation conditions (For TAMC Incubate the plates of Soybean Casein Digest Agar at 30°C-35°C for 48hours and For TYMC incubate the plates of Sabouraud Dextrose Agar at 20°C - 25°C for 72hours Test for Absence of Pathogen Test for E. coli: Using same dilution - Transfer 1ml of the sample into a sterile McCartney bottle containing 9ml of sterile MacConkey broth. - Incubate the MacConkey broth bottle at 300C-350C for 24 hours. - After incubation shake the tube and transfer loop full portion of MacConkey broth on the surface of the pre incubated Eosin Methylene Blue Agar petri plate. - Incubate plates at 30°C to 35°C for 48 hours. - For negative control, proceeding same as above except without sample, streak a portion of incubated sterile MacConkey broth medium on the surface of pre incubated Eosin Methylene Blue Agar. - Observe and record the presence or absence of specified microorganisms. - Include positive and negative controls in each batch of tests. - Verify the performance of media and reagents used in the analysis. Interpretation of Results - The bacterial count should not exceed 102cfu/ml while that of the fungal count should not be more than 102cfu/ml. There should be absence of fecal coliform (E coli). - If growth occurs in the negative control plate, the test is invalid, failed negative control must be investigated - The total aerobic microbial count (TAMC) is considered to be equal to the number of CFU found using casein soya bean digest agar; if colonies of fungi are detected on this medium, they are counted as part of TAMC. - The total combined yeasts/mould count (TYMC) is considered to be equal to the number of CFU found using Sabouraud dextrose agar; if colonies of bacteria are detected on this medium, they are counted as part of TYMC - When an acceptance criterion for microbiological quality is prescribed it is interpreted as follows: 101 CFU : maximum acceptable count＝20 102 CFU: maximum acceptable count＝200, 103 CFU: maximum acceptable count＝2000. |

| - CURRENT REVISION | - SUPERSEDE | - REASON FOR REVIEW |
| --- | --- | --- |
| - 00 | - NIL | - FIRST ISSUE |
| - 01 | - 00 | - Review of the procedures - Change in header and footer format |