| S/N | TEST | DESCRIPTIONS OF PROCEDURE |
| --- | --- | --- |
|  | DESCRIPTION | Describe the colour and appearance of the material. |
|  | SOLUBILITY | Determine its miscibility with water. |
|  | IDENTIFICATION | Determine its miscibility with water. A. Examine the chromatograms obtained in the assay. Results The principal peak in the chromatogram obtained with the test solution is similar in retention time to the principal peak in the chromatogram obtained with reference solution (a). B. To 7.0 g add 40 ml of water R and 6.4 g of disodium tetraborate R. Allow to stand for 1 hour, shaking occasionally and dilute to 50.0 ml with water R. Filter if necessary. Determine the angle of rotation. C. Comment on the clarity and nature of the material at 250C. |
|  | APPEARANCE OF SOLUTION | Dilute 7.0 g to 50 ml with water R. Observe and record the clarity and colour. |
|  | CONDUCTIVITY | Measure the conductivity of the undiluted liquid while gently stirring with a magnetic stirrer. |
|  | REDUCING SUGAR | To 5.0 g add 6 ml of water R, 20 ml of cupri-citric solution R and a few glass beads. Heat so that boiling begins after 4 min and maintain boiling for 3 min. Cool rapidly and add 100 ml of a 2.4 % V/V solution of glacial acetic acid R and 20.0 ml of 0.025 M iodine. With continuous shaking, add 25 ml of a mixture of 6 volumes of hydrochloric acid R and 94 volumes of water R and when the precipitate has dissolved, titrate the excess of iodine with 0.05 M sodium thiosulphate using 1 ml of starch solution R, added towards the end of the titration, as indicator. Record the volume of 0.05 M sodium thiosulphate used. |
|  | REDUCING SUGARS AFTER HYDROLYSIS | To 6.0 g add 35 ml of water R, 40 ml of 1M hydrochloric acid and a few glass beads. Boil under a reflux condenser for 4hr. Cool and neutralize with dilute sodium hydroxide solution R using 0.2 ml of bromothymol blue solution R1 as indicator. Cool and dilute to 100.0 ml with water R. To 3.0 ml of the solution add 5 ml of water, 20 ml of cupri-citric solution R and a few glass beads. Heat so that boiling begins after 4 mins and maintain boiling for 3 min. Cool rapidly and add 100 ml of a 2.4% V/V solution of glacial acetic acid R and 20.0 ml of 0.025 M iodine. With continuous shaking, add 25 ml of a mixture 6 volumes of hydrochloric acid R and 94 volumes of water R. When the precipitate has dissolved, titrate the excess of iodine with 0.05 M sodium thiosulphate using 1 ml of starch solution R, added towards the end of the titration, as indicator. Record the volume of 0.05 M sodium thiosulphate required. |
|  | WATER CONTENT | Determine the water content of the material by Karl Fischer apparatus. |
|  | ASSAY | Liquid chromatography. Test solution Mix 1.00 g of the substance to be examined with 20 ml of water R and dilute to 50.0 ml with the same solvent. Reference solution (a) Dissolve 55.0 mg of sorbitol CRS in 2 ml of water R and dilute to 5.0 ml with the same solvent. Reference solution (b) Dissolve 55 mg of mannitol R and 55 mg of sorbitol R in 2 ml of water R and dilute to 5.0 ml with the same solvent. Column: -- size: l = 0.3 m, Ø = 7.8 mm, -- stationary phase: strong cation exchange resin (calcium form) R (9µm), -- temperature: 85 + 10C. Mobile phase Degassed water R. Flow rate 0.5 ml/min Detection Refractometer maintained at a constant temperature Injection 20 µl Run time 3 times the retention time of sorbitol. Relative retention with reference to sorbitol (retention time = about 27 min): mannitol = about 0.8 System suitability Reference solution (b): -- resolution: minimum 2 between the peaks due to mannitol and sorbitol. Calculate the percentage content of D - sorbitol from the area of the peaks and the declared content of sorbitol CRS. |
|  | MICROBIAL ENUMERATION TEST | Determination of total aerobic microbial count (TAMC) and total combined yeasts and molds count (TYMC) by the pour plate method. - Slightly open the lid of the petri plate and dispense 1 ml of diluted sample in the center of the Petri plate using a sterile calibrated pipette. - Open the lid of the prepared media. (soya bean digest agar for TAMC and Sabouraud Dextrose Agar for TYMC) - Pour about 15-20ml of the sterilized molten media at the appropriate temperature above the sample. - Close the lid of the plate then mix the sample in the media properly by gently swirling the plate. The plate is generally swirled in an “S” or “8” shape. - Allow the media to completely solidify. - Simultaneously perform the negative control by plating 1ml of diluent used instead of the sample for each medium. - Incubate the plate in an inverted position under suitable incubation conditions (For TAMC Incubate the plates of Soybean Casein Digest Agar at 30°C-35°C for 48hours and For TYMC incubate the plates of Sabouraud Dextrose Agar at 20°C - 25°C for 72hours Test for Absence of Pathogen Test for E. coli: Using same dilution - Transfer 1ml of the sample into a sterile McCartney bottle containing 9ml of sterile MacConkey broth. - Incubate the MacConkey broth bottle at 300C-350C for 24 hours. - After incubation shake the tube and transfer loop full portion of MacConkey broth on the surface of the pre incubated Eosin Methylene Blue Agar petri plate. - Incubate plates at 30°C to 35°C for 48 hours. - For negative control, proceeding same as above except without sample, streak a portion of incubated sterile MacConkey broth medium on the surface of pre incubated Eosin Methylene Blue Agar. - Observe and record the presence or absence of specified microorganisms. - Include positive and negative controls in each batch of tests. - Verify the performance of media and reagents used in the analysis. Interpretation of Results - The bacterial count should not exceed 102cfu/ml while that of the fungal count should not be more than 102cfu/ml. There should be absence of fecal coliform (E coli). - If growth occurs in the negative control plate, the test is invalid, failed negative control must be investigated - The total aerobic microbial count (TAMC) is considered to be equal to the number of CFU found using casein soya bean digest agar; if colonies of fungi are detected on this medium, they are counted as part of TAMC. - The total combined yeasts/mould count (TYMC) is considered to be equal to the number of CFU found using Sabouraud dextrose agar; if colonies of bacteria are detected on this medium, they are counted as part of TYMC - When an acceptance criterion for microbiological quality is prescribed it is interpreted as follows: 101 CFU : maximum acceptable count＝20 102 CFU: maximum acceptable count＝200, 103 CFU: maximum acceptable count＝2000. |

| - CURRENT REVISION | - SUPERSEDE | - REASON FOR REVIEW |
| --- | --- | --- |
| - 00 | - NIL | - FIRST ISSUE |
| - 01 | - 00 | - Review of the procedures - Change in header and footer format |