| S/N | TEST | DESCRIPTIONS OF PROCEDURE |
| --- | --- | --- |
|  | DESCRIPTION | Determine the colour and appearance of material |
|  | SOLUBILITY | Determine its solubility in water and in ethanol (96 per cent). |
|  | IDENTIFICATION | First identification: A Second identification: B, C A. Infrared absorption spectrophotometry B. Thin-layer chromatography Test solution Dissolve 10 mg of the substance to be examined in a mixture of 2 volumes of water R and 3 volumes of methanol R and dilute to 20 mL with the same mixture of solvents. Reference solution (a) Dissolve 10 mg of sucrose CRS in a mixture of 2 volumes of water R and 3 volumes of methanol R and dilute to 20 mL with the same mixture of solvents. Reference solution (b) Dissolve 10 mg of fructose R, 10 mg of glucose R, 10 mg of lactose monohydrate R and 10 mg of sucrose R in a mixture of 2 volumes of water R and 3 volumes of methanol R and dilute to 20 mL with the same mixture of solvents. PlateTLC silica gel G plate R. Mobile phasecold saturated boric acid solution R, 60 per cent V/V solution of glacial acetic acid R, anhydrous ethanol R, acetone R, ethyl acetate R (10:15:20:60:60 V/V/V/V/V). Application 2 µL. Development In an unsaturated tank, over 3/4 of the plate. Drying In a current of warm air. Detection Spray with a solution of 0.5 g of thymol R in a mixture of 5 mL of sulfuric acid R and 95 mL of ethanol (96 per cent) R. Heat the plate at 130 °C for 10 min. System suitability The chromatogram obtained with reference solution (b) shows 4 clearly separated spots. C. Dilute 1 mL of solution S to 100 mL with water R. To 5 mL of the solution add 0.15 mL of freshly prepared copper sulfate solution R and 2 mL of freshly prepared dilute sodium hydroxide solution R. The solution is blue and clear and remains so after boiling. To the hot solution add 4 mL of dilute hydrochloric acid R and boil for 1 min. Add 4 mL of dilute sodium hydroxide solution R. |
|  | APPEARANCE OF SOULTION | ### Solution S Dissolve 50.0 g in water R and dilute to 100 mL with the same solvent. |
|  | CONDUCTIVITY | Dissolve 31.3 g in carbon dioxide-free water R prepared from distilled water R and dilute to 100 mL with the same solvent. Measure the conductivity of the solution (C1), while gently stirring with a magnetic stirrer, and that of the water used for preparing the solution (C2). The readings must be stable within 1 per cent over a period of 30 s. Calculate the conductivity of the solution of the substance to be examined using the following expression: |
|  | SPECIFIC OPTICAL ROTATION | Dissolve 26.0 g in water R and dilute to 100.0 mL with the same solvent. |
|  | COLOR VALUE | Dissolve 50.0 g in 50.0 mL of water R. Mix, filter (diameter of pores 0.45 µm) and degas. Measure the absorbance at 420 nm, using a minimum path length of 4 cm (a path length of 10 cm or more is preferred). Calculate the colour value using the following expression: A; =; absorbance measured at 420 nm;. b; =; path length in centimetres;. c; =; concentration of the solution, in grams per millilitre, calculated from the refractive index of the solution; use Table 0204.-1 and interpolate the values if necessary.. System suitability: — repeatability: the absolute difference between 2 results is not greater than 3 |
|  | DEXTRIN | To 2 mL of solution S add 8 mL of water R, 0.05 mL of dilute hydrochloric acid R and 0.05 mL of 0.05 M iodine. |
|  | REDUCING SUGAR | To 5 mL of solution S in a test-tube about 150 mm long and 16 mm in diameter add 5 mL of water R, 1.0 mL of 1 M sodium hydroxide and 1.0 mL of a 1 g/L solution of methylene blue R. Mix and place in a water-bath. After exactly 2 min, take the tube out of the bath and examine the solution immediately. |
|  | SULFITES | Determine the sulfites content by a suitable enzymatic method based on the following reactions. Sulfite is oxidised by sulfite oxidase to sulfate and hydrogen peroxide which in turn is reduced by nicotinamide-adenine dinucleotide-peroxidase in the presence of reduced nicotinamide-adenine dinucleotide (NADH). The amount of NADH oxidised is proportional to the amount of sulfite. |
|  | REALTED SUBSTANCES | Test solution Dissolve 4.0 g of the substance to be examined in freshly prepared distilled water R and dilute to 10.0 mL with the same solvent. Reference solution Dissolve 4.0 g of the substance to be examined in freshly prepared distilled water R, add 0.5 mL of sulfite standard solution (80 ppm SO2) R and dilute to 10.0 mL with freshly prepared distilled water R. Blank solution Freshly prepared distilled water R. Separately introduce 2.0 mL each of the test solution, the reference solution and the blank in 10 mm cuvettes and add the reagents as described in the instructions in the kit for sulfite determination. Measure the absorbance at the absorption maximum at about 340 nm before and at the end of the reaction time and subtract the value obtained with the blank. The absorbance difference of the test solution is not greater than half the absorbance difference of the reference solution. |
|  | LOSS ON DRYING | Determined the Loss on drying on 2.000 g by drying in an oven at 105 °C for 3 h. |
|  | MICROBIAL ENUMERATION TEST | Determination of total aerobic microbial count (TAMC) and total combined yeasts and molds count (TYMC) by the pour plate method. - Slightly open the lid of the petri plate and dispense 1 ml of diluted sample in the center of the Petri plate using a sterile calibrated pipette. - Open the lid of the prepared media. (soya bean digest agar for TAMC and Sabouraud Dextrose Agar for TYMC) - Pour about 15-20ml of the sterilized molten media at the appropriate temperature above the sample. - Close the lid of the plate then mix the sample in the media properly by gently swirling the plate. The plate is generally swirled in an “S” or “8” shape. - Allow the media to completely solidify. - Simultaneously perform the negative control by plating 1ml of diluent used instead of the sample for each medium. - Incubate the plate in an inverted position under suitable incubation conditions (For TAMC Incubate the plates of Soybean Casein Digest Agar at 30°C-35°C for 48hours and For TYMC incubate the plates of Sabouraud Dextrose Agar at 20°C - 25°C for 72hours Test for Absence of Pathogen Test for E. coli: Using same dilution - Transfer 1ml of the sample into a sterile McCartney bottle containing 9ml of sterile MacConkey broth. - Incubate the MacConkey broth bottle at 300C-350C for 24 hours. - After incubation shake the tube and transfer loop full portion of MacConkey broth on the surface of the pre incubated Eosin Methylene Blue Agar petri plate. - Incubate plates at 30°C to 35°C for 48 hours. - For negative control, proceeding same as above except without sample, streak a portion of incubated sterile MacConkey broth medium on the surface of pre incubated Eosin Methylene Blue Agar. - Observe and record the presence or absence of specified microorganisms. - Include positive and negative controls in each batch of tests. - Verify the performance of media and reagents used in the analysis. Interpretation of Results - The bacterial count should not exceed 102cfu/ml while that of the fungal count should not be more than 102cfu/ml. There should be absence of fecal coliform (E coli). - If growth occurs in the negative control plate, the test is invalid, failed negative control must be investigated - The total aerobic microbial count (TAMC) is considered to be equal to the number of CFU found using casein soya bean digest agar; if colonies of fungi are detected on this medium, they are counted as part of TAMC. - The total combined yeasts/mould count (TYMC) is considered to be equal to the number of CFU found using Sabouraud dextrose agar; if colonies of bacteria are detected on this medium, they are counted as part of TYMC - When an acceptance criterion for microbiological quality is prescribed it is interpreted as follows: 101 CFU : maximum acceptable count＝20 102 CFU: maximum acceptable count＝200, 103 CFU: maximum acceptable count＝2000. |

| - CURRENT REVISION | - SUPERSEDE | - REASON FOR REVIEW |
| --- | --- | --- |
| - 00 | - NIL | - FIRST ISSUE |
| - 01 | - 00 | - Review of the procedures - Change in header and footer format |