MATERIAL NAME: SUCROSE

SOP Ref. No.: BML-QDG-QA-105                		       		 EFFECTIVE DATE: JAN., 2024

| S/No | TESTS REQUIRED | SPECIFICATIONS | SPECIFICATION REFERENCES |
| --- | --- | --- | --- |
|  | Description | White or almost white, crystalline powder, or lustrous, colourless or white or almost white crystals.. | BP 2023 |
|  | Solubility | Very soluble in water, slightly soluble in ethanol (96 per cent), practically insoluble in anhydrous ethanol. | BP 2023 |
|  | Identification | A.Infrared absorption spectrophotometry . Comparison sucrose CRS. B.Thin-layer chromatography. The principal spot in the chromatogram obtained with the test solution is similar in position, colour and size to the principal spot in the chromatogram obtained with reference solution (a). C. An orange precipitate is formed immediately. | BP 2023 |
|  | Appearance of Solution | Solution S is clear | BP 2023 |
|  | Conductivity | Maximum 35 µS⋅cm-1 at 20 °C. | BP 2023 |
|  | Specific Optical Rotation | + 66.3 to + 67.0. | BP 2023 |
|  | Color Value | Maximum 45. | BP 2023 |
|  | Dextrin | The solution remains yellow. | BP 2023 |
|  | Reducing Sugar | The blue colour does not disappear completely. Ignore any blue colour at the air/solution interface. | BP 2023 |
|  | Sulfites | Maximum 10 ppm, calculated as SO2. | BP 2023 |
|  | Loss on Drying | Maximum 0.1 per cent. | BP 2023 |
|  | Microbial Contamination; TMC Total Yeast and Mould count Pathogens | NMT 1000cfu/g NMT 100cfu/g Must be absent | BP 2023 |

TMC= Total Microbial Count, NMT= Not More Than, cfu/g= Colony Forming Unit per Gram