| S/N | TEST | DESCRIPTIONS OF PROCEDURE |
| --- | --- | --- |
|  | DESCRIPTION | Determine the colour and appearance of material |
|  | SOLUBILITY | Determine its solubility in water, acetone, ethanol (96 per cent) and dilute solutions of alkali hydroxides and in dilute mineral acids. |
|  | IDENTIFICATION | First identification: A, B. Second identification: A, C, D. - Determine the Melting Point - Examine by infrared absorption spectrophotometry , comparing with the spectrum obtained with sulfamethizole CRS. Examine the substances prepared as discs. C.Thin-layer chromatography. Test solution Dissolve 20 mg of the substance to be examined in 3 mL of a mixture of 2 volumes of concentrated ammonia R and 48 volumes of methanol R and dilute to 5 mL with the same mixture of solvents. Reference solution Dissolve 20 mg of sulfamethoxazole CRS in 3 mL of a mixture of 2 volumes of concentrated ammonia R and 48 volumes of methanol R and dilute to 5 mL with the same mixture of solvents. Plate TLC silica gel F254 plate R. Mobile phase dilute ammonia R1, water R, nitromethane R, dioxan R (3:5:41:51 V/V/V/V). Application 5 µL. Development Over 3/4 of the plate. Drying At 100-105 °C. Detection Examine in ultraviolet light at 254 nm. D.Dissolve about 5 mg in 10 mL of 1 M hydrochloric acid. Dilute 1 mL of the solution to 10 mL with water R. The solution, without further acidification, gives the reaction of primary aromatic amines. Acidify the prescribed solution with dilute hydrochloric acid R and add 0.2 mL of sodium nitrite solution R. After 1 min to 2 min, add 1 mL of β-naphthol solution R. An intense orange or red colour and usually a precipitate of the same colour are produced. |
|  | APPEARANCE OF SOLUTION | Dissolve 1.0 g in a mixture of 5 mL of dilute sodium hydroxide solution R and 5 mL of water R. Observe the clarity of Solution S and a fivefold dilution of solution S. Compare it colour intensity with reference solution Y5, BY5 or GY5. |
|  | ACIDITYALKALINITY | To 1.25 g add 25 mL of carbon dioxide-free water R and heat at 70 °C for 5 min. Cool for about 15 min in iced water and filter. To 20 mL of the filtrate add 0.1 mL of bromothymol blue solution R1. |
|  | RELATED SUBSTANCES | Liquid chromatography. Solvent mixturemethanol R, water R (15:85 V/V). Test solution Dissolve 20.0 mg of the substance to be examined in 1 mL of methanol R, sonicate if necessary and dilute to 20.0 mL with the solvent mixture. Reference solution (a) Dilute 1.0 mL of the test solution to 100.0 mL with the solvent mixture. Dilute 1.0 mL of this solution to 10.0 mL with the solvent mixture. Reference solution (b) Dissolve 2 mg of sulfamethizole for system suitability CRS (containing impurities B, C and D) in 0.1 mL of methanol R, sonicate if necessary and dilute to 2 mL with the solvent mixture. Column: — size: l = 0.25 m, Ø = 4.6 mm; — stationary phase: end-capped octadecylsilyl silica gel for chromatography compatible with 100 per cent aqueous mobile phases R (5 µm); — temperature: 30 °C. Mobile phase: — mobile phase A: acetic acid R, methanol R, water for chromatography R (1:14:85 V/V/V); — mobile phase B: acetic acid R, methanol R (1:99 V/V); Time (min); Mobile phase A (per cent V/V); Mobile phase B (per cent V/V). 0 - 20; 100; 0. 20 - 40; 100 → 30; 0 → 70. 40 - 45; 30; 70. Flow rate 1.0 mL/min. Detection Spectrophotometer at 254 nm. Injection 20 µL. Identification of impurities Use the chromatogram supplied with sulfamethizole for system suitability CRS and the chromatogram obtained with reference solution (b) to identify the peaks due to impurities B, C and D. Relative retention With reference to sulfamethizole (retention time = about 17 min): impurity B = about 0.23; impurity C = about 0.24; impurity D = about 1.9. System suitability Reference solution (b): — resolution: minimum 1.5 between the peaks due to impurities B and C. Calculation of percentage contents: — correction factor: multiply the peak area of impurity D by 1.4; — for each impurity, use the concentration of sulfamethizole in reference solution (a). |
|  | LOSS ON DRYING | Determined the Loss on drying on 1.000 g by drying in an oven at 105 °C. |
|  | SULPHATED ASH | Determine the Sulphated Ash on 1.0g of sample @ 8000C. |
|  | ASSAY | Carry out the determination of primary aromatic amino-nitrogen , using 0.2500 g and determining the end-point electrometrically. Dissolve the prescribed quantity of the substance to be examined in 50 mL of dilute hydrochloric acid R or in another prescribed solvent and add 3 g of potassium bromide R. Cool in ice-water and titrate by slowly adding 0.1 M sodium nitrite with constant stirring.Determine the end-point electrometrically or by the use of the prescribed indicator. 1 mL of 0.1 M sodium nitrite is equivalent to 27.03 mg of C9H10N4O2S2. |
|  | MICROBIAL ENUMERATION TEST | Determination of total aerobic microbial count (TAMC) and total combined yeasts and molds count (TYMC) by the pour plate method. - Slightly open the lid of the petri plate and dispense 1 ml of diluted sample in the center of the Petri plate using a sterile calibrated pipette. - Open the lid of the prepared media. (soya bean digest agar for TAMC and Sabouraud Dextrose Agar for TYMC) - Pour about 15-20ml of the sterilized molten media at the appropriate temperature above the sample. - Close the lid of the plate then mix the sample in the media properly by gently swirling the plate. The plate is generally swirled in an “S” or “8” shape. - Allow the media to completely solidify. - Simultaneously perform the negative control by plating 1ml of diluent used instead of the sample for each medium. - Incubate the plate in an inverted position under suitable incubation conditions (For TAMC Incubate the plates of Soybean Casein Digest Agar at 30°C-35°C for 48hours and For TYMC incubate the plates of Sabouraud Dextrose Agar at 20°C - 25°C for 72hours Test for Absence of Pathogen Test for E. coli: Using same dilution - Transfer 1ml of the sample into a sterile McCartney bottle containing 9ml of sterile MacConkey broth. - Incubate the MacConkey broth bottle at 300C-350C for 24 hours. - After incubation shake the tube and transfer loop full portion of MacConkey broth on the surface of the pre incubated Eosin Methylene Blue Agar petri plate. - Incubate plates at 30°C to 35°C for 48 hours. - For negative control, proceeding same as above except without sample, streak a portion of incubated sterile MacConkey broth medium on the surface of pre incubated Eosin Methylene Blue Agar. - Observe and record the presence or absence of specified microorganisms. - Include positive and negative controls in each batch of tests. - Verify the performance of media and reagents used in the analysis. Interpretation of Results - The bacterial count should not exceed 102cfu/ml while that of the fungal count should not be more than 102cfu/ml. There should be absence of fecal coliform (E coli). - If growth occurs in the negative control plate, the test is invalid, failed negative control must be investigated - The total aerobic microbial count (TAMC) is considered to be equal to the number of CFU found using casein soya bean digest agar; if colonies of fungi are detected on this medium, they are counted as part of TAMC. - The total combined yeasts/mould count (TYMC) is considered to be equal to the number of CFU found using Sabouraud dextrose agar; if colonies of bacteria are detected on this medium, they are counted as part of TYMC - When an acceptance criterion for microbiological quality is prescribed it is interpreted as follows: 101 CFU : maximum acceptable count＝20 102 CFU: maximum acceptable count＝200, 103 CFU: maximum acceptable count＝2000. |

| - CURRENT REVISION | - SUPERSEDE | - REASON FOR REVIEW |
| --- | --- | --- |
| - 00 | - NIL | - FIRST ISSUE |
| - 01 | - 00 | - Review of the procedures - Change in header and footer format |