| S/N | TEST | DESCRIPTIONS OF PROCEDURE |
| --- | --- | --- |
|  | DESCRIPTION | Determine the colour and appearance of material |
|  | SOLUBILITY | Determine its solubility in water, in glycerol, in ethanol (96 per cent), and in heptane. |
|  | IDENTIFICATION | First identification: A, C. Second identification: B, C. Infrared absorption spectrophotometry . Comparison thiamine hydrochloride CRS. If the spectra obtained show differences, dissolve the substance to be examined and the reference substance separately in water R, evaporate to dryness and record new spectra using the residues. B. Dissolve about 20 mg in 10 mL of water R, add 1 mL of dilute acetic acid R and 1.6 mL of 1 M sodium hydroxide, heat on a water-bath for 30 min and allow to cool. Add 5 mL of dilute sodium hydroxide solution R, 10 mL of potassium ferricyanide solution R and 10 mL of butanol R and shake vigorously for 2 min. The upper alcoholic layer shows an intense light-blue fluorescence, especially in ultraviolet light at 365 nm. Repeat the test using 0.9 mL of 1 M sodium hydroxide and 0.1 g of anhydrous sodium sulfite R instead of 1.6 mL of 1 M sodium hydroxide. C. It gives reaction (a) of chlorides. Dissolve in 2 mL of water R a quantity of the substance to be examined equivalent to about 2 mg of chloride (Cl–) or use 2 mL of the prescribed solution. Acidify with dilute nitric acid R and add 0.4 mL of silver nitrate solution R1. Shake and allow to stand. A curdled, white precipitate is formed. Centrifuge and wash the precipitate with three quantities, each of 1 mL, of water R. Carry out this operation rapidly in subdued light, disregarding the fact that the supernatant solution may not become perfectly clear. Suspend the precipitate in 2 mL of water R and add 1.5 mL of ammonia R. The precipitate dissolves easily with the possible exception of a few large particles which dissolve slowly. |
|  | APPEARANCE OF SOULTION | ### Solution S--Dissolve 2.5 g in carbon dioxide-free water R and dilute to 25 mL with the same solvent. Dilute 2.5 mL of solution S to 5 mL with water R |
|  | pH | Dilute 2.5 mL of solution S to 10 mL with water R. |
|  | RELATED SUBSTANCES | Liquid chromatography. Test solution Dissolve 0.35 g of the substance to be examined in 15.0 mL of a 5 per cent V/V solution of glacial acetic acid R and dilute to 100.0 mL with water R. Reference solution (a) Dissolve the contents of a vial of thiamine for system suitability CRS (containing impurities A, B and C) in 1 mL of a 0.75 per cent V/V solution of glacial acetic acid R. Reference solution (b) Dilute 1.0 mL of the test solution to 100.0 mL with water R. Dilute 1.0 mL of this solution to 10.0 mL with water R. Column: — size: l = 0.25 m, Ø = 4.0 mm; — stationary phase: end-capped octadecylsilyl silica gel for chromatography R (5 µm); — temperature: 45 °C. Mobile phase: — mobile phase A: 3.764 g/L solution of sodium hexanesulfonate R adjusted to pH 3.1 with phosphoric acid R; — mobile phase B: methanol R1; Time (min); Mobile phase A (per cent V/V); Mobile phase B (per cent V/V). 0 - 2; 90; 10. 2 - 27; 90 → 70; 10 → 30. 27 - 35; 70 → 50; 30 → 50. 35 - 42; 50; 50. Flow rate 1.0 mL/min. Detection Spectrophotometer at 248 nm. Injection 25 µL. Identification of impurities Use the chromatogram supplied with thiamine for system suitability CRS and the chromatogram obtained with reference solution (a) to identify the peaks due to impurities A, B and C. Relative retention With reference to thiamine (retention time = about 30 min): impurity A = about 0.3; impurity B = about 0.9; impurity C = about 1.2. System suitability Reference solution (a): — resolution: minimum 3.0 between the peaks due to impurity B and thiamine; minimum 2.0 between the peaks due to thiamine and impurity C. Calculation of percentage contents: — for each impurity, use the concentration of thiamine hydrochloride in reference solution (b). |
|  | SULPHATE | Dilute 5 mL of solution S to 15 mL with distilled water R. All solutions used for this test must be prepared with distilled water R. Add 3 mL of a 250 g/L solution of barium chloride R to 4.5 mL of sulfate standard solution (10 ppm SO4) R1. Shake and allow to stand for 1 min. To 2.5 mL of this suspension add 15 mL of the prescribed solution and 0.5 mL of acetic acid R. Prepare a standard in the same manner using 15 mL of sulfate standard solution (10 ppm SO4) R instead of the prescribed solution. After 5 min, any opalescence in the test solution is not more intense than that in the standard. |
|  | WATER | Determined on 0.400 g. |
|  | SULPHATED ASH | Determined on 1.0 g @ 8000C. |
|  | ASSAY | Dissolve 0.110 g in 5 mL of anhydrous formic acid R and add 50 mL of acetic anhydride R. Titrate immediately with 0.1 M perchloric acid, determining the end-point potentiometrically and carrying out the titration within 2 min. Carry out a blank titration. 1 mL of 0.1 M perchloric acid is equivalent to 16.86 mg of C12H18Cl2N4OS. |
|  | MICROBIAL ENUMERATION TEST | Determination of total aerobic microbial count (TAMC) and total combined yeasts and molds count (TYMC) by the pour plate method. - Slightly open the lid of the petri plate and dispense 1 ml of diluted sample in the center of the Petri plate using a sterile calibrated pipette. - Open the lid of the prepared media. (soya bean digest agar for TAMC and Sabouraud Dextrose Agar for TYMC) - Pour about 15-20ml of the sterilized molten media at the appropriate temperature above the sample. - Close the lid of the plate then mix the sample in the media properly by gently swirling the plate. The plate is generally swirled in an “S” or “8” shape. - Allow the media to completely solidify. - Simultaneously perform the negative control by plating 1ml of diluent used instead of the sample for each medium. - Incubate the plate in an inverted position under suitable incubation conditions (For TAMC Incubate the plates of Soybean Casein Digest Agar at 30°C-35°C for 48hours and For TYMC incubate the plates of Sabouraud Dextrose Agar at 20°C - 25°C for 72hours Test for Absence of Pathogen Test for E. coli: Using same dilution - Transfer 1ml of the sample into a sterile McCartney bottle containing 9ml of sterile MacConkey broth. - Incubate the MacConkey broth bottle at 300C-350C for 24 hours. - After incubation shake the tube and transfer loop full portion of MacConkey broth on the surface of the pre incubated Eosin Methylene Blue Agar petri plate. - Incubate plates at 30°C to 35°C for 48 hours. - For negative control, proceeding same as above except without sample, streak a portion of incubated sterile MacConkey broth medium on the surface of pre incubated Eosin Methylene Blue Agar. - Observe and record the presence or absence of specified microorganisms. - Include positive and negative controls in each batch of tests. - Verify the performance of media and reagents used in the analysis. Interpretation of Results - The bacterial count should not exceed 102cfu/ml while that of the fungal count should not be more than 102cfu/ml. There should be absence of fecal coliform (E coli). - If growth occurs in the negative control plate, the test is invalid, failed negative control must be investigated - The total aerobic microbial count (TAMC) is considered to be equal to the number of CFU found using casein soya bean digest agar; if colonies of fungi are detected on this medium, they are counted as part of TAMC. - The total combined yeasts/mould count (TYMC) is considered to be equal to the number of CFU found using Sabouraud dextrose agar; if colonies of bacteria are detected on this medium, they are counted as part of TYMC - When an acceptance criterion for microbiological quality is prescribed it is interpreted as follows: 101 CFU : maximum acceptable count＝20 102 CFU: maximum acceptable count＝200, 103 CFU: maximum acceptable count＝2000. |

| - CURRENT REVISION | - SUPERSEDE | - REASON FOR REVIEW |
| --- | --- | --- |
| - 00 | - NIL | - FIRST ISSUE |
| - 01 | - 00 | - Review of the procedures - Change in header and footer format |