| S/N | TEST | DESCRIPTIONS OF PROCEDURE |
| --- | --- | --- |
|  | DESCRIPTION | Determine the colour and appearance of material |
|  | SOLUBILITY | Determine its solubility in water and in ethanol (96 per cent). |
|  | IDENTIFICATION | A. Solution S gives the reactions of sulfates . Dissolve about 45 mg of the substance to be examined in 5 mL of water R or use 5 mL of the prescribed solution. Add 1 mL of dilute hydrochloric acid R and 1 mL of barium chloride solution R1. A white precipitate is formed. b) To the suspension obtained during reaction (a), add 0.1 mL of 0.05 M iodine. The suspension remains yellow (distinction from sulfites and dithionites), but is decolorised by adding dropwise stannous chloride solution R (distinction from iodates). Boil the mixture. No coloured precipitate is formed (distinction from selenates and tungstates). B. Solution S gives the reaction of zinc .Dissolve 0.1 g of the substance to be examined in 5 mL of water R or use 5 mL of the prescribed solution. Add 0.2 mL of strong sodium hydroxide solution R. A white precipitate is formed. Add a further 2 mL of strong sodium hydroxide solution R. The precipitate dissolves. Add 10 mL of ammonium chloride solution R. The solution remains clear. Add 0.1 mL of sodium sulfide solution R. A flocculent white precipitate is formed. C. It complies with the limits of the assay. |
|  | APPEARANCE OF SOULTION | ### Solution S Dissolve 2.5 g in carbon dioxide-free water R and dilute to 50 mL with the same solvent. |
|  | pH | Determine the pH of solution S |
|  | CHLORIDES | Dilute 3.3 mL of solution S to 15 mL with water R. To 15 mL of the prescribed solution add 1 mL of dilute nitric acid R and pour the mixture as a single addition into a test-tube containing 1 mL of silver nitrate solution R2. Prepare a standard in the same manner using 10 mL of chloride standard solution (5 ppm Cl) R and 5 mL of water R. Examine the tubes laterally against a black background. After standing for 5 min protected from light, any opalescence in the test solution is not more intense than that in the standard. |
|  | IRON | Dilute 2 mL of solution S to 10 mL with water R. Use in this test 0.5 mL of thioglycollic acid R. Dissolve the prescribed quantity of the substance to be examined in water R and dilute to 10 mL with the same solvent or use 10 mL of the prescribed solution. Add 2 mL of a 200 g/L solution of citric acid monohydrate R and 0.1 mL of thioglycollic acid R. Mix, make alkaline with ammonia R and dilute to 20 mL with water R. Prepare a standard in the same manner, using 10 mL of iron standard solution (1 ppm Fe) R. After 5 min, any pink colour in the test solution is not more intense than that in the standard. |
|  | ASSAY | Dissolve 0.200 g in 5 mL of dilute acetic acid R. Carry out the complexometric titration of zinc. 1 mL of 0.1 M sodium edetate is equivalent to 28.75 mg of ZnSO4,7H20. |
|  | MICROBIAL ENUMERATION TEST | Determination of total aerobic microbial count (TAMC) and total combined yeasts and molds count (TYMC) by the pour plate method. - Slightly open the lid of the petri plate and dispense 1 ml of diluted sample in the center of the Petri plate using a sterile calibrated pipette. - Open the lid of the prepared media. (soya bean digest agar for TAMC and Sabouraud Dextrose Agar for TYMC) - Pour about 15-20ml of the sterilized molten media at the appropriate temperature above the sample. - Close the lid of the plate then mix the sample in the media properly by gently swirling the plate. The plate is generally swirled in an “S” or “8” shape. - Allow the media to completely solidify. - Simultaneously perform the negative control by plating 1ml of diluent used instead of the sample for each medium. - Incubate the plate in an inverted position under suitable incubation conditions (For TAMC Incubate the plates of Soybean Casein Digest Agar at 30°C-35°C for 48hours and For TYMC incubate the plates of Sabouraud Dextrose Agar at 20°C - 25°C for 72hours Test for Absence of Pathogen Test for E. coli: Using same dilution - Transfer 1ml of the sample into a sterile McCartney bottle containing 9ml of sterile MacConkey broth. - Incubate the MacConkey broth bottle at 300C-350C for 24 hours. - After incubation shake the tube and transfer loop full portion of MacConkey broth on the surface of the pre incubated Eosin Methylene Blue Agar petri plate. - Incubate plates at 30°C to 35°C for 48 hours. - For negative control, proceeding same as above except without sample, streak a portion of incubated sterile MacConkey broth medium on the surface of pre incubated Eosin Methylene Blue Agar. - Observe and record the presence or absence of specified microorganisms. - Include positive and negative controls in each batch of tests. - Verify the performance of media and reagents used in the analysis. Interpretation of Results - The bacterial count should not exceed 102cfu/ml while that of the fungal count should not be more than 102cfu/ml. There should be absence of fecal coliform (E coli). - If growth occurs in the negative control plate, the test is invalid, failed negative control must be investigated - The total aerobic microbial count (TAMC) is considered to be equal to the number of CFU found using casein soya bean digest agar; if colonies of fungi are detected on this medium, they are counted as part of TAMC. - The total combined yeasts/mould count (TYMC) is considered to be equal to the number of CFU found using Sabouraud dextrose agar; if colonies of bacteria are detected on this medium, they are counted as part of TYMC - When an acceptance criterion for microbiological quality is prescribed it is interpreted as follows: 101 CFU : maximum acceptable count＝20 102 CFU: maximum acceptable count＝200, 103 CFU: maximum acceptable count＝2000. |

| - CURRENT REVISION | - SUPERSEDE | - REASON FOR REVIEW |
| --- | --- | --- |
| - 00 | - NIL | - FIRST ISSUE |
| - 01 | - 00 | - Review of the procedures - Change in header and footer format |