| S/N | TEST | DESCRIPTIONS OF PROCEDURE |
| --- | --- | --- |
|  | pH | - Take sample sufficient to immerse the electrode-head, and dip the electrode in the sample. - Wait until the reading stabilises, then take readings. |
|  | Aluminium Hydroxide Magnesium hydroxide | METHOD 1: Procedure: Mix a quantity of the oral suspension equivalent to 5mL of syrup with 30 mL of 2M hydrochloric acid and add sufficient water to produce 100 mL. Dilute 10 mL of the solution obtained with 20 mL of water. Add 25 mL of 0.05M disodium edetate VS, mix, allow to stand for 5 minutes and add 20 mL of acetate buffer pH 4.4, 60 mL of ethanol (96%) and 2 mL of a 0.026% w/v solution of dithizone in ethanol (96%). Titrate with 0.05M zinc sulfate VS until a bright pink colour is obtained. Repeat the procedure without the substance being examined. The difference between the titrations represents the amount of disodium edetate required. Each mL of 0.05M disodium edetate VS is equivalent to 3.90 g of Al(OH)3. METHOD 1: Mix a quantity of the oral suspension equivalent to 5mL of syrup with 30 mL of 2M hydrochloric acid and add sufficient water to produce 100 mL. Dilute 10 mL of this solution to 200 mL with water. Add, while stirring, 1 g of ammonium chloride, 20 mL of triethanolamine, 10 mL of ammonia buffer pH 10.9 and 0.4 mL of mordant black 11 solution. Titrate with 0.05M disodium edetate VS until a blue colour is obtained. Repeat the procedure without the substance being examined. The difference between the titrations represents the amount of disodium edetate required. Each mL of 0.05M disodium edetate VS is equivalent to 2.916 g of Mg (OH)2. REFERENCE: British Pharmacopoeia, 2024, Ammonium Chloride: Ph. Eur. Monograph 0007. |
|  | Microbial Enumeration Test | - Determination of total aerobic microbial count (TAMC) and total combined yeasts and molds count (TYMC) by the pour plate method. - Dispense 1 ml of diluted sample in the center of the Petri plate using a sterile micropipette or calibrated pipette. - Open the lid of the prepared media, flame its mouth. - Pour about 15 mL of sterilized molten media at the appropriate temperature above the sample. - Close the lid of the plate then mix the sample in the media properly by gently swirling the plate. The plate is generally swirled in an “S” or “8” shape. - Allow the media to completely solidify. - Incubate the plate in an inverted position under suitable incubation conditions (Incubate the plates of Soybean Casein Digest Agar at 30°C-35°C for 3-5days for TAMC and the plates of Sabouraud Dextrose Agar at 20°C to 25°C for 5-7 days. TEST FOR ABSENCE OF PATHOGEN Test for E. coli: - Measure 10ml /10g of the samples, dilute 1n 90ml of nutrient broth - Transfer 10ml into 90ml of Tryptone soya broth - Incubate at 30-350c for 18-24hours - Transfer 1 ml of the dilution into 100ml of MacConkey broth and incubate at 42-44Oc in 18-72hours - Subculture on MacConkey Agar or Eosin Methylene blue Agar and incubate at 30-35Oc - For negative control, proceeding same as above except without sample, streak a portion of incubated sterile MacConkey broth medium on the surface of Eosin Methylene Blue Agar. - Incubate plates at 30 to 35° for 48 hours. Result Interpretation - Following the incubation, developed colonies are observed. Each colony will account for one viable microbial cell or one colony-forming unit. - If all the colonies are of the same type, then we can predict that the sample contained only one type of microbial genera. However, if there are different genera or species producing similar types of colonies, it is require to carry out identification test. - If the colonies are of different morphology, then we can conclude that the sample contained a mixed population. - Count the colonies and calculate CFU/mL by using the formula: CFU/mL = Total number of colonies obtained x dilution factor Volume of sample used (aliquot) - This will give the total number of viable microbial cells present in the given sample. - For optimum count, the number of colonies must be between 20 – 200 CFU/mL. Beyond this limit, the whole procedure must be repeated. - If the number of the colony is less than 20, it is suggested to use the sample of lower dilution, whereas, if the total number of colonies exceeds 200, it is suggested to use the sample of higher dilution on successive repeats. - If the colonies are fused or the whole plate is covered with a single colony, then report as “too numerous to count” (TNTC) and repeat the process of taking the sample at a higher dilution. |

| - CURRENT REVISION | - SUPERSEDE | - REASON FOR REVIEW |
| --- | --- | --- |
| - 00 | - NIL | - FIRST ISSUE |
| - 01 | - 00 | - Review of the procedures - Change in header and footer format |