BIOMEDICAL LIMITED

# INSTALLATION QUALIFICATION (IQ)

# REPORT

# FOR

# NON-PVC FILM AUTOMATIC BAG FORM-FILL-SEAL MACHINE

dm

BIOMEDICAL LIMITED

HORIZONTAL AUTO-PACKING MACHINE INSTALLATION QUALIFICATION PROTOCOL

EQUIPMENT SR. NO. –

EQUIPMENT ID. –

LOCATION – INFUSION FILLING ROOM

PROTOCOL No. – BML/PDF/FR/PT/25/001

REPORT No. - BML/PDF/FR/RP/25/001

PROVIDED BY:

BIOMEDICAL LIMTED

KWARA STATE, NIGERIA

# TABLE OF CONTENT

Cover Page											1

Table of content										2

Report Format Approval Sheet								3

Objective											4

Scope											5

Responsibilities										5

Equipment identification and description							5

Prerequisite										6

Abbreviations and definitions								6

References											7

Equipment installation Checklist								7-17

Installation Qualification Report								17

Deviation Report										18

Conclusion											19

List of attachments										19

Report Approval										20

- REPORT FORMAT APPROVAL:

Signatures in this page indicate that this document and its format have been prepared, checked and approved by the personnel concerned. It shall be signed and date as shown below

|  | WRITTEN: | REVIEWED: |
| --- | --- | --- |
| Signature: |  |  |
| Position: |  |  |
| Date: |  |  |

|  | APPROVED | AUTHORIZED |
| --- | --- | --- |
| Signature: |  |  |
| Position: |  |  |
| Date: |  |  |

2.0 OBJECTIVE

The purpose of the study is to ensure proper installation of the two NON-PVC FILM AUTOMATIC BAG FORM-FILL-SEAL MACHINE at THE FILLING ROOM, INFUSION FACTORY, BIOMEDICAL LIMITED.

This study verifies:

- Qualification of the equipment, reinstallation and to ensure that all background information is performed to assure conformance to BIOMEDICAL LIMITED and manufacturer’s specifications.

- The equipment meets the current good manufacturing practices requirements and all other regulatory obligations

- No unauthorized or unrecorded modifications have taken place.

- Operation procedures/preventive maintenance procedures have been identified and listed.

- All critical instrumentation has been identified.

- All utilities are properly connected.

- All safety features are accounted for.

- All relevant data have been checked.

3.0 SCOPE

- This report is applicable in carrying out the installation qualification of the two Form Fill Seal Machine at the infusion filling room, Biomedical Limited, Kwara State.

- To be performed after the completion of the design qualification

- To be performed at the time of relocation.

4.0 RESPONSIBILITY AND QUALIFICATION TEAM

The following departments shall be involved in:

| DEPARTMENT | RESPONSIBILITY |
| --- | --- |
| Engineering Team | Provision of needed utility and machine installation |
| Head Of Engineering | Review of the Installation Qualification Document |
| QA Manager | Final Documentation and system acceptability |
| Superintendent Pharmacist/factory Manager | Authorization |

The following personnel will be involved in the execution of the Qualification

| S/N | DEPARTMENT | NAME | SIGNATURE |
| --- | --- | --- | --- |
| 1 |  |  |  |
| 2 |  |  |  |
| 3 |  |  |  |
| 4 |  |  |  |
| 5 |  |  |  |

5.0 EQUIPMENT IDENTIFICATION AND DESCRIPTION

IDENTIFICATION

| Equipment Name | NON-PVC FILM AUTOMATIC BAG FORM-FILL-SEAL MACHINE |
| --- | --- |
| Manufacturer’s Name | SHANGAI IVEN PHARMATECH ENGINEERING CO., LTD, CHINA |
| Supplier’s name | SHANGAI IVEN PHARMATECH ENGINEERING CO., LTD, CHINA |
| Location of Installation | FILLING ROOM (INFUSION FACTORY) |

DESCRIPTION:

………………………………………………………………………………………………………………………………………………..………………………………………………………………………………………………………………………………………………..

………………………………………………………………………………………………………………………………………………..

………………………………………………………………………………………………………………………………………………..

………………………………………………………………………………………………………………………………………………..

………………………………………………………………………………………………………………………………………………....................................................................................................................................……………..

6.0 PREREQUISITE

The equipment shall conform to the specifications and requirements as specified in the Design Qualification

| S/N | ACCEPTANCE CRITERIA |
| --- | --- |
| 1 | User Requirement Specification |
| 2 | Design Qualification |

7.0 ABBREVIATIONS AND DEFINITIONS

| Abbreviation | Definition |
| --- | --- |
| SOP | Standard Operating Procedure |
| MOC | Material of Construction |
| CO | Company |
| ENGR | Engineering |
| IQ | Installation Qualification |
| QA | Quality Assurance |
| VAC | Voltage Alternating Current |
| S. S | Stainless Steel |
| LTD | Limited |
| °C | Degree Celsius |
| V | Voltage |
| GMP | Good manufacturing practices |
| AC | Alternating Current |
| Amps | Ampere |
| W | Watt |
| Mm | Millimeter |
| VDC | Voltage Direct Current |
| DC | Direct Current |
| PVC | Polyvinyl Chloride |
| MCB | Miniature Circuit Breaker |
| MCCB | Molded case circuit breaker |
| KW | Kilowatt |
| PLC | Programmable Logic Control |

8.0 REFERENCES:

The list of reference documents, which are referred for preparation and review of this document, is as follows: -

- S.O.P for Installation, Operation & Performance Qualification of equipment & Instrument: BML-QDG-QA-111

- Machine Manual supplied by SHANGAI IVEN PHARMATECH ENGINEERING CO., LTD

- Design Qualification supplied by SHANGAI IVEN PHARMATECH ENGINEERING CO., LTD

9.0 EQUIPMENT INSTALLATION CHECKLIST:

Pre-Installation Consideration

| S/N | Description | Qualified / Not Qualified | Checked By | Verified By |
| --- | --- | --- | --- | --- |
| 1 | There is no observable physical damage to the machine and all the contact parts are free from scratches & damages. |  |  |  |
| 2 | Sufficient room for servicing, handling & maintenance is provided (around the machine) |  |  |  |
| 3 | Check the floor space. It should not be inclined. |  |  |  |

Assembly of non-assembled items

| S/N | Description | Qualified / Not Qualified | Checked By | Verified By |
| --- | --- | --- | --- | --- |
| 1 | Assemble/connect the machine Laminar air flow hood to the main machine assembly |  |  |  |
| 2 | Assemble/connect the HEPA Filters to the main machine assembly |  |  |  |
| 3 | Assemble/connect the machine Laminar air flow hood prefilters to the main machine assembly |  |  |  |
| 4 | Assemble/connect the machine Perspex doors to the main machine assembly |  |  |  |

Utilities

| S/N | Description | Qualified / Not Qualified | Checked By | Verified By |
| --- | --- | --- | --- | --- |
| 1 | Power Requirement (Three-phase, 5 wires, L1, L2, L3, Neutral & Earth 415VAC ±10% 50Hz) |  |  |  |
| 2 | Adequate illumination at the Proposed Machine room |  |  |  |
| 3 | Proper Earthing System available |  |  |  |

Equipment compliance

| S/N | Description | Qualified / Not Qualified | Checked By | Verified By |
| --- | --- | --- | --- | --- |
| 1 | Machine name plate present |  |  |  |
| 2 | Machine Model Number: SRD4-3600BPH |  |  |  |
| 3 | Machine Factory Number: 20244755-202R |  |  |  |
| 4 | Power Rating: 32KW |  |  |  |

Safety compliance

| S/N | Description | Qualified / Not Qualified | Checked By | Verified By |
| --- | --- | --- | --- | --- |
| 1 | All external surfaces are free of sharp edges. |  |  |  |
| 2 | Safety earthing arrangements are available. |  |  |  |
| 3 | Emergency Stop Buttons available |  |  |  |
| 4 | Electrical disconnect available |  |  |  |

Electrical diagram check

| S/N | Description | Qualified / Not Qualified | Checked By | Verified By |
| --- | --- | --- | --- | --- |
| 1 | Verify the electrical diagram |  |  |  |
| 2 | Check whether the position of each component in the electrical circuitry is correct |  |  |  |
| 3 | Check whether the wiring of the equipment is consistent with electrical diagrams |  |  |  |
| 4 | Check if the electrical diagram is attached to the operation manual |  |  |  |

P&ID check

| S/N | Description | Qualified / Not Qualified | Checked By | Verified By |
| --- | --- | --- | --- | --- |
| 1 | Check that the filling pipe system is consistent with the P&ID drawing |  |  |  |
| 2 | Check that the supplier and model number of each component match |  |  |  |
| 3 | Check that the component installed are consistent with the P&ID part list |  |  |  |
| 4 | Check that the P&ID is attached to the operation manual |  |  |  |

List of Main Components with their material of construction

| S/N | Component | Origin/ Configuration | Qualified / Not Qualified | Checked By |  | Verified By |
| --- | --- | --- | --- | --- | --- | --- |
| MECHANICAL PART LIST |  |  |  |  |  |  |
| 1 | Compact cylinder | Location: Film feeding station, Brand: Festo, Specification: ADN-20-25-I-P-A |  |  |  |  |
| 2 | Brake Motor | Location: Film feeding station, Brand: Sew, Specification: KAF37DRN71MS4/BE03/TH |  |  |  |  |
| 3 | Ionic wind bar | Location: Film feeding station, Brand: SMC, Specification: ZS41-340P-06B |  |  |  |  |
| 4 | Compact double force cylinder | Location: Printing station, Brand: Festo, Specification: ADNH-100-50-A-P-A-2N-100K8 |  |  |  |  |
| 5 | Standard cylinder | Location: Printing station, Brand: Festo, Specification: ADN-16-10-I-P-A |  |  |  |  |
| 6 | Brake motor | Location: Printing station, Brand: Sew, Specification: WAF30 DR63L4/BR/THWAF30DRN71MS4/BE0 3/TH |  |  |  |  |
| 7 | Parallel gripper | Location: Film Pulling, Brand: Festo, Specification: DHPS-25-A |  |  |  |  |
| 8 | Double piston- cylinder | Location: Film Pulling, Brand: Festo, Specification: DPZJ-32-50-P-A-S2 |  |  |  |  |
| 9 | Electric cylinder | Location: Film Pulling, Brand: Festo, Specification: ELGA-TB-KF-80-830-0H |  |  |  |  |
| 10 | Parallel gripper | Location: pre heating, Brand: Festo, Specification: DHPS-35-A |  |  |  |  |
| 11 | X Eliminator | Location: Bag Transferring, Brand: SMC, Specification: IZN10E-11P06Z-B1 |  |  |  |  |
| 12 | Cylinder with guide rod | Location: Bag making station, Brand: Festo, Specification: DFM-25-30-P-A-GF |  |  |  |  |
| 13 | Double piston cylinder | Location: Bag making station, Brand: Festo, Specification: DPZJ-25-25-P-A-S2 |  |  |  |  |
| 14 | Cylinder with guide rod | Location: Bag making station, Brand: Festo, Specification: DFM-32-20-P-A-GF |  |  |  |  |
| 15 | TOX buster Cylinder | Location: Bag making station, Brand: TOX, Specification: EL8.160.50.6D |  |  |  |  |
| 16 | Pneumatic linear drive unit | Location: port feeding, Brand: Festo, Specification: DGPL-32-250-PPV-A-B-KF-GK-SH-GA |  |  |  |  |
| 17 | Hydraulic buffer | Location: port feeding, Brand: Festo, Specification: YSR-12-12-C |  |  |  |  |
| 18 | Medium drive guider | Location: port feeding, Brand: Festo, Specification: DFM-50-50-P-A-GF |  |  |  |  |
| 19 | Straight ball guide pneumatic drive unit | Location: port feeding, Brand: Festo, Specification: DGPL-25-185-PPV-A-KF-B |  |  |  |  |
| 20 | Gripper | Location: port feeding, Brand: Festo, Specification: DHPS-25-A |  |  |  |  |
| 21 | Cylinder | Location: port feeding, Brand: Festo, Specification: ADNGF-25-15-P-A |  |  |  |  |
| 22 | X eliminator | Location: port feeding, Brand: SMC, Specification: IZN10E-11P06Z-B1 |  |  |  |  |
| 23 | Cylinder | Location: port welding, Brand: Festo, Specification: DFM-63-25-P-A-GF |  |  |  |  |
| 24 | Rotating gripper | Location: port welding, Brand: Festo, Specification: DHRS-25-A |  |  |  |  |
| 25 | Circular cylinder | Location: port welding, Brand: Festo, Specification: DSNU-20-50-PPV-A |  |  |  |  |
| 26 | Hydraulic buffer | Location: transferring station, Brand: Festo, Specification: DYSC-7-5-Y1F |  |  |  |  |
| 27 | Cylinder with guide rod | Location: transferring station, Brand: Festo, Specification: DFM-32-20-P-A-GF |  |  |  |  |
| 28 | Double rod cylinder | Location: transferring station, Brand: Festo, Specification: DPZJ-20-80-P-A-S2 |  |  |  |  |
| 29 | Double rod cylinder | Location: transferring station, Brand: Festo, Specification: DPZJ-25-80-P-A-S2 |  |  |  |  |
| 30 | Rodless cylinder | Location: transferring station, Brand: Festo, Specification: DGPL-40-450-PPV-A-B-KF-GK-SH-GA |  |  |  |  |
| 31 | Rotating Cylinder | Location: transferring station, Brand: Festo, Specification: DSM-25-270-P-A-B |  |  |  |  |
| 32 | Hydraulic Buffer | Location: transferring station, Brand: Festo, Specification: DG-GA-40-YSR |  |  |  |  |
| 33 | Compact Guide Cylinder | Location: filling, Brand: Festo, Specification: ADNGF-100-30-P-A |  |  |  |  |
| 34 | Double Rod Cylinder | Location: filling, Brand: Festo, Specification: DPZJ-32-40-P-A-S2 |  |  |  |  |
| 35 | Double Acting Cylinder | Location: filling, Brand: Festo, Specification: DPZJ-32-50-P-A-S2 |  |  |  |  |
| 36 | Hydraulic Buffer | Location: filling, Brand: Festo, Specification: YSR-12-12-C |  |  |  |  |
| 37 | Steam Trap | Location: filling, Brand: customized, Specification: 1811-DN20,1/8" |  |  |  |  |
| 38 | Compact cylinder | Location: cap sealing, Brand: Festo, Specification: ADNGF-32-30-P-A |  |  |  |  |
| 39 | Toothed belt electric cylinder | Location: cap sealing, Brand: Festo, Specification: ELGA-TB-KF-80-300-0H |  |  |  |  |
| 40 | Double piston cylinder | Location: cap sealing, Brand: Festo, Specification: DPZ-32-40-P-A |  |  |  |  |
| 41 | Standard cylinder | Location: cap sealing, Brand: Festo, Specification: DSBC-32-100-PPVA-N3 |  |  |  |  |
| 42 | Double piston cylinder | Location: cap sealing, Brand: Festo, Specification: DPZJ-20-25-P-A-S2 |  |  |  |  |
| 43 | X Eliminator | Location: cap sealing, Brand: SMC, Specification: IZN10E-11P06Z-B1 |  |  |  |  |
| 44 | Rotating gripper | Location: Output station, Brand: Festo, Specification: DHWS-32-A |  |  |  |  |
| 45 | Rodless Cylinder | Location: Output station, Brand: Festo, Specification: DGPL-40-750-PPV-A-B-KF-GK-SV-GA |  |  |  |  |
| 46 | Hydraulic Buffer | Location: Output station, Brand: Festo, Specification: YSR-16-20-C |  |  |  |  |
| 47 | European standard cylinder | Location: Output station, Brand: Customized, Specification: DNC06013 |  |  |  |  |
| CRITICAL SENSORS |  |  |  |  |  |  |
| 48 | Mass flow meter | Function: Measure filling volume, Vendor: Endress + Hauser, Model: 8BE08-AFTSABAB41A3, Range: 0.1g-∞, Accuracy: 0.1 |  |  |  |  |
| 49 | Thermocouples | Function: Measure die Temperature, Vendor: Tianjin Zhonghuan, Model: RSY-DO/K-01, Range: -200-1250°C, Accuracy: 1.1°C |  |  |  |  |
| 50 | Thermocouples | Function: Measure die Temperature, Vendor: Tianjin Zhonghuan, Model: RSY-DO/K-04, Range: -200-1250°C, Accuracy: 1.1°C |  |  |  |  |
| 51 | Thermocouples | Function: Measure die Temperature, Vendor: Tianjin Zhonghuan, Model: RSY-DO/K-02, Range: -200-1250°C, Accuracy: 1.1°C |  |  |  |  |
| 52 | Magnetic switch | Function: Detect Cylinder position, Vendor: IFM, Model: MK5101, Range: NA, Accuracy: NA |  |  |  |  |
| 53 | Inductive Transducer | Function: Detect position, Vendor: IFM, Model: IE5329, Range: NA, Accuracy: NA |  |  |  |  |
| 54 | Photoelectric Switch | Function: Detect position, Vendor: BANNER, Model: Q12AB6FF30, Range: 0-30mm, Accuracy: NA |  |  |  |  |
| 55 | Photoelectric Switch | Function: Detect material level in the hopper, Vendor: BANNER, Model: QS30AF, Range: 0-30cm, Accuracy: NA |  |  |  |  |
| 56 | Photoelectric sensor | Function: Detect bag, Vendor: BANNER, Model: QS18VP6XLPQ7, Range: NA, Accuracy: NA |  |  |  |  |
| 57 | Mutual Inductor | Function: Measure heating plate current, Vendor: Tianjin Zhonghuan, Model: TCYT, Range: 0-200A, Accuracy: 1A |  |  |  |  |
| 58 | Temperature sensor | Function: Measure SIP Temperature, Vendor: Zibo Feiyan, Model: XH-PT2-143122-150, Range: -50-200°C, Accuracy: 1.1°C |  |  |  |  |
| 59 | Flow switch | Function: Detect cooling water, Vendor: SENTINEL, Model: PLSX-1/2G-AP6D-SC, Range: 0-5 m/s, Accuracy: NA |  |  |  |  |
| 60 | Pressure sensor | Function: Detecting negative pressure gas, Vendor: SMC, Model: ZSE40AF-01-Y-MA, Range: -1-16 bar, Accuracy: 1.5%FS |  |  |  |  |
| 61 | Pressure sensor | Function: Detect gas source Pressure, Vendor: Festo, Model: SPAU-P10R-W-G18FD-L PNLK-PNVBA-M8U, Range: 0-10 bar, Accuracy: 1.5%FS |  |  |  |  |
| ELECTRICAL HARDWARE LIST |  |  |  |  |  |  |
| 62 | PLC | Function: Handle all program running events as the main controller, Vendor: SIEMENS, Order Number: 6ES7512-1CK01-0AB0 |  |  |  |  |
| 63 | Digital Input Module (DI) | Function: Read digital input signal, Vendor: SIEMENS, Order Number: 6ES7521-1BL00-0AB0 |  |  |  |  |
| 64 | Digital Output Module (DQ) | Function: Output digital input signal, Vendor: SIEMENS, Order Number: 6ES7532-5HD00-0AB0 |  |  |  |  |
| 65 | Analog Input Module (AI) | Function: Read analog input signal, Vendor: SIEMENS, Order Number: 6ES7531-7QF00-0AB0 |  |  |  |  |
| 66 | Analog Output Module (AQ) | Function: Output analog input signal, Vendor: SIEMENS, Order Number: 6ES7532-5HD00-0AB0 |  |  |  |  |
| 67 | Memory Card (SD) | Function: Store Program, Vendor: SIEMENS, Order Number: 6ES7954-8LE03-0AA0 |  |  |  |  |
| 68 | Touch screen (HMI) | Function: Human Machine Interface, interactive data with controller, Vendor: SIEMENS, Order Number: 6AV2124-0MC01-0AX0 |  |  |  |  |
| 69 | Switchboard | Function: Exchange data for various controller, Vendor: SIEMENS, Order Number: 6GK5008-0BA10-1AB2 |  |  |  |  |
| 70 | Switching Power Supply (PW) | Function: Provide power supply, Vendor: MEANWELL, Order Number: SDR-960-24 |  |  |  |  |
| 71 | Relay (KA) | Function: Isolate power supply and extend contacts, Vendor: OMRON, Order Number: MY2NJ-24V |  |  |  |  |
| 72 | Solid State Relay (GK) | Function: Control output in temperature control, Vendor: CYDON, Order Number: KSI240D25-LM(068) |  |  |  |  |
| 73 | Safety Relay (KNA) | Function: Provide personnel and equipment in case of emergency, Vendor: Schmersal, Order Number: SRB301MC-24V |  |  |  |  |
| 74 | Silicon Controlled Rectifier Power Regulator (KKG) | Function: Provide the required current value to the heater chip Vendor: CYDON, Order Number: CRB480H75P;75A;480VAC |  |  |  |  |
| 75 | Molded case circuit breaker (QF) | Function: Connect and disconnect circuit, Vendor: SCHNEIDER, Order Number: C11E6TM063BZ |  |  |  |  |
| 76 | Contactor (KM) | Function: isolate power supply and extends contact, Vendor: SCHNEIDER, Order Number: LC1D12BDC LC1D38BDC |  |  |  |  |
| 77 | Miniature Circuit breaker (QF) | Function: connect and disconnect circuit, Vendor: SCHNEIDER, Order Number: OSMC32N3C16 OSMC32N3C40 OSMC32N1C10 OSMC32N1C16 OSMC32N1C1 OSMC32N1C4 OSMC32N2D16 OSMC32N2C25 |  |  |  |  |
| 78 | Motor Starter (Q) | Function: protect the motor against phase loss, Vendor: SCHNEIDER, Order Number: GV2ME10C GV2ME07C |  |  |  |  |
| 79 | Frequency Converter (BP) | Function: Adjust the motor frequency speed, Vendor: SCHNEIDER, Order Number: ATV310H037N4A |  |  |  |  |
| 80 | Controller (SF) | Function: Precise control of motor positioning, Vendor: LENZE, Order Number: E70ACMSE0104SA2ETE |  |  |  |  |
| 81 | Valve Terminal (FD) | Function: Provide Input Signal and control cylinder action, Vendor: Festo, Order Number: 45P-R-W-LVMX-8EU8EU-32M |  |  |  |  |
| LAMINAR AIR FLOW HOOD COMPONENTS |  |  |  |  |  |  |
| 82 | Air Blower | Model: EBM355/310 Fan, MOC: SS 304, Manufacturer: EBM, Quantity: 28 Pieces |  |  |  |  |
| 83 | LED Light | Model: T5, Manufacturer: Philips, Quantity: 56 Pieces |  |  |  |  |
| 84 | Differential Pressure Gauge | Model: 0-500A/DM-105-SS, Manufacturer: Dwyer Quantity: 12 Pieces |  |  |  |  |
| 85 | High Efficiency Filter (H14) | Dimensions: 961*569*90mm (2 Pieces) 961*506*90mm (5 Pieces) 961*656*90mm (1 Piece) 961*545*90mm (1 Piece) 743*468*90mm (2 Pieces) 961*553*90mm (2 Pieces) 791*468*90mm (2 Pieces) 645*403*90mm (4 Pieces) 941*403*90mm (2 Pieces) 821*612*90mm (4 Pieces) 821*447*90mm (2 Pieces) 821*417*90mm (2 Pieces) Manufacturer: IVEN |  |  |  |  |
| 86 | PAO Injection Port | MOC: SS 304, Manufacturer: IVEN, Quantity: 28 Pieces |  |  |  |  |
| 87 | PAO Test Port | MOC: SS 304, Manufacturer: IVEN, Quantity: 28 Pieces |  |  |  |  |
| 88 | Aluminum frame, black nylon mesh | 525*345*10 (4 Pieces) 740*345*10 (2 Pieces) 1090*345*10 (1 Pieces) 940*345*10 (1 Pieces) 640*345*10 (7 Pieces) 1040*345*10 (1 Pieces) 540*345*10 (2 Pieces) 840*345*10 (1 Pieces) 500*345*10 (2 Pieces) 920*345*10 (4 Pieces) |  |  |  |  |

10.0   INSTALLATION QUALIFICATION REPORT:

Attachment of all reports written as:

#Attachment 1

#Attachement 2 etc.

Date of Installation:

Date of Completion:

11.0 DEVIATION REPORT

Prepare a deviation report including the justification of acceptance and impact on the function of the system/equipment

DEVIATIONS:

JUSTIFICATION FOR ACCEPTANCE:

IMPACT ON OPERATION:

Written By:										Date:

Reviewed By:										Date:

Approved By:										Date:

12.0 CONCLUSION

13.0 List of Attachment

Attach all completed, signed data record forms

14.0 REPORT APPROVAL

Written By:										Date:

Reviewed By:										Date:

Approved By:										Date: