- OBJECTIVE:

  - To ensure proper representative and aseptic sample collection to reduce the risk of contamination of either the sample or the material remaining in the container or of cross contamination

  - To ensure specified tests on all raw materials are carried out.

  - To ensure only raw materials tested, passed and approved are used for production

2.0 	SCOPE:

This SOP applies to sampling of all starting materials (actives and excipients), cleaning and sanitizing agents, water treatment chemicals and other processing agents sampling by the Quality Control Unit.

3.0 	RESPONSIBILITIES/ACCOUNTABILITIES:

- The sampler shall carry out aseptic sampling of the raw material to reduce the risk of contamination of either the sample or the material remaining in the container or of cross contamination

- The Chemical analyst shall report any deviation or out of specification

- The Quality Control manager reviews results and recommends for release

4.0	ABBREVIATIONS/DEFINITIONS:

  - Sample: A portion of a material collected according to a defined sampling procedure. The size of any sample should be sufficient to allow all anticipated test procedures to be carried out, including all repetitions and retention samples.

  - Combined sample: Sample resulting from combining all or parts of two or more samples of the material

  - Sampler: Person responsible for performing the sampling operation

  - RLAF: Reverse Lamina Air Flow

  - Batch: A quantity of any drug produced within a given cycle of manufacture. If the manufacturing process is continuous, the batch originates in a given period of time in which manufacturing process are stable and not modified

  - Consignment: The quantity of a starting material or drug product made by one manufacturer or supplied by an agent in response to a particular request or order. A consignment may comprise one or more lot-identified packages or containers and may include materials belonging to more than one lot identified batch

  - Final sample: Sample ready for the application of the test procedure

  - Representative sample: Sample obtained according to a sampling procedure designed to ensure that the different parts of a batch or the different properties of a non-uniform material are proportionately represented

  - Random sample: Sample in which the different fractions of the material have an equal probability of being represented.

  - Sampling unit: discrete part of a consignment such as an individual package, drum or container

  - Original sample: sample collected directly from the material

  - IPQA: In-process Quality Assurance

5.0       PROCEDURE:

- Upon receipt of material Intimation form BML-WHG-WH-003-F01 request for the sampling of raw materials from the Materials warehouse to quality assurance department.

- The in-process quality assurance personnel shall acknowledge the intimation and receipt of the manufacturer’s certificate of analysis.

- Read and understand the precautions to be observed for the safe handling of the material

- Gather the required sampling equipment and check that all items are clean

- Inspect all material containers in the warehouse for damages, dents, leakages, tear before sampling. Report any faults to the materials manager and the quality control manager or designee

- Examine all the containers for obvious differences and signs of changes, including obliterations and mislabeling. Ensure that the following information is clearly written on both the Certificate of Analysis and Raw Material container.

    - Material Name

    - Material Batch Number

    - Manufacturing date

    - Expiry date

    - Manufacturers Name and Address.

- Report any fault to the Quality Control Manager or designee

- Segregate any damaged containers and those with suspected spoiled/contaminated content for separate examination. These should be referred or rejected.

- Separate containers according to batch numbers and number them.

- Choose the appropriate sample plan

- Sampling n-plan shall be used to determine the number of containers of to be sampled. n = √N +1 (where N is the number of containers/bags per batch, n is obtained by simple rounding up) is to be used when material to be sampled is considered uniform and is supplied from a recognized source. Samples can be withdrawn from any part of the container (usually the top). Where N is ≤ 4, when every container is sampled. However, 100% sampling and identification test shall also be carried out for APIs which will be carried out just before the point of dispensing.

- Sampling r-plan shall be used to determine the number of containers to be sampled. r = 1.5 √N (where N is the number of containers/bags per batch, r is obtained by simple rounding up to the next highest integer) is to be used when material to be sampled is considered non - uniform and is supplied from a source not well known. Samples can be withdrawn from any part of the container (usually the top). This plan shall be used for pre purchase materials. Samples will be taken from each unit

- The p-plan may be used when the material is uniform, is received from a recognized source and the purpose is to test for identity. The p-plan is based on the formula p=0.4√N where N is the number of sampling units. The figures for p are obtained by rounding up to the next higher integer. According to this plan, samples are taken from each of the N sampling units of the consignment and placed in different sample containers.

- Samplers are to observe clean-room practice with respect to absence of watches, jewelries, cosmetics et al.

- Personal Protective Equipment like nose mask, overalls, factory shoes or covers and disposable latex hand gloves are to be worn.

- Ensure that neither the material nor the sample taken is exposed to contaminants.

- Do not touch any material with your hands.

- Containers due for sampling should be cleaned prior to sampling if necessary.

- Sampling of starting materials from large containers can present difficulties. Whenever possible, sampling can be carried out in a separate, clean area in the warehouse to reduce the risk of contamination

- Pass the material for sampling through the pass box from the quarantine area.

- Remove the material from the pass-box and close the door.

- Ensure the sampling booth is cleaned before use.

- Open containers one at a time and physically inspect the content. Report any differences

- Open the outer packaging carefully with pliers, scissors.

- Then open the inner container carefully or a quantity sufficient for analysis,

- Using the appropriate sampling tool take a representative sample, sufficient for testing in accordance with specification, repetition and retention, of the raw material.

- Use the weighing balance provided.

- Reseal, properly, the containers from which samples were withdrawn and label ‘SAMPLED’. Label on the container should indicate the container number (E.g X of Y) where X is the actual container number and Y is the total number of the batch.

- Closures and labels should be such that unauthorized opening can be detected

- Samples should NEVER be returned to the bulk

SAMPLED MATERIAL LABEL Material Name: __________________________________ Item code: ______________________________________ Lot/Batch No: ___________________________________ Mfg date: _______________________________________ Expiry Date: _____________________________________ Sampled by: _____________________________________ Sign/Date/Time: __________________________________ Container No: ____________________ of _____________ SAMPLED

- Label the sampled material container appropriately which has the following information:

    - Name of sampler

    - Date Sampled

    - Name of material and Batch number

- Under Test label (Yellow) should be placed on the material and the material moved to the quarantine zone of the warehouse.

- After sampling, all sampling tools used and the sampling booth are properly cleaned.

- Log in the sampled material in raw material logbook (LGB/QDG/05/CLF/01) or (LGB/QDG/05/CLS/01) respectively. The following information must be written in the log book:

Raw Material Logbook (LGB/QDG/05/CLF/01) or (LGB/QDG/05/CLS/01)

| Date | Request No. | Material name | MFD | EXD | Remark | AR. NO. | Manufacturer/ Supplier name | Analysed By |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

- Assign laboratory reference numbers to each batch/consignment of raw material received.

- The raw material sample(s) is/are assigned to an analyst on the raw material bench.

- Proceed for analysis as per the standard testing procedure

- Ensure all reagents, chemicals and solutions for analysis are available and in good condition

- If volumetric solutions is required for analysis ensure the standard factor is current by checking the reagent preparation register for its validity.

- Where applicable, chemical, physical, instrumental, Microbiological, Pyrogen analyses are to be carried out on all materials

- After all test/analysis is done, reports are to be made in the analytical report and all report are to be endorsed by the analyst, check and sign by the QA Manager, after which Certificate of analysis must be prepared.

- Prepared COA must be checked and signed by the QA Manager; each COA is filed in the appropriate place.

- Retain sample(s) following the raw material retention procedure.

- REFERENCE AND ENCLOSURE:

- Material Intimation form BML-WHG-WH-003-F01

- WHO technical report series; WHO guidelines for sampling pharmaceutical products and related materials.

- ISO 2859-10-2006

7.0 TRAINING INVOLVEMENT

| All departmental managers | All sectional heads |
| --- | --- |
| QA Doc. Unit staff | Store staff |

8.0 DISTRIBUTION AND DOCUMENTATION CONTROL:

| MASTER COPIES | QUALITY ASSURANCE |
| --- | --- |
| CONTROLLED COPIES | ALL DEPARTMENT |

9.0 REVISION HISTORY

| CURRENT REVISION | SUPERSEDE | REASON FOR REVIEW |
| --- | --- | --- |
| ### 02 | ### 01 | To Update |
| ### 03 | ### 02 | - Inclusion of enclosures format - Change in sop format generally |