- OBJECTIVE:

- To provide the steps to be employed in finger dabbing and analysis.

- To determine the level of hygiene of personnel in the clean areas.

- SCOPE:

- This procedure needs to be performed every week.

- It is applicable for the personnel in the critical areas such as Filling Room and mixing room

- RESPONSIBILITY/ACCOUNTABILITY:

- Microbiologists perform this procedure on each personnel in the clean areas.

- The Quality Control Head to ensure that it is performed.

4.0 ABBREVIATIONS AND DEFINITIONS

- CFU – Colony forming unit

- PPE – Personal Protective Equipment

- PROCEDURE

- Prepare the media SDA, TSA and EMB as per sop for preparation of culture media BML-QDG-MC-001

- In the laminar air flow, aseptically pour approximately 15-20 ml of sterile media (45oc) into sterile petri plates and allow to solidify

- pre incubate the plates to check for any contamination

- label the plates appropriately and place in a sterile sampling box and transfer to the clean area

- The work bench on which sampling is to perform should be disinfected with 70% IPA.

- The lid or cover of the Petri dish containing the different media should be slightly opened aseptically one a time by the analyst while the person to be fingers dabbed gently touches the media with the tips of right-hand fingers followed by the left-hand fingers momentarily.

- The cover should be placed immediately and labeled with the personnel data.

- The plates containing tryptic soybean agar and EMB agar should be incubated invertedly in an incubator at 30 – 35oC for 24 – 48 hours while that containing sabouroud dextrose agar should be incubated at 20 – 25oC for 72hours.

Precautions

- Maintain aseptic condition during testing

- After finger dab testing, immediately wash the hands with sterile 70% IPA solution

- Result should be expressed per five fingers

- All pre incubated plates should be rejected if a single plate shows evidence of microbial contamination

READING AND INTERPRETATION OF RESULTS:

- The total microbial count for any staff should not exceed 10 cfu/ five fingers and there must be absence of fecal coliform.

- The result should be recorded into the finger dabbing analysis record sheet BML-QDG-MC-006-F01(infusion) and BML-QDG-MC-006-F02(syrup)

- Frequency: should be done on daily basis

REPORTING:

- The results should be recorded in the Finger dabbing sheet/record book.

- The results should be reported to QAM and appropriate production personnel if specification is exceeded.

MATERIALS AND EQUIPMENT:

- Sterile Petri dishes

- Sterile tryptic soybean agar

- Sterile sabouroud dextrose agar

- Sterile Eosin Methylene blue

- 70% IPA

- Cotton wool

- Laminar flow cabinet

- Incubator

- Bunsen burner

Safety, Health and Environment Precautions

Wear appropriate PPE

- REFERENCE/ENCLOSURE

- Infusion Finger dabbing analysis record sheet - BML/QDG/MC/006/F01

- Syrup Finger dabbing analysis record sheet - BML/QDG/MC/006/F02

- TRAINING INVOLVEMENT

| Quality Control Head | Quality control microbiologist, Laboratory assistant |
| --- | --- |

8.0 DISTRIBUTION AND DOCUMENTATION CONTROL:

| MASTER COPIES | QUALITY ASSURANCE |
| --- | --- |
| CONTROLLED COPIES | QUALITY CONTROL LABORATORIES |

9.0 REVISION HISTORY

| CURRENT REVISION | SUPERSEDE | REASON FOR REVIEW |
| --- | --- | --- |
| ### 00 | ### Nil | First issue |
| ### 02 | ### 01 | To Update |
| ### 03 | ### 02 | - Inclusion of enclosures format - Change in sop format generally |