1.0 	OBJECTIVE:

To ensure the proper storage of chemicals, reagents are used in the chemistry unit of the quality control laboratory.

2.0 	SCOPE:

This SOP applies to all reagents, media and test kits used in the quality control laboratory.

3.0 	RESPONSIBILITIES/ACCOUNTABILITIES:

- The Chemical analyst shall receive and prepare the reagents and solutions as prescribed in the monograph

- The Chemical analyst shall carry out the tests and standardization of reagents as prescribed in the monograph

- The Quality Control Analyst reports any incident that may occur during procedure

- ABBREVIATIONS AND DEFINATIONS:

- Reagents: reagents are substances used either as such or as constituents of solutions

- Indicators: indicators are reagents used to determine the specified end point in a chemical reaction, to measure pH or show colour change

- Solution (fluid with substance dissolved in it): a substance consisting of two or more substances mixed together and uniformity dispersed, most commonly the result of dissolving a solid, fluid or gas in a liquid.

- Buffers: Buffers resist changes in the activity of an ion on the addition of substances that are expected to change the activity of the ion.

- Test solutions (TS): TS are solutions of reagents in such solvents and of such definite concentrations as to be suitable for the specified purpose

- Volumetric solution (VS): VS are solutions of reagents of known concentration intended primarily for use in quantitative determination

- Concentration (strength of a solution): the amount of a substance dissolved in another.

- Dilution: (less concentrated liquid): a substance, especially liquid, that has been made weaker by the addition of water or another liquid

- Molarity: number of moles of a substance present in one litre of a solution (mol/L or mol/dm3)

- MSDS – Material Safety Data Sheets

- PROCEDURE:

GENERAL STORAGE

- Upon receipt of chemicals, container appearance should be physically examined for dents, swellings, defects, appropriate label. Chemicals shall be rejected if any discrepancy is noticed.

- Strictly follow the instructions in the SOP on Chemical Handling BML-QDS-CL-007 when handling any chemical, reagent, media or test kit.

- Stock reagents are received into the laboratory reagent dedicated room store and logged into the logbook store bin card.

- With proper inventory, store chemicals at appropriate temperatures and conditions as indicated on the container.

- Keep inflammable liquids and substances away from naked flame of electrical spark.

- At the time of opening of bottle/container, indicate the date of opening and expiry date

- Indicate a shelf life of 2 years from date of opening, unless otherwise stated on the label or MSDS.

- Stock reagents should be dispensed into reagent bottles from the stock reagent containers. The expiry date should be assigned as follows:

- Use the expiry date recommended by the manufacturer on the product label or from the certificate of analysis accompanied

- If not indicated on the label and certificate of analysis, an expiry date not more than five years is assigned to all reagents and chemicals that are stable based on previous experience or literature review

- If previous experience or scientific judgments warrants an earlier expiration date, this must be applied to the reagent and chemicals.

- An expiry date of three (3) months is acceptable for all prepared solutions of reagents known to be stable.

- Regents should be labeled and stored in tight containers made of resistant glass or other suitable material.

Reagent Bottle Name: __________________________________ Concentration: ___________________________ Prepared by: _____________________________ Date prepared: ___________________________ Expiry date: ______________________________

- Store appropriately; light sensitive reagents should be kept in amber bottles. Avoid using glass covers for alkali hydroxides.

- Expiry date for alkali should not be more than 1 month e.g. NaOH, KOH, CaOH.

- Stoppers and stopcocks brought into contact with substances capable of attacking or penetrating their surfaces may be given a protective coating of a thin film of a suitable lubricant unless specifically prohibited.

- Reagents should be checked daily for mould, particles or any physical contamination.

MOBILE PHASE.

- Mobile phases should be used up within 1 month of preparation.

- Mobile phase prepared with buffer should be used up within 5 days.

TEST SOLUTIONS

- Dilute acids should not be stored for more than 3 months

- Dilute bases should not be stored for more than 3 week

- Test solutions should be used up within 1 month of preparation

- Reagents solutions used for spot test, heavy metals test, colorimetric test or standards for limits tests should be prepared immediately before use, and discarded after use of the complexes because a change in potency can reduce the intensity formed using the reagents.

VOLUMETRIC SOLUTIONS

- Volumetric solution should be standardized before use and should be used up before the restandardization date.

- After the restandardisation date, volumetric solution should be re-standardized before reuse, if not physical changes to the solution

- Volumetric solutions of molarity less than 0.1M should be prepared immediately before use and then discarded after use.

BUFFER SOLUTIONS

Buffer solutions should be used up within 1 week of preparation

INDICATORS

    - Indicator solutions should not be stored for more than 1 months

    - Sensitivity tests should be carried out weekly.

    - For spillage do the following:

- If spillage is due to liquids like acids or solvents add sufficient quantity of absorbent silica and leave to stand for about 10minutes. Collect the material using a hand gloves in a plastic bag.

- For spillage of solids, remove and discard in the dustbin and clean the area

- For spillage of organic solvents, put on goggles and mask during cleaning.

- For spillage of liquids like alkali, add sufficient water to dilute it and mop up.

- While cleaning the area of spillage of poisonous or hazardous chemicals, take precautions as per MSDS of respective chemicals.

- For water spillage, only dry mop.

    - Discard waste/expired/mobile phase effluent chemicals to the effluent treatment plant through the fume hood sink

6.0     REFERENCE:

  - British Pharmacopoeia 2018, Volume V. Appendix 1B V-A21 – A166

  - United states Pharmacopeia 2018, volume 1, page 1369 - 1465

7.0 TRAINING INVOLVEMENT

Quality Manager

Quality control staff

All sectional heads in the quality department

8.0 DISTRIBUTION AND DOCUMENTATION CONTROL:

| MASTER COPIES | QUALITY ASSURANCE |
| --- | --- |
| CONTROLLED COPIES | QUALITY CONTROL LABORATORY |

9.0 REVISION HISTORY

| CURRENT REVISION | SUPERSEDE | REASON FOR REVIEW |
| --- | --- | --- |
| ### 00 | ### Nil | First issue |
| ### 02 | ### 01 | To Update |
| ### 03 | ### 02 | - Inclusion of enclosures format - Change in sop format generally |