- OBJECTIVE:

- This procedure is written as specific instructions for the preparation, sterilization, use and storage of culture media in the Microbiology Laboratory.

- This procedure is performed to ensure that all culture media are prepared in the laboratory according to manufacturer’s specifications and are stored under appropriate conditions.

- SCOPE:

- This procedure will be performed whenever a culture media is prepared in the laboratory.

- This procedure applies to all culture media used in the laboratory except where stated otherwise.

3.0 RESPONSIBILITY/ACCOUNTABILITY:

  - Microbiologist/Assistant is responsible for the media preparation and sterilization.

  - The Quality control Laboratory Manager ensures the procedures outlined are followed.

.4.0 ABBREVIATIONS AND DEFINITIONS

Media: are substances that encourage the growth, support, and survival of

Microorganisms

- PROCEDURE

Precautions:

- Always ensure that expired culture media are not used

- Always ensure that only sterile/sterilized culture bottles and plates are used

- Personnel must always work under aseptic environment.

- Always ensure that every culture media is properly labeled.

Procedure:

- Sterilize all glass wares to be used for the culture media preparation

- Check manufacturer’s instruction for each culture medium to be prepared and weigh appropriate amount accordingly.

- Dissolve the weighed powder agar in appropriate amount of distilled water (follow manufacturer’s instruction by mixing or boiling).

- Cork flask lid with absorbent cotton wool such that there will not be air leak

- Label culture flasks properly with date of preparation.

- Place culture flasks in the portable autoclave.

- Operate the autoclave as per SOP on bucket autoclave operation

- Make a set of culture plates from the culture media prepared to verify if media is sterile.

- Store every culture media under appropriate conditions as specified by manufacturer

- Document all activities in the logbook for media/reagent preparation LGB/QDG/001/MC/01S or 01F.

Media/Reagent preparation logbook LGB/QDG/001/MC/01S OR LGB/QDG/001/MC/01F

| DATE | MEDIUM | VOLUME PREPARED (ml) | QUANTITY WEIGHED(g) | pH OF WATER | PREPARED BY | CHECKED BY | REMARK | SIGN |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

- MATERIALS AND EQUIPMENT:

- Culture Media

- Culture Preparation Logbook

- Portable Autoclave

- Conical Flasks and Beakers

- Incubator

- Refrigerator

- Hot Air Oven

- Weighing Balance

- Sterile Bottles and Plates

- Distilled Water

- Laminar Flow Cabinet

- Cotton wool

- Aluminum  foil

- 70% IPA

- REFERENCE/ENCLOSURE

- NIL

- TRAINING INVOLVEMENT

| Quality Control Head | Quality control microbiologist, Laboratory assistant |
| --- | --- |

8.0 DISTRIBUTION AND DOCUMENTATION CONTROL:

| MASTER COPIES | QUALITY ASSURANCE |
| --- | --- |
| CONTROLLED COPIES | QUALITY CONTROL LABORATORIES |

9.0 REVISION HISTORY

| CURRENT REVISION | SUPERSEDE | REASON FOR REVIEW |
| --- | --- | --- |
| ### 00 | ### Nil | First issue |
| ### 02 | ### 01 | To Update |
| ### 03 | ### 02 | - Inclusion of enclosures format - Change in sop format generally |