- OBJECTIVE:

  - Validation is establishing documented evidence which provides a high level of assurance that facility or process will consistently produce a product that meets its pre-determined specifications and quality attributes. This forms the basis for efficient and effective manufacturing and control.

  - It is essential to ensure that all validation activities are appropriately approved and documented as well as to establish controls to ensure that equipment remains qualified, and processes validated.

  - To ensure that all facilities (equipment, utilities, raw materials, and components) and operations (procedures, processes,) that may affect product quality from development through to sales are qualified and validated.

  - To provide guidelines for conducting and documenting validation activities of BIOMEDICAL LTD. processes.

- SCOPE:

  - This Procedure is applicable to new and existing facilities and operations that may affect the quality of products.

  - It is also applicable whenever any change is made to existing facilities and processes.

- RESPONSIBILITIES/ACCOUNTABILITIES:

  - The Validation team is responsible for the review and re-issue or revision of this procedure.

  - Quality Assurance Manager is responsible for approving all validation exercise.

- DEFINITIONS AND ABBREVIATIONS:

  - PROTOCOL: A prospective experimental plan that when executed is intended to produce documented evidence that a system or subsystem has been properly qualified

  - ACCEPTANCE CRITERIA: The criteria that must be met to successfully complete a test phase or to achieve pass requirements.

  - URS: Users Requirements Specification. This is generated by the process owner, it specifies what the end user requires the system or equipment to perform. It is the basis for a commercial contract between vendor and purchaser

  - Functional Specification: Design output document generated by the supplier to describe the detailed functions of the equipment or system

  - Design Specification: Design output document generated by the supplier to provide complete definition of the equipment or system in sufficient detail to enable it to be built.

  - DQ: Design Qualification

  - IQ: Installation Qualification

  - LOQ: Limit of Quantitation

  - LOD: Limit of Detection

PQ

PQ

URS

URS

PROCEDURE

Supplier Assessment

Supplier Assessment

FS

FS

OQ

OQ

DS

DS

IQ

IQ

DQ

DQ

IMPLEMENT

IMPLEMENT

Diagrammatic overview of plant validation activities

  - The initial considerations for a validation study shall include-

- Identifying and describing the processes

- Specifying process parameters and desired output

- Deciding on whether each of these processes require verification and/or validation.

  - If the decision is to validate –

- Form a multi-functional team for validation (Validation Team)

- Plan the approach and define the requirements

- Create a Validation Master Plan.

- Select methods and tools for validation

- Create validation protocols

- Perform (Installation Qualification (IQ), Operational Qualification (OQ) and Performance

Qualification (PQ) and document results

- Determine continuous process controls

- Prepare final report and secure management approval

- Maintain the process in a state of validation by monitoring and controlling the process

- continuously

- Revalidate as appropriate

  - VERIFICATION / VALIDATION

B

Is Verification Sufficient & Cost Effective

B

Is Verification Sufficient & Cost Effective

A

Is Process Output

Verifiable

A

Is Process Output

Verifiable

C

Verify & Control

the Process

C

Verify & Control

the Process

YES					YES

NO				NO

Yes

# D

Validate

# D

Validate

E           Redesign the Product and/or Process

E           Redesign the Product and/or Process

A 	The Risk Analysis is used to determine whether the output of a process can be verified by subsequent monitoring or measurement.

B	If the consideration for A above is positive, then the consideration should be made as

to whether verification alone is sufficient to eliminate unacceptable risk and if it is

a cost-effective solution.

C	If the answer to B above is YES, the output should be verified, and the process should

be appropriately controlled.

D	If the output of the process is not verifiable, the decision should be to validate

the process.

E	It may become apparent that the product or process should be redesigned to

reduce variation and improve the product or process.

  - Validation Master Plan: It is a document pertaining to the whole facility that describes which equipment, systems, methods and processes will be validated and when they will be validated. The document provides the formats required for each particular validation document (Installation Qualification, Operational Qualification and Performance Qualification for equipment and systems; Process Validation; Analytical Method Validation ;), and indicate what information is to be contained within each document.

The Master Validation Plan should also indicate why and when revalidations will be performed.

  - Validation Committee: The Validation Committee shall comprise of the Validation Analyst, Warehouse Officer, Production Pharmacist, Production Manager, Factory engineer, Documentation Head, Electrical Engineer, Quality control Head, Quality Assurance Manager, Executive Director etc.

The Validation Committee shall be meeting from time to time. The committee shall be responsible for

- planning and supervision of all validation activities

- providing assurance that validation processes are well thought out

- ensuring that protocols are comprehensive and that the final validation packages are well documented and easy to follow.

  - Validation Team: It is an ad hoc, multi-functional team with members drawn from Quality, Engineering, Warehouse and Production. The Quality Analyst in consultation with the Head of the Department constitutes the team where validation is to be conducted. The team shall be responsible for conducting specific validation activities.

  - Calibration: It is the set of operations that establish, under specified conditions, the relationship between values indicated by an instrument or system for measuring (especially weighing), recording, and controlling, or the values represented by a material measured, and the corresponding known value of a reference standard. Limits for acceptance of the result of measuring should be established.

  - Protocols: It is a documented plan, which is reviewed, and approved prior to execution for the test of a process, system or piece of equipment. Upon completion, the protocol and the report serve as the basis for providing documented evidence that the process, system or piece of equipment performs as intended.

Protocol is a document describing the activities to be performed in a Qualification or Validation exercise. These include test parameters, equipment/product characteristics, and decision points on what constitutes acceptance criteria.

  - Qualification: It is the action of proving and documenting that any premises, systems and equipment are properly designed, installed, and or operates correctly and lead to the expected results.

Qualification is often a part (initial stage) of Validation, but the individual qualification steps alone do not constitute Process Validation.

Equipment is validated by performing an Installation Qualification (IQ) and Operational Qualification (OQ), while a system is validated by performing an Installation Qualification (IQ), Operational Qualification (OQ) and a Performance Qualification (PQ). Process Validation and Performance Qualification are often synonymously used

  - Design Qualification: It is the documented evidence that the premises, supporting utilities, equipment and Processes have been designed in accordance with the requirements of GMP.

  - Equipment Qualification: When the need arises to purchase new equipment, which can have effect on product quality, or where existing equipment is relocated or modified, the Head of Department must set up a validation team. The introduction of the equipment into routine production should include the following stages:

Installation Qualification (IQ):  It is the documented verification (evidence) that, at the time of installation, a system or equipment complies with the recommendations of the manufacturer as well as with design or system specifications.

The Protocol for performing an Installation Qualification is detailed in the Guide to DQ/IQ/OQ/PQ BML-QDG-QA-111-G01. Installation Qualification is the responsibility of the Engineering department.

Operational Qualification (OQ): It is the documented verification (evidence) that, a system or equipment performs in accordance with the manufacturer‘s specifications and process requirements and that the appropriate GMP systems (e.g training, calibration and maintenance, etc. ) are in place. The Protocol for performing an Operational Qualification is detailed in the Guide to DQ/IQ/OQ/PQ BML-QDG-QA-111-G01. Operational Qualification is the responsibility of the Process Owner.

Performance Qualification (PQ): It is the documented verification (evidence) that the equipment or system operates consistently and gives reproducibility within specifications and parameters for prolonged periods.

Requalification: It is the repeated qualification of a qualified system or equipment. Re-qualification should be done in accordance with a defined schedule. The frequency of re-qualification may be determined based on the factors such as the analysis of results relating to calibration, verification and maintenance. There should be periodic re-qualification after changes (such as changes to utilities, systems, equipment, maintenance work (overhauling), and relocation). Re-qualification should be considered as part of the change control procedure.

BML-QDG-QA-071

Validation: Action of proving and documenting that any process, procedure, material activity, system or method actually leads to the expected results.

Revalidation: It is the repeated validation of an approved process, procedure, material activity, system or method whenever a change is made to the process, procedure, material activity, system or method. In completing the Validation report the objective should be to specify in-process controls that are sufficient to ensure continued satisfactory operation of the process. There may, however, be instances where the Validation Team deems a periodic formal re-validation necessary. The need for such revalidation must be assessed at the time the protocol is completed for Acceptance of Results.

Process Validation: It is the documented verification (evidence) which provides a high degree of assurance that specific process will consistently produce a product that meets its pre – determined specifications and quality attributes.

Cleaning Validation:

    - It is the documented evidence to demonstrate that cleaning procedures are removing residues from Active Pharmacopoeia Ingredients (APIs) and excipients, cleaning agents, etc., to predetermined levels of acceptability, taking into consideration batch size, dosing, toxicology, equipment size, etc

    - Only cleaning procedures for product contact surfaces of the equipment need to be validated. Dedicated piece of equipment should be used for products which are difficult to clean, equipment which is difficult to clean, or for product with a high safety risk.

    - In performing a cleaning validation, the following should be considered;

- Disassembly of the system or equipment,

- Pre-cleaning,

- Cleaning agent: concentration, solution volume, water quality,

- Time and temperature,

- Flow rate, pressure and rinsing,

- Complexity and design of the equipment, and

- Training of operators.

    - The cleaning validation protocol should in addition to the general protocol for process validation, include

- The interval between the end of production and cleaning and the commencement of the cleaning procedure (Interval may be part of the validation challenge study itself).

- the maximum period that equipment may be left dirty before being cleaned as well as the establishment of the time after cleaning and before use.

- The microbiological levels (bioburden)

- The cleaning procedure (documented in an existing SOP) to be used for each product, each manufacturing system or each piece of equipment.

- The number of cleaning cycles to be performed consecutively.

- The sampling procedure used (e. g direct sampling, rinse sampling, in-process monitoring, sampling locations) and the rationale.

- The data on recovery studies

- The analytical methods (specificity and sensitivity) including the limits of detection, LOD, and the limits of quantitation, LOQ.

- The choice of cleaning agent should be documented and approved by the QA and should be scientifically justified based on, e. g

-the solubility of the materials to be removed

- the design and construction of the equipment and surface materials to be cleaned

- the safety of the cleaning agent

- the ease of removal and detection

- the product attributes

- the minimum temperature and volume of cleaning agent and rinse solution

- the manufacturer’s recommendations

ix)	  Acceptable limits should be defined for residues from cleaning agents (e.g detergents

Cleaning Validation is done as per BML-QDG-QA-047.

  - Analytical Method Validation: It is documented evidence to demonstrate that the analytical procedure is suitable for its intended purpose.

When Pharmacopoeia methods are used, evidence should be available to prove that the 	methods are suitable for routine use in the laboratory (verification is required).

Non – Pharmacopoeia methods should be appropriately validated.

  - Protocol for Validation (Process & Analytical Method validation):

    - The Protocol (format) for performing a Process validation is detailed in the SOP titled “How to perform a Process validation” BML-QDG-QA-112 and Protocol for performing Analytical Method validation, equally detailed in the SOP titled “Analytical Method validation” Protocol for Qualification shall be as per an Operational Qualification is detailed in the Guide to DQ/IQ/OQ/PQ.

    - Raw data generated during any validation activity would be documented in controlled logbooks and columns provided for performed by and checked by.

    - Protocols would be generated, reviewed, and approved prior to carrying out the validation exercises. The protocols would be separated from the report.

    - The protocols would reference the source of methodology, acceptance criteria, procedure etc.

  - Validation Of New Products: Prior to the release of a validation batch of product for use or sale, assurance is required that the batch is fit for its intended purpose. It is therefore essential that there is a system for reviewing data generated during the manufacturing, inspection and testing process to assess compliance of the batch of product with all relevant standards and specifications.

  - Approaches To Validation: There are two basic approaches to validation – one based on evidence obtained through testing (Prospective and Concurrent validation), and the other based on the analysis of accumulated (historical) data (Retrospective validation).

  - Prospective Validation:

    - The execution and documentation of pre-approved test protocol, which is designed to prove that a system performs as intended, prior to the release of a manufactured product for distribution. A minimum of three (3) batches of the product is required.

    - Critical factors that may affect the quality of the finished product should be determined during product development. To achieve this, the production process should be broken down into individual steps, where the impact of each step should be evaluated (e.g. on the basis of experience or theoretical considerations).

    - Prospective validation should include,

- The variables (critical parameters) to be monitored

- The samples to be taken – where, when, how and how many

- The product performance characteristics/attributes to be monitored, together with the test methods

- The acceptance limits

- Time schedules, personnel responsibilities, and details of methods for recording and evaluating results, including statistical analysis

    - All equipment, the production environment and analytical testing methods to be used must have been fully validated (e. g Installation and Operational Qualification)

    - Personnel participating in the validation work must have been appropriately trained

    - The Batch Manufacturing Record (BMR) to be used should then be prepared after these critical parameters of the process have been identified; machine settings, component specifications and environmental conditions have been determined and specified.

    - Three (3) consecutive batches of the final product should then be produced as validation batches. The analytical results of these batches are used to establish the normal extent of variation and trends and to provide data for evaluation. If the data (results) from these batches are within specified limits, and give product of the desired quality, this information can constitute a proper validation of the process.

    - The batches should be of the same size and should be the same as the intended batch size for

full scale production.

    - The results should be documented in the validation report. The report should include at least;

- a description of the process – Batch Manufacturing Record, including details of critical steps,

- a detailed summary of the results obtained from in-process and final testing, including data from failed tests. Where raw data are not included reference should be made to the sources and where they can be found.

- A review and comparison of the results obtained with those expected, and formal acceptance/rejection of the work by the team/persons designated as being responsible for validation.

- A conclusion and recommendation should be made on the extent of monitoring and the in-process controls necessary for routine production, based on the results obtained.

- These should be incorporated into the BMR and/or SOPs for routine use. Limits and frequencies should be specified. Action to be taken in the event of the limits being exceeded should be specified.

  - Concurrent Validation: Validation carried out during routine production of products intended for sale.

In certain cases, it may be appropriate to validate a process during routine production, e.g where the product is a different strength of a previously validated product, a different shape/size or where the process is well understood. The decision to carry out concurrent validation should be made by appropriately authorized personnel. It is important that the premises and equipment to be used during concurrent validation have been qualified previously. The results should be documented in the validation report.

  - Retrospective Validation

    - This involves the examination of past experience (data, results, trend, etc)) of production on the assumption that compositions, procedures, and equipment remain unchanged.

    - It is based on a comprehensive review of historical data to provide the necessary documentary evidence that the process is doing what it is believed to do.

    - It requires the preparation of the report of results of the data review, a conclusion and recommendation.

    - Retrospective validation is not the preferred method of validation and should be used in exceptional cases only. It is only acceptable for well-established processes and it will be inappropriate where there have been changes in the composition of the product, SOPs or equipment.

    - Sufficient data should be reviewed to provide a statistically significant conclusion.

    - When the result of retrospective validation is satisfactory, it should serve only as an indication that the process does not need to be subjected to validation in the immediate future.

  - Change Control:

    - A formal process in which changes to equipment, systems, procedures or processes are proposed by individual or units planning to implement them, the changes are reviewed by qualified representatives of QA and other relevant department to determine whether the changes will affect the status of validation and qualification.

    - The reviewers shall determine whether it is required to validate the system or take other action necessary to maintain the validated status of the system.

    - The QA, Engineering and Production Departments must assess the impacts of all changes of the following nature and the processes are revalidated in case it is required.

    - The changes may include but are not limited to product s manufactured by processes subjected to changes should not be released for sale without full awareness and consideration of the change and the impact on the process validation.

    - Changes that are likely to require revalidation may include;

- changes in the manufacturing process (e. g mixing times, autoclaving temperatures)

- changes in the equipment

- production area and support system changes (e.g. rearrangement of areas, new water treatment method)

- transfer of processes to another site, and

- unexpected changes (e. g those observed during self-inspection or during routine analysis of process trend data)

  - Process Flow Sheet: Process flow sheets should be prepared, when appropriate, in order to identify critical aspects of the process to be covered by the validation exercise. If operations are as large as to require sub-division inter-linking between them must be clearly indicated. The validation team must consider the need for documenting non-critical items and reasons for not validating them.

  - Documentation And Final Approval

i)	All documents related to the Validation study must be collated in a file suitable for presentation to any regulatory authority.

ii) 	The Validation Protocol gives an overview of the process to be carried out and must be agreed upon by the process owner before the exercise commences. The Validation Report contains the results obtained from the exercise.

ii)	QA must approve validation reports.

- Final approval will be the joint responsibility of the Departmental Manager and QA

- Where periodic re-validation is deemed necessary the timing of the re-validation should be agreed.

- Where there is inconclusive task in the approved validation/Qualification protocols, a deviation is to be raised and handled according to handling of Deviation procedure BML-QDG-QA-058 and this will be recorded in the deviation section of the validation protocol and report.

  - Generating Validation Protocol Number

Guidelines for generating validation protocol numbers is shown as in

BML/AAA/BB/CC/YY/XXX

Where:

BML= BIOMEDICAL ltd.

AAA= The originating department

BB = The unit in such department where the machine or process is located or done

CC = Protocol or Report i.e PT or RP

YY=Last two digit of the year

XXX= Sequential numbering of the particular validation / qualification for the year.

E.g. BML/QDF/CL/PT/24/001 (Protocol)

And BML/QDF/CL/RP/24/001 (Report)

Departmental abbreviation:

PDG, PDF, PDS 	-	Production (GENERAL, INFUSION, SYRUP)

ENG, ENF, ENS		-	Engineering (GENERAL, INFUSION, SYRUP)

QDG, QDF, QDS	-	Quality Department (GENERAL, INFUSION, SYRUP)

WTG, WTF, WTS	-	Water Treatment Plant (GENERAL, INFUSION, SYRUP)

  - Equipment Impact Assessment

Impact assessment will be carried out on equipment/instrument to be validated as per

BML-QDG-QA-111-F01

Addendum to an existing Validation Master Plan will be done as per Addendum to Validation Master Plan

Commissioning Report

Commissioning report will have details of the equipment’s, system or utilities, suppliers name and address and who commissioned and verified. This shall be done as per Commissioning Report Guideline BML-QDG-QA-111-G02

- REFERENCES AND ENCLOSURE

  - Validation Protocol Guide to DQ/IQ/OQ/PQ - BML-QDG-QA-111-G01

  - Equipment/Instrument Impact Assessment form - BML-QDG-QA-111-F01

  - Commissioning Report Guideline BML-QDG-QA-111-G02

- TRAINING INVOLVEMENT

| All departmental managers | All sectional heads |
| --- | --- |
| QA Doc. Unit staff | All staff |

- DISTRIBUTION AND CONTROL COPY:

This is the various department/section where the SOP copies will be utilized.

| MASTER COPY | QUALITY |
| --- | --- |
| CONTROLLED COPY | QUALITY, PRODUCTION, ENGINEERING, ADMIN |

- CHANGE HISTORY

| CURRENT REVISION | SUPERSEDE | REASON FOR REVIEW |
| --- | --- | --- |
| 00 | Nil | First issue |