- OBJECTIVE

To lay down procedure for action to be taken during spillage in the facility

- SCOPE

Applies to spilled materials, media and solvents at the facility

- RESPONSIBILITIES/ACCOUNTABILITIES

- The Warehouse head shall be responsible for the implementation and maintenance of this procedure.

- The Quality Control Head shall be responsible for this procedure

- The Quality Assurance Manager shall be responsible for total compliance of this procedure.

- ABBREVIATIONS AND DEFINITIONS

ETP:   Effluent Treatment Plant.

IPA :  Isopropyl Alcohol.

- PROCEDURE

DRIED/SOLID MATERIAL

  - If any spillage occurs, personnel shall segregate or shift other surrounding materials to avoid contamination.

  - Give notice to QA for corrective action.

  - Use Vacuum cleaner.

  - Spilled materials shall be collected by taking sufficient safety precautions before cleaning the area. (wear safety goggle, nose mask, hand gloves)

  - Transfer the contents of the affected container or bag to another good & cleaned container/Bag.

  - Weigh and note the quantity of spilled materials and destroy as per destruction procedure.

  - The same quantity is to be deducted from the tally card.

  - Prepare the report of the cause of the incident and appropriate corrective action to avoid re-occurrence of such incident.

  - Note the cause of the incident and take corrective action to avoid re-occurrence of such incidents.

SOLVENT

    - In case of spillage or leakages of solvent, immediately open the door for proper ventilation.

    - Restrict movement of people to the area of the incident.

    - Mop the area with all safety precautions like wearing Nose Mask , Safety Goggle and discard the spilled solvent in the ETP.

  - Weigh and note the quantity of the spilled materials as per destruction procedure.

  - The quantity of the spilled solvent should be deducted from the stock card   .

  - Note the cause of the incident and take corrective action to avoid re- occurrence of such incident.

MEDIA

- Upon spillage of the solid or liquid media containing growth, wear surgical glove and collect solid media and remove excess fluid from the surface with the help of cotton piece.

- Spray 70% of IPA and leave it for 5mins

- Mop the surface twice using cotton piece with 2.5% Dettol.

- Let it dry off

- Discard the media with the cotton piece properly.

- REFERENCE AND ENCLOSURE:

- Nil

7.0 TRAINING INVOLVEMENT

| All QC staff | All Manager |
| --- | --- |
| QA Doc. Unit staff | All warehouse staff |

8.0 DISTRIBUTION AND CONTROL:

| MASTER COPIES | QUALITY ASSURANCE |
| --- | --- |
| CONTROLLED COPIES | ALL DEPARTMENT |

9.0 CHANGE HISTORY

| CURRENT REVISION | SUPERSEDE | REASON FOR REVIEW |
| --- | --- | --- |
| 00 | Nil | First issue |