- PURPOSE: To lay down the guideline to follow for safety precaution in the Laboratory.

- SCOPE: This SOP covers the procedure to follow safety precaution in the Laboratory.

- RESPONSIBILITY:  Q.A Officer, Microbiologist, Chemist, Q.A. Manager, Supt. Pharmacist/ Factory Manager.

- PROCEDURE:

  - Guidelines for personal laboratory safety

- Eating, drinking or smoking in the laboratory should be strictly prohibited.

- Use safety goggles for eye protection while handling corrosive and highly toxic materials.

- Wear lab coats when handling corrosive, toxic, or flammable materials.

- Wear gloves when handling corrosive and highly toxic materials.

- Know where safety equipment (extinguishers) is located and how to use it.

- Do not carry corrosives by the mouth of the bottle.

- Never pipette corrosive liquids by mouth. Use a pipetting bulb or other mechanical pipetting devices.

- Do not attempt to identify chemicals by smell or taste. Never sniff reagents, especially corrosive one

- Know how to clean up spills of the chemicals that you use.

- Wash your hands after handling chemicals and before leaving the lab.

- Bare legs are not acceptable when handling hot, cold, toxic or corrosive chemicals.

- Eyewash facilities should be within easy access.

- Keep appropriate safety equipment readily available and properly maintained to prevent or respond to such laboratory emergencies as personal contamination, fires, spills or splashes.

- Obey the "two person" rule. A second person should be aware that you are working in the laboratory.

  - Checklist for preliminary hazard analysis

- Always check the safety precautions for the material you are using to understand the specific hazards involved.

- Review the characteristics of all reagents, reactants in terms of flammability, toxicity and reactivity hazards. Where information is not available, treat the materials as hazardous.

- What is the type of hazard (inhalation, ingestion, eye or skin contact)? What protective measures are required?

- What is the recommended first aid in case of accidental exposure?

- Can waste be safely handled and arrangements for disposal completed?

  - Safety controls

- Engineering controls are always the best choice for regulating hazardous materials; Use wherever applicable:

- Fume hoods

- Local exhaust hoods

  - Chemical storage

- Chemical storage should be under the supervision of a qualified person; store rooms must have adequate security.

  - Conduct an annual inventory

- To remove surplus hazardous chemicals.

- To remove chemicals that will not or have not been used in the past 1-3 years.

- To correct incompatible storage.

- To identify which chemicals are present.

- To conduct a regular clean-up of containers and shelving.

  - General rules

- Shelving should be accessible with chemicals at eye level or lower; no high shelf storage.

- Avoid floor chemical storage.

- Do not overcrowd shelves.

- Chain compressed gas cylinders.

- Keep solvent containers closed. Ensure chemical containers are intact. Ensure container lids are intact and closed. Regularly vent materials capable of building up pressure; e.g. formic acid.

- For emergencies have fire extinguishers of the approved type positioned near an 
escape route spill control and clean-up materials.

- Proper safety measures and equipment must be followed during breakage of glass materials, cleaning of dust generating area.

- No naked wire or electrical appliances must be left on attended to in the Laboratory.

- Switch off the appliance when notice unusual noise or after closing

- Do not use any unlabeled chemical or reagent.

  - Label properly

- Label contents clearly.

- Labels must be intact and legible.

- Do not overwrite labels.
4.8 	Refrigerator storage

- In the event that chemicals need to be stored in a refrigerator, follow the rules listed below:

- The materials must be securely packaged, tightly sealed and properly labeled.

- Ensure that materials, especially those that are highly reactive or corrosive, are inspected regularly and that lids are intact.

- Microbiology Laboratory

- All contaminated materials should be disinfected before proper washing and sterilization.

- Before exit any culture split on the floor or bench should be swabbed immediately with

Cotton wool soaked in disinfectant solution and properly disposes off in the waste

container.

- Remove gloves, nose mask and dispose of them appropriately.

- Always wash your hands with soap and disinfectant.

- Follow good laboratory practice at all time.

- ABBREVIATIONS
- NO. – Number

- SOP - Standard Operating Procedure
- QC - Quality Control

7.0	Materials, Equipment, Apparatus and Reagents

N/A

8.0	Training Requirements

- On relevant SOP’s

- On good documentation practice.

- Distribution and Documentation control:

As per SOP on Document Control and Management – SOP No.: BML-QA-002-00