1.0	Purpose:

- To provide written instruction for carry out analysis on Filter use for Filtration during the  manufacturing process

2.0	Scope:

- To determine the effectiveness  of the filter

3.0	Responsibility:

- Microbiologist, Chemist, QA Officer, QA. Manager, Supt. Pharmacist

4.0	References

-		In House

5.0	Materials, Equipment, Apparatus and Reagents

- Ph meter

- Conductivity meter

- Sterile petri-dishes

- Sterile 1ml pipettes

- Syringe

- Pipette filler

- Cotton wool

- 70% Ethanol

- Aluminium foil

- Lamina flow cabinet

- Bunsen burner

- Spatula

- Incubator

- Adhesive Tape

- Media. i.e NA, SDA, EMB/ MA, BGA, PSA and MSA

## 6.0	Procedure

- Ensure that the sampling booth,sampling apparatus and containers are clean.

## Take one pack of filter to be sampled inside sampling booth.

- Only one item must be sampled at a time and always make sure that the sampling area is clean.

- Wear hand gloves and nose masks during sampling

- Collect the sample in a clean container and label them with all necessary Details e.g. Item description, B. No. date, sampled by etc. and take it inside inoculation room for analysis.

- Cut the filter  into pieces  with a sterile scissors

- Pour it inside the cleaned conical flask

- Pour 500mls of sterile water inside conical flask.

- Allow it to stay for 45 minutes.

7.0 MICROBIAL ANALYSIS

- 1ml of the water sample should be placed in a Petri-dish then flood with sufficient quantity of sterile molten nutrient agar, SDA, EMB, swirled and allowed to set. (pour plate technique)

- The plates should be labeled accordingly.

- SDA is incubated at 20-250c for 96hours while the remaining plates are to be incubated at 35-370c for 24hours. All plates are incubated upside down

7.1READING AND INTERPRETATION OF RESULTS:

- There should be absence of fungal  and Microbial growth in plates containing SDA and NA It must be pyrogen free

7.2 REPORTING:

- The result should be recorded in record book.

- The final result should be reported to QAM

- If the specification is exceeded,report immediately to QAM.

8.0 CHEMICAL ANALYSIS

8.1CONDUCTIVITY:

- Take 50ml of water sample in a beaker.

- Immerse the conductivity meter probe.

- Invigorate and read.

- Expected observation: Not more than 1.3 s/cm at room temperature.

8.2 ACIDITY / ALKALINITY:

- Pipette 20ml of water sample into a clean test tube.

- Add 0.05ml of Phenol red solution.

- Expected observation: Not more than 0.1ml/0.15ml of 0.01M Sodium hydroxide / 0.01M Hydrochloric acid is required to turn the colour to red/yellow.

8.3 OXIDISABLE SUBSTANCES:

- Take 10ml of the water sample in a 250ml beaker.

- Add 1ml of dilute sulphuric acid and boil.

- Add 0.02ml of 0.02M Potassium permanganate.

- Boil further for 5 minutes.

- Expected observation: The solution remains faintly pink.

LIMIT TEST:

8.4 Heavy Metals

- Heat 100ml of sample in a glass evaporating dish on a water bath until the volume in reduced to 20ml.

- Take 1.2ml of the concentrated sample in a test tube and mixture of 0.1ml of lead standard solution (1ppm) with 2ml of concentrated sample in another test tube.

- Add 0.2ml of acetate buffer 3.5 to each test tube.

- Mix and add 1.2ml of Thioacetamidle reagent to each test tube.

- Mix immediately and allow them to stand for 2 minutes.

- Expected observation: The standard solution is more intensely coloured than the test solution.

9.0       Training Requirements

- On relevant SOP’s

- On good cleaning practices

- On cGMP

- On good documentation practice.

10. Distribution:

- Micro Lab.

- Chemical Lab.

- Controlled copy

- Master copy