1.01.0 PURPOSE: To provide written guidelines for Effluent treatment plant.

- 2.0 SCOPE: This SOP covers the written procedure for Effluent treatment plant.

- 3.0 RESPONSIBILITY: Water treatment officer, Engineering Officer, Quality Control Officer, Quality Assurance Manager, Superintendent Pharmacist/Factory Manager.

- 4.0 PROCEDURE

  - Domestic Effluent:

- Domestic effluent from the toilets and urinals is received to the septic tank collection tank by a registered contractor.

4.2 Industrial Effluent:

- Check the raw effluent Ph from the equalization tank and add the neutralizing chemical, Sodium Hydroxide (NaOH)/ citric Acid till the ph 6.5-9.0

- Switch “ON” the raw effluent pump and allow the effluent to mix with in the Equalization tank continuously.

- Take sample from the chamber of the tank and check the PH if it is within the specifications

- Add NaOH if the effluent sample is low i.e <6

- Add citric acid if the effluent sample has a high Ph >9

- Switch the pump on and mix again

4.3 Physio-Chemical Treatment:

- The physio-chemical treatment process includes neutralizing, chemical coagulation and precipitation.

- Chemical Preparation.

- Prepare the 1% alum in the tank.

- 1% Alum: 1kg of alum dissolved per 100 litres of Solvent

- Operate the Dosing pump

- Dose the prepared Alum into the effluent tank

- The effluent after adjusting PH, allowed to settle then the settled sludge at the bottom shall be evacuated by the registered contractor (Sludge handling system).

- Collect the effluent water and check

- Parameters like PH, TSS, TDS, COD, BOD, Oil and grease etc shall be analyzed

- The effluent water shall meet the following parameter,

- Materials, Equipment, Apparatus and Reagents

NAOH - Sodium Hydroxide

6.0 Reference Documents

N/A

7.0 Definitions and Abbreviation

PH: Potential of Hydrogen

TSS: Total suspended solid

TSD: Total Dissolved solved

BOD: Biochemical Oxygen Demand

COD: Chemical Oxygen Demand

8.0 Training Requirement

- On relevant SOP’s

- On Cgmp

9.0 Distribution:

- QA

- Controlled copy

- Master copy

Annexure i

Effluent Water Parameter

| Sr.No. | Parameter | Unit | Concentration |
| --- | --- | --- | --- |
| 1. | Ph | - | 6.0 to 9.0 |
| 2. | TDS | mg/l | <100 |
| 3. | TSS | mg/l | <100 |
| 4. | BOD | mg/l | <20 |
| 5. | COD | mg/l | <40 |
| 6. | Oil and Grease | mg/l | <10 |
| 7. | Chloride | mg/l | <5 |
| 8. | Sulphates | mg/l | <100 |
| 9. | Heavy metal ( lead ) | mg/l | <0.1 |
| 10. | Feacal coliform | Cfu/ml | Must be absent |
| 11. | Total plate count | Cfu/ml | Max 102 |