- OBJECTIVE:

The objective of this SOP is to lay down the standard operating procedure for the entry-exit procedure in production area.

- SCOPE:

This SOP applies to all personnel, including visitors entering or leaving the production area of the factory.

- RESPONSIBILITIES/ACCOUNTABILITIES:

All personnel, House Keeper, Production Pharmacist, HR Manager, Production Manager, Quality Assurance Manager, Superintendent Pharmacist.

- ABBREVIATION AND DEFINITIONS

- QA – Quality Assurance

- cGMP  - current Good Manufacturing Practices

- PPE – personal protective equipment

- PROCEDURES

5.1 General:

- Visitors into the factory Production and Quality Control Areas are not allowed to touch, pick and operate any instrument equipment or machine as well as materials

- That carrying of edibles, cigarettes, medicines, cosmetics, flammable materials and other chewable are strictly prohibited inside the factory

5.2 Instructions Entry-exit Procedure:

- Before the take entry into a change room, drop all the edible, chewable, and beverages in the canteen. Eating and chewing are prohibited inside the production area.

- Visitors shall be accompanied by an authorized person of the company who shall guide them regarding entry and exit.

- In case of emergency exit, used factory clothes should be discarded in the “used garment Bin,” and clean factory PPE should be worn and follow the standard operating procedure.

5.3 Entry procedure for employees:

- To enter the changing room, make sure street shoes are dust-free.

- Remove and keep Street footwear in the shoe rack provided.

- Keep the person belongings like a wristwatch, Ornaments, wallet purses etc. in the personal locker provided for the purpose. These items are prohibited in the production and warehouse areas.

- Ensure that feet are dust-free and clean well on the foot mat provided before entering the change room.

- Enter in the primary change room, remove the street cloth and keep it in the designated cupboard provided for the purpose.

- Wear the slippers provided in the changing Primary room for the washroom area.

- Wash hands with a soap solution and water in the washroom.

- Take your bath if require

- Dry hands by using a hand dryer (use a napkin paper if required).

- Remove the sleeper and keep it in the rack provided outside the washroom.

- Rub the bare feet over the tacky foot mat provided.

- Wear the factory footwear while crossing over the crossover bench.

- Take out the factory garment and Cap from the personal shelves provided in the next change room.

- Wear the first Cap and then the factory garment.

- Wear the Cap while adjusting hair well inside the Cap.

- Check the full PPE for proper dress-up in the mirror.

- Enter in Buffer change room and disinfect hands by using the disinfectant solution 70%IPA provided.

- Enter in the respective production area.

5.4 Exit procedure for the employees

- Enter the change room through a buffer change room.

- Remove the factory garment and cap and keep them in the ‘Used Garment Basket” provided.

- Remove factory footwear and keep it in the rack provided while crossing the cross over the bench.

- Take out the street cloth from the cupboard and wear it in the changing room.

- Enter into the primary change room.

- Collect personal belongings from personal lockers.

- Wear street footwear.

- Open the door to exit from the changing room.

5.5 Entry and exit procedure for visitors

- Deposit personal belongings in the locker provided in the administration area.

- Rub the shoes over the foot mat provided at the entry stage to ensure that all the dust particles are properly removed from the street footwear before entering into the primary change room.

- Take the factory garment and Cap provided in the next change room.

- Wear first the Cap and then the factory.

- Check the full PPE for proper dress-up in the mirror.

- Visitors must be properly gown with all required personal protective equipment (PPE) like, overall, nose mask, head cover, shoe cover etc. Check the outfit in mirror for proper gowning before been led into the Production Area.

- Enter in Buffer change room, disinfect hands by using the disinfectant solution 70%IPA provided.

- Enter in the respective production area.

Note: - Visitors may wear a disposable factory dress over the street clothes and shall be provided with shoe cover while footwear change takes place.

- Entry into the clean area is restricted. Only when utmost necessary (like regulatory agency visit) can such entry is permitted.

- When entry into the clean area , visitor must follow the procedure in the SOP for entry into the clean room SOP No.: BML-QDG-QA-012

- In case of any clarifications/doubts please ask the accompanying technical staff, do not assume anything on your own.

- Materials, Equipment, Apparatus and Reagents

- Over all gown/coat

- Head and shoe covers

- Nose mask

- Hand gloves

- REFERENCE AND ENCLOSURE

In- House Method

8.0  TRAINING INVOLVEMENT

| All departmental managers | All sectional heads |
| --- | --- |
| QA Doc. Unit staff | All staff |

8.0 DISTRIBUTION AND CONTROL:

| MASTER COPIES | QUALITY ASSURANCE |
| --- | --- |
| CONTROLLED COPIES | ALL DEPARTMENT |

9.0 CHANGE HISTORY

| CURRENT REVISION | SUPERSEDE | REASON FOR REVIEW |
| --- | --- | --- |
| 00 | Nil | First issue |
| 06 | 05 | - Change in sop format. - Inclusion of traceability of enclosures to parent SOP |