- OBJECTIVE:

To provide the instructions for cleaning of the mixing Tank.

- SCOPE:

To provide written guidelines for line clearance for mixing tanks use in production of IV Fluids.

- RESPONSIBILITIES/ACCOUNTABILITIES:

- Operators/Production: Perform physical clearing and cleaning of the tank.

- Supervisor: Oversee the process and ensure personnel are following the SOP.

- Quality Assurance (QA) Officer: Verify all steps have been completed satisfactorily and provide final approval.

- ABBREVIATION AND DEFINITIONS

BMR – Batch Manufacturing Record

SOP – Standard Operating Procedure

PPE- Personnel Protective equipment

- PROCEDURES

- Pre-Clearance Checks, Physical Clearing and Cleaning should be done by the operator while Checking and Verification will be carried out by the Production Pharmacist/supervisor and final verification and approval by QA Officer.

- Verify that the previous batch's manufacturing record (BMR) is completed and signed.

- Ensure all empty containers, packaging, and excess materials from the previous batch have been removed from the area.

- Check that all equipment accessories (e.g., scoops, stirring rods) are cleaned.

- Confirm the status of the area's environmental conditions (temperature and humidity) and that the relevant HVAC filters are clean.

- Manually or mechanically remove all bulk materials (APIs, raw materials, excipients) from the mixing tank and related equipment.

- Remove any internal filters, strainers, or accessories not used in the current batch.

- Disassemble parts of the mixing tank and associated pipelines for detailed cleaning in an appropriate washing area, as per the established cleaning SOP.

- Clean all accessible surfaces of the mixing tank using approved cleaning agents and procedures to remove residues.

- Visually inspect the tank and all associated areas to ensure no residues are visible.

- Confirm that the tank's cleaning status label, indicating "Clean," is affixed.

- If necessary, submit swab or rinse samples to Quality Control for analysis to confirm the absence of cleaning agents and previous product residues.

- Verify that the weighing balance used for raw materials is clean and calibrated.

Documentation and Final Approval:

- Complete a line clearance checklist, documenting details of the previous and upcoming batches, the removal of all materials, and the signing of the checklist by the operator and supervisor.

- Obtain the final signature from the QA officer on the line clearance checklist, signifying approval to proceed with the new batch.

- Retain the original copy of the line clearance form with the QA department for auditing purposes.

Materials, Equipment, Apparatus and Reagents

- PPE

- REFERENCE AND ENCLOSURE

ENCLOSURE 1- MIXING LINE CLEARANCE FORM- BML-QDG-QA-021-F01

7.0 TRAINING INVOLVEMENT

| All departmental managers | All sectional heads |
| --- | --- |
| QA Doc. Unit staff | Production staff |

8.0 DISTRIBUTION AND CONTROL:

| MASTER COPIES | QUALITY ASSURANCE |
| --- | --- |
| CONTROLLED COPIES | ALL DEPARTMENT |

9.0 CHANGE HISTORY

| CURRENT REVISION | SUPERSEDE | REASON FOR REVIEW |
| --- | --- | --- |
| 00 | Nil | First issue |
| 06 | 05 | - Change in sop format. - Inclusion of traceability of enclosures to parent SOP |

ENCLOSURE 1

| Previous Product: B. NO.: |  |  |
| --- | --- | --- |
| Check List | Checked by (Production) | Verified by (QA) |
| Area Cleanliness |  |  |
| Equipment &accessories (Mixing tank, stirrer, transfer pumps etc.) cleanliness |  |  |
| Absence of previous product / batch materials |  |  |
| Personnel Accessories (PPE and cleanliness) |  |  |
| Environmental condition Std Temp. : 25±2°C |  |  |

| MIXING LINE CLEARANCE MONITORING FORM |  |  |  |  |
| --- | --- | --- | --- | --- |
| Time: | Clean |  | Not clean |  |
| Area |  |  |  |  |
| Personnel |  |  |  |  |
| Equipment/Vessel | Equipment/Vessel No. | Cleaned by | Status | Inspected by QA |
| Weighing Balance | BL/SPR/PE/002 |  |  |  |
| Lamina Air flow ( Dispensing Booth ) | BL/DSB/PE/001 |  |  |  |
| Mixing Vessel ( 2000litres ) | BL/SPR/PE/007- |  |  |  |
| Present / Absent of previous product | Parameters |  | Specification | Result |
| Previous product: | Colour |  | colourless |  |
| Batch No: | Identification |  | Must comply |  |
| Present product: | pH |  | 5.0-7.0 |  |
| Batch No: | Conductivity |  | NMT 1.3µs |  |
| Remark : |  |  | QA Officer | Date |